3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2208364

Online, August 17-29, 2022

Agenda Item:		6.10.3.2.3

Source:				ASUSTeK

Title:		Discussion on configuration of harq-ProcessNumberSizeDCI-0-2

Document for: 	Discussion and Decision

- Introduction

In the current TS 38.331 [1], the configuration of harq-ProcessNumberSizeDCI-0-2 is not aligned with the description in TS 38.212 [2].

In this contribution, we discuss how to fix the misalignment.

- Discussion

According to TS 38.212 V17.2.0 [2], as quoted below, the parameter harq-ProcessNumberSizeDCI-0-2-r17 could be configured to indicate 0, 1, 2, 3, 4, or 5 bits:

- HARQ process number – number of bits determined by the following: - 0, 1, 2, 3, 4 or 5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2-r17 if configured; - otherwise 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2

However, according to TS 38.331 V17.1.0 [1], as quoted below, the configuration of harq-ProcessNumberSizeDCI-0-2-v1700 can only be configured to indicate 5 bits

PUSCH-Config ::=                        SEQUENCE {

[…]

harq-ProcessNumberSizeDCI-0-2-r16       INTEGER (0..4)               OPTIONAL,  -- Need R

[…]

harq-ProcessNumberSizeDCI-0-2-v1700     INTEGER (5)                  OPTIONAL,  -- Need R

harq-ProcessNumberSizeDCI-0-1-r17       INTEGER (5)                  OPTIONAL,  -- Need R

[…]

We think it is not intended to have the configuration to indicate only 5 bits instead of configuring 0~5 bits. A reference is the case of DCI format 1_2, as quoted below:

PDSCH-Config ::=                        SEQUENCE {

[…]

harq-ProcessNumberSizeDCI-1-2-r16        INTEGER (0..4)              OPTIONAL,   -- Need R

[…]

harq-ProcessNumberSizeDCI-1-2-v1700      INTEGER (0..5)              OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-1-1-r17        INTEGER (5)                 OPTIONAL,   -- Need R

[…]

This is aligned with the description in TS 38.212:

- HARQ process number – number of bits determined by the following: - 0, 1, 2, 3, 4 or 5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-1-2-r17 if configured; - otherwise 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-1-2

To fix the misalignment of harq-ProcessNumberSizeDCI-0-2 between TS 38.331 and TS 38.212, we think the intended configuration of harq-ProcessNumberSizeDCI-0-2-v1700 should be INTEGER (0..5). A possible text proposal is provided below:

PUSCH-Config ::=                        SEQUENCE {

[…]

harq-ProcessNumberSizeDCI-0-2-r16       INTEGER (0..4)               OPTIONAL,  -- Need R

[…]

harq-ProcessNumberSizeDCI-0-2-v1700     INTEGER (0..5)              OPTIONAL,  -- Need R

harq-ProcessNumberSizeDCI-0-1-r17       INTEGER (5)                  OPTIONAL,  -- Need R

[…]

However, it may be difficult to make such correction after ASN.1 freeze. Another way to fix the misalignment is to change the description in TS 38.212. A draft CR to correct the description in TS 38.212 is provided in [3] for RAN1 discussion, which is quoted below:

- HARQ process number – number of bits determined by the following: - 5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2-r17 if configured; - otherwise 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2

But this may be less desirable since it creates another misalignment between harq-ProcessNumberSizeDCI-0-2 and harq-ProcessNumberSizeDCI-1-2. RAN2 is requested to discuss where to fix the misalignment in TS 38.331 or leave it to RAN1.

Proposal 1: Discuss where to fix the misalignment of harq-ProcessNumberSizeDCI-0-2:

- Option 1: In TS 38.331;

- Option 2: Leave it to RAN1.

If RAN2 decides to correct the misalignment in TS 38.331, the text proposal specified above could be adopted.

- Conclusion

In this contribution, we discuss how to fix the misalignment of harq-ProcessNumberSizeDCI-0-2 and have the following proposal:

Proposal 1: Discuss where to fix the misalignment of harq-ProcessNumberSizeDCI-0-2:

- Option 1: In TS 38.331;

- Option 2: Leave it to RAN1.

- Reference

- TS 38.331 V17.1.0, NR RRC specification.

- TS 38.212 V17.2.0.

- R1-2206233, Correction on HARQ process number for NTN.