3GPP TSG-RAN WG2 Meeting #119 electronic					      R2-2208396

Online, Aug 17-29, 2022

Source: 	vivo

Title:  	Discussion on FDM solution for R18 IDC

Agenda Item:	8.10.2

Document for:	Discussion and Decision

Introduction

In RAN#96 meeting, RAN approved the revised WID for R18 IDC [1]. The objectives are as follows:

This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi). - Enhancements to FDM solution, to allow more granular indication of affected frequencies (e.g. granularity of BWP or PRB level). (RAN2) Note: Enhancements to FDM solution is prioritized. - Introduction of TDM solution (e.g. indication of UE preferred TDM pattern for UL/DL). (RAN2, RAN4). Note: The TDM solution is considered complementary to the FDM solution. - Specify RRM requirements for TDM solution (RAN4) Note: LTE IDC solution should be considered as the baseline for the solutions developed in this WI.

In this contribution, we will discuss the FDM solution for R18 IDC including the following aspects, and give our proposals:

- R18 IDC framework for MR-DC;

- SRB for reporting FDM info;

- The granularity of FDM information

FDM solution for R18 IDC

R18 IDC framework for MR-DC

Based on the latest WID for R18 IDC, the R18 IDC project focus on the NR-DC and EN-DC architecture. According to 36.331 [2], IDC for the EN-DC scenario has been supported in LTE R15. The text of the LTE FDM solutions for EN-DC in the TS 36.331 is quoted as follows. The UE can report the EN-DC affected frequency to LTE. The NR affected frequency is reported via the “ARFCN-ValueNR”, and the LTE affected frequency is reported via the “MeasObjectId”. Moreover, UE also provides the victimsystemTyp field and interferenceDirectionMRDC field to NW. Even though the NR affected frequencies will be reported via finer granularity (e.g. granularity of BWP or PRB level) in R18 IDC, there is no problem that the R15 IDC framework for EN-DC is reused in R18 IDC.

Proposal 1: The R15 IDC framework for EN-DC is reused in R18 IDC.

In NR R16 TEI, RAN2 work on the FDM solution for NR-SA. The text of the R16 FDM solutions for NR-SA in the TS 38.331 is quoted as follows [3]. The affectedCarrierFreqCombList field is used to indicate a list of NR carrier frequencies combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA. Moreover, UE also provides the victimsystemTyp field to NW. This Rel-16 IDC framework for NR SA can be reused for NR DC in R18 IDC where an affectedCarrierFreqCombLis alike field can be used to indicate the affected NR frequency for NR DC.

Proposal 2: The Rel-16 IDC framework for NR SA is reused for NR-DC.

SRB for reporting FDM information

Currently, SRB3 is supported in EN-DC and NR-DC. Then, the potential issue is whether UE can report FDM information via SRB3. Based on the description of the TS 38.331 below, if UE is in EN-DC or NR-DC, UE will report the UEAssistanceInformation message via SRB3 if SRB3 is configured. As the IDC assistance information is carried by the UEAssistanceInformation message, reporting FDM info vias SRB3 has been supported from the NR specification perspective.

TS 38.331 1> else if the UE is in (NG)EN-DC: 2> if SRB3 is configured: 3> submit the UEAssistanceInformation message via SRB3 to lower layers for transmission; 2> else: 3> submit the UEAssistanceInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10]. 1> else if the UE is in NR-DC: 2> if the UE assistance configuration that triggered this UE assistance information is associated with the SCG: 3> if SRB3 is configured: 4> submit the UEAssistanceInformation message via SRB3 to lower layers for transmission; 3> else: 4> submit the UEAssistanceInformation message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

Proposal 3: FDM information is reported via SRB3 if SRB3 is configured.

The granularity of FDM information

In the current mechanism, the NR affected frequency is indicated via the “ARFCN-ValueNR”. Due to the wider bandwidth of the NR, if the UE has a certain frequency range affected, it is not very clear which NR ARFCN should be indicated by the UE and how many NR ARFCN(s) the UE should indicate. The simplest way is for UE to report the affected BWP info (e.g., BWP ID) for the affected NR frequency. In NR, 3 different BWP configuration is supported. That is BWP1 with a width of 40 MHz and subcarrier spacing of 15 kHz, BWP2 with a width of 10 MHz and subcarrier spacing of 15 kHz, and BWP3 with a width of 20 MHz and subcarrier spacing of 60 kHz. Considering that the bandwidth of the LTE cell is 20MHz, reporting BWP info can achieve comparable performance with LTE (e.g., NW handles the IDC issue by removing the affected LTE cell). Then, FDM information per BWP level is enough. In addition, on top of per BWP level indication of affected frequencies, per PRB level indication of affected frequencies seems not necessary. The reason is that it will complex the NW implementation if NW adjusts the radio resource based on the granularity of the PRB level to handle the IDC issue.

Proposal 4: FDM information is reported per BWP level.

In NR R16, NW can configure UE to report FDM assistance information via the IDC-AssistanceConfig field carried in the otherConfig IE. As shown below, the reporting function of FDM assistance information is configured per frequency. In R18, FDM information will be reported with finer granularity (e.g. BPW level). The configuration for the reporting function of FDM information should be correspondingly enhanced so that UE can perform different reporting behavior (e.g. BPW level) with the legacy mechanism.

IDC-AssistanceConfig-r16 ::=    SEQUENCE {

candidateServingFreqListNR-r16  CandidateServingFreqListNR-r16                     OPTIONAL, -- Need R

...

}

Proposal 5: The reporting of FDM information is configured per BWP level.

Conclusions

In this contribution, we will discuss the FDM solution for R18 IDC and give the following proposals.

Proposal 1: The R15 IDC framework for EN-DC is reused in R18 IDC.

Proposal 2: The Rel-16 IDC framework for NR SA is reused for NR DC.

Proposal 3: FDM information is reported via SRB3 if SRB3 is configured.

Proposal 4: FDM information is reported per BWP level.

Proposal 5: The reporting of FDM information is configured per BWP level.

Reference

- RP-221281 Revised WID: In-Device Co-existence (IDC) enhancements for NR and MR-DC.

- TS 36.331 Radio Resource Control (RRC).

- TS 38.331 Radio Resource Control (RRC).