3GPP TSG-RAN WG2 #119e                                                                                         R2-2208464

eMeeting, 17th – 26th August, 2022

Agenda Item:	    6.22

Source: 	    MediaTek Inc.

Title:		    Remaining Issues for MGE Configurations

Document for:	    Discussion and decision

# 1 Introduction

In this paper, we discussed the remaining open issues for MGE configurations.

# 2 Discussion

## 2.1 Pre-configured MG

RAN4 has discussed the configuration on NW-controlled pre-configured MG and inform RAN2 the following RAN4 observation in LS R4-2210587 [1].

RAN4 would like to inform RAN2 that RAN4 has reached the following conclusions related to Pre-configured MG during RAN4#103-e.

RAN4 understands that switching between BWP#0 without dedicated configuration and other BWPs can be done by RRC-based or timer-based BWP switching. RAN4 observes that the RRC-based Pre-MG (de)activation mechanism is implemented via IE BWP-DownlinkDedicated, which is not applicable to BWP#0 without dedicated configuration.

RAN4 asks RAN2 to confirm above observations. If confirmed, RAN4 understands that it is up to RAN2 on whether to resolve it.

We understand there are two observations from RAN4 LS.

- Observation 1: switching between BWP#0 without dedicated configuration and other BWPs is possible

- Observation 2: Pre-MG (de)activation controlling parameter (i.e. preConfGapStatus-r17) is defined in BWP-DownlinkDedicated and does not apply to “BWP#0 without dedicated configuration”.

We understand both observations from RAN4 LS are correct. Indeed, current ASN.1 put the controlling parameter in BWP-DownlinkDedicated, which does not apply to the scenario “BWP#0 without dedicated configuration”.

BWP-DownlinkDedicated ::=    SEQUENCE {

pdcch-Config                  SetupRelease { PDCCH-Config }                   OPTIONAL,   -- Need M

pdsch-Config                  SetupRelease { PDSCH-Config }                   OPTIONAL,   -- Need M

sps-Config                    SetupRelease { SPS-Config }                     OPTIONAL,   -- Need M

radioLinkMonitoringConfig     SetupRelease { RadioLinkMonitoringConfig }      OPTIONAL,   -- Need M

...,

<Skip>

[[

preConfGapStatus-r17        BIT STRING (SIZE (maxNrofGapId-r17))      OPTIONAL,   -- Cond PreConfigMG

<Skip>

]]

}

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated             OPTIONAL,   -- Cond TDD

initialDownlinkBWP           BWP-DownlinkDedicated                        OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList    SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id   OPTIONAL,   -- Need N

downlinkBWP-ToAddModList     SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink  OPTIONAL,  -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                          OPTIONAL,  -- Cond SyncAndCellAdd

<Skip>

}

However, we don’t really see any problem to be resolved even though the observations are correct. We understand that RAN4 was thinking whether more controlling parameter is needed for BWP Configuration option #1 described in 38.331 Annex B.2. The BWP Configuration option #1 (as illustrated in the following picture from 38.331 Annex B.2) is considered as BWP#0 without dedicated configuration.

Some companies in RAN4 think that controlling parameter of Pre-MG (de)activation status is NOT a dedicate configuration and additional signaling is needed for BWP configuration option 1. It is not clear to us why the pre-configured gap status is NOT dedicate configuration. It seems clear that the configuration is UE specific, not cell specific. Our understand is that for NW-controlled pre-configured MG, dedicate configuration is needed for BWP#0 and BWP configuration option 1 should not be used. As nothing is really broken in current SPEC, we suggest not to have further change for this scenario.

Proposal 1: For pre-configured MG configuration, RAN2 confirms that NO further SPEC change is needed for the BWP#0 scenario mentioned in LS R4-2210587.

## 2.2 Concurrent gap

For concurrent gap, RAN2 has added new list of gap configuration (gapToAddModList-r17) to add/modify multiple gap configurations, the field is also used to configure other new R17 features (pre-configured MG, NCSG). Current ASN.1 is defined as following.

MeasGapConfig ::=                   SEQUENCE {

gapFR2                              SetupRelease { GapConfig }      OPTIONAL,   -- Need M

...,

[[

gapFR1                              SetupRelease { GapConfig }      OPTIONAL,   -- Need M

gapUE                               SetupRelease { GapConfig }      OPTIONAL    -- Need M

]],

[[

gapToAddModList-r17    SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17  OPTIONAL,   -- Need N

gapToReleaseList-r17   SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17  OPTIONAL,   -- Need N

posMeasGapPreConfigToAddModList-r17    PosMeasGapPreConfigToAddModList-r17     OPTIONAL,   -- Need N

posMeasGapPreConfigToReleaseList-r17   PosMeasGapPreConfigToReleaseList-r17    OPTIONAL    -- Need N

]]

}

In last RAN2 meeting, there was discussion on whether the network could use the new fields (gapToAddModList-r17 and gapToReleaseList-r17) to configure a “legacy” gap [2]. Unfortunately, there is no conclusion on this topic. According to [2], they could be two way to configure “legacy” gap from signaling point of view.

- Type 1 gap configuration - A gap is setup by legacy fields (gapFR2, gapFR1, gapUE)

- Type 2 gap configuration - A gap is setup by new fields (gapToAddModList-r17). However, this gap is NOT configured with pre-MG indicator (preConfigInd-r17) and is NOT configured as NCSG (nscgInd-r17). In addition, this gap is not associated with specific frequencies. (It is the only per UE gap or FR1 gap or FR2 gap)

  - In this case, this “legacy” gap will be configured with a gap ID which is used for release purpose only.

There are some concerns to support type 2 gap configuration as some company prefers to keep legacy configuration in legacy field. On the other hand, some companies that think type 2 gap configuration would allow NW to do efficient one-step “transformation” of gaps.

For UE that does not support any of Rel-17 MGE features, it seems no clear motivation/benefit to use new field for legacy gap configuration. So, it would be good to confirm that the new field is NOT used for UEs not supporting MGE features. However, if the UE supports any of Rel-17 MGE features, it seems no difficult to support type 2 gap configuration. Therefore, we suggest RAN2 to confirm type 2 configuration is possible only for UEs supporting MGE features.

Proposal 2: RAN2 confirms that the new gap configuration fields (gapToAddModList-r17 and gapToReleaseList-r17) field could only be used for UE supporting any of Rel-17 MGE features. The network may configure legacy gap using new field for MGE-capable UE.

For concurrent gap, RAN2 agreed that it is possible that the NW configure one gap using legacy fields and another gap using new fields. In this case, RAN2 has specified some default association rule for gaps configured via legacy field. However, the association rule is not enough for RAN4 RRM requirement of concurrent gap. We also need priority definition while operating in multiple gap configuration. Since there is no priority in gap configured by legacy fields, we suggest to define a default value in this scenario.

Proposal 3: RAN2 to define default priority value for the gap configured via legacy fields (gapFR2, gapFR1, gapUE) while the gap is used for concurrent gap operation.

38.331 TP based P2 and P3 is provided in Annex A for reference.

# 3 Conclusions

Based on the discussion in section 2, we propose the following:

Proposal 1: For pre-configured MG configuration, RAN2 confirms that NO further SPEC change is needed for the BWP#0 scenario mentioned in LS R4-2210587.

Proposal 2: RAN2 confirms that the new gap configuration fields (gapToAddModList-r17 and gapToReleaseList-r17) field could only be used for UE supporting any of Rel-17 MGE features. The network may configure legacy gap using new field for MGE-capable UE.

Proposal 3: RAN2 to define default priority value for the gap configured via legacy fields (gapFR2, gapFR1, gapUE) while the gap is used for concurrent gap operation.

# 4 References

[1] R2-2206939/R4-2210587, “LS on R17 NR MG enhancements – Pre-configured MG”, RAN4

[2] R2-2206453, “Report of [AT118-e][059][MGE] Concurrent MG (MediaTek)”, MediaTek

# Annex: TP for 38.331

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/release of measurement gaps.

MeasGapConfig information element

-- ASN1START

-- TAG-MEASGAPCONFIG-START

MeasGapConfig ::=                   SEQUENCE {

gapFR2                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

...,

[[

gapFR1                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

gapUE                               SetupRelease { GapConfig }                                              OPTIONAL    -- Need M

]],

[[

gapToAddModList-r17           SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17                    OPTIONAL,   -- Need N

gapToReleaseList-r17          SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17                    OPTIONAL,   -- Need N

posMeasGapPreConfigToAddModList-r17      PosMeasGapPreConfigToAddModList-r17                                OPTIONAL,   -- Need N

posMeasGapPreConfigToReleaseList-r17     PosMeasGapPreConfigToReleaseList-r17                               OPTIONAL    -- Need N

]]

}

GapConfig ::=                       SEQUENCE {

gapOffset                           INTEGER (0..159),

mgl                                 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                                ENUMERATED {ms20, ms40, ms80, ms160},

mgta                                ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator                ENUMERATED {pCell, pSCell, mcg-FR2}                                 OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16           ServCellIndex                                                       OPTIONAL,   -- Cond AsyncCA

mgl-r16                             ENUMERATED {ms10, ms20}                                             OPTIONAL    -- Cond PRS

]]

}

GapConfig-r17 ::=                   SEQUENCE {

measGapId-r17                       MeasGapId-r17,

gapType-r17                         ENUMERATED {perUE, perFR1, perFR2},

gapOffset-r17                       INTEGER (0..159),

mgl-r17                             ENUMERATED {ms1, ms1dot5, ms2, ms3, ms3dot5, ms4, ms5, ms5dot5, ms6, ms10, ms20},

mgrp-r17                            ENUMERATED {ms20, ms40, ms80, ms160},

mgta-r17                            ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot75},

refServCellIndicator-r17            ENUMERATED {pCell, pSCell, mcg-FR2}                                 OPTIONAL,   -- Cond NEDCorNRDC

refFR2-ServCellAsyncCA-r17          ServCellIndex                                                       OPTIONAL,   -- Cond AsyncCA

preConfigInd-r17                    ENUMERATED {true}                                                   OPTIONAL,   -- Need R

ncsgInd-r17                         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

gapAssociationPRS-r17               ENUMERATED {true}                                                   OPTIONAL,   -- Need R

gapSharing-r17                      MeasGapSharingScheme                                                OPTIONAL,   -- Need R

gapPriority-r17                     GapPriority-r17                                                     OPTIONAL,   -- Need R

...

}

PosMeasGapPreConfigToAddModList-r17 ::= SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF PosGapConfig-r17

PosMeasGapPreConfigToReleaseList-r17 ::= SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF MeasPosPreConfigGapId-r17

PosGapConfig-r17 ::=                SEQUENCE {

measPosPreConfigGapId-r17           MeasPosPreConfigGapId-r17,

gapOffset-r17                       INTEGER (0..159),

mgl-r17                             ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

mgrp-r17                            ENUMERATED {ms20, ms40, ms80, ms160},

mgta-r17                            ENUMERATED {ms0, ms0dot25, ms0dot5},

gapType-r17                         ENUMERATED {perUE, perFR1, perFR2},

...

}

MeasPosPreConfigGapId-r17 ::= INTEGER (1..maxNrofPreConfigPosGapId-r17)

-- TAG-MEASGAPCONFIG-STOP

-- ASN1STOP

MeasGapConfig field descriptions

gapAssociationPRS Indicates that PRS measurement is associated with this measurement gap. The network only includes this field for one per UE gap. If concurrent gap (i.e. one of the gap combination as defined in Table 9.1.8-1 in TS 38.133 [14]) is configured and no gap is configured with this field, the PRS measurement is associated with the gap configured via gapUE, if available.

gapFR1 Indicates measurement gap configuration that applies to FR1 only. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 measurement gap). In NE-DC, gapFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap). In NR-DC, gapFR1 can only be set up in the measConfig associated with MCG. gapFR1 can not be configured together with gapUE. The applicability of the FR1 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14]. If more than one measurement gap is configured (i.e. concurrent measurement gap as specified in TS 38.133[14], clause 9.1.8) using this field, the measurement gap priority of gapFR1 is defined as value 1.

gapFR2 Indicates measurement gap configuration applies to FR2 only. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap). In NR-DC, gapFR2 can only be set up in the measConfig associated with MCG. gapFR2 cannot be configured together with gapUE. The applicability of the FR2 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14]. If more than one measurement gap is configured (i.e. concurrent measurement gap as specified in TS 38.133[14], clause 9.1.8) using this field, the measurement gap priority of gapFR2 is defined as value 1.

gapOffset Value gapOffset is the gap offset of the gap pattern with MGRP indicated in the field mgrp. The value range is from 0 to mgrp-1. If ncsgInd-r17 is present, this offset value refers to the starting point of VIL1 (the visible interruption length before the ML).

gapPriority Indicates the priority of this measurement gap (see TS 38.133 [14], clause FFS). Value 1 indicates highest priority, value 2 indicates second level priority, and so on.

gapSharing Indicates the measurement gap sharing scheme that applies to this GapConfig. For applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapToAddModList A list of of measurement gap configuration to be added or modified. If more than one measurement gap is configured (i.e. concurrent measurement gap as specified in TS 38.133[14], clause 9.1.8), the maximum number of configured measurement gap is limited by the gap combinations defined in Table 9.1.8-1 in TS 38.133 [14]. The network configures at most one NCSG or pre-configured measurement gap for a given gap type. In this version of the specification, the network configures this field only in NR standalone. This field is used only for a UE that supports pre-configured measurement gap, concurrent measurement gap, or NCSG.

gapToReleaseList A list of measurement gap configuration to be released.

gapType Indicates the type of this measurement gap. Value perUE indicates that it is a per UE measurement gap, value perFR1 indicates that it is an FR1 measurement gap, and value perFR2 indicates that it is an FR2 measurement gap.

gapUE Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). In (NG)EN-DC, gapUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE measurement gap). In NE-DC, gapUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap). In NR-DC, gapUE can only be set up in the measConfig associated with MCG. If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. The applicability of the per UE measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14]. If more than one measurement gap is configured (i.e. concurrent measurement gap as specified in TS 38.133[14], clause 9.1.8) using this field, the measurement gap priority of gapUE is defined as value 1.

measGapId The ID of this measurement gap configuration.

mgl Value mgl is the measurement gap length in ms of the measurement gap. If ncsgInd-r17 is not present, the measurement gap length is according to in Table 9.1.2-1 in TS 38.133 [14]. If ncsgInd-r17 is present, this field indicates the measurement length (ML) in NCSG pattern and is configured according to Table 9.1.2C-1 in TS 38.133 [14]. Value ms1dot5 corresponds to 1.5 ms, ms3 corresponds to 3 ms and so on. If mgl-r16 is present, UE shall ignore the mgl (without suffix). Value ms1, ms2, and ms5 can only be configured if ncsgInd is present.

mgrp Value mgrp is measurement gap repetition period in (ms) of the measurement gap. The measurement gap repetition period is according to Table 9.1.2-1 in TS 38.133 [14].

mgta Value mgta is the measurement gap timing advance in ms. The applicability of the measurement gap timing advance is according to clause 9.1.2 of TS 38.133 [14]. Value ms0 corresponds to 0 ms, ms0dot25 corresponds to 0.25 ms and ms0dot5 corresponds to 0.5 ms. For FR2, the network only configures 0 ms and 0.25 ms. If ncsgInd is present, value ms0dot25 can not be configured. Value ms0dot75 can only be configured if ncsgInd is present.

ncsgInd Indicates that the measurement gap is a NCSG as specified in 38.133 [14].

posMeasGapPreConfigToAddModList List of preconfigured measurement gap for positioning to add and/or modify. All the gaps configured are associated with the measurement of PRS for RSTD, UE-RxTx Time Difference, PRS-RSRP and PRS-RSRPP as defined in TS 38.215 [9].

posMeasGapPreConfigToReleasList List of preconfigured measurement gap for positioning to release.

preConfigInd Indicates whether the measurement gap is a pre-configured measurement gap.

refFR2ServCellAsyncCA Indicates the FR2 serving cell identifier whose SFN and subframe is used for FR2 gap calculation for this gap pattern with asynchronous CA involving FR2 carrier(s).

refServCellIndicator Indicates the serving cell whose SFN and subframe are used for gap calculation for this gap pattern. Value pCell corresponds to the PCell, pSCell corresponds to the PSCell, and mcg-FR2 corresponds to a serving cell on FR2 frequency in MCG.

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present when configuring FR2 gap pattern to UE in: - (NG)EN-DC or NR SA with asynchronous CA involving FR2 carrier(s); - NE-DC or NR-DC with asynchronous CA involving FR2 carrier(s), if the field refServCellIndicator is set to mcg-FR2. In case the gap pattern to UE in NE-DC and NR-DC is already configured and the serving cell used for the gap calculation corresponds to a serving cell on FR2 frequency in MCG, then the field is optionally present, need M. Otherwise, it is absent, Need R. |
| NEDCorNRDC | This field is mandatory present when configuring gap pattern to UE in NE-DC or NR-DC. In case the gap pattern to UE in NE-DC and NR-DC is already configured, then the field is absent, need M. Otherwise, it is absent. |
| PRS | This field is optionally present, Need R, when configuring gap pattern to UE for measurements of DL-PRS configured via LPP (TS 37.355 [49]). Otherwise, it is absent. |