3GPP TSG-RAN WG2 #119e                                                                                         R2-2208477

eMeeting, 17th – 26th August, 2022

Agenda Item:	    8.4.3

Source: 	    MediaTek Inc.

Title:		    Discussion on selective activation of CG

Document for:	    Discussion and decision

# 1 Introduction

In this contribution, we discuss the following objective #2 from mobility enhancement work item [1].

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

We would like to discuss the supporting scenario for this selective activation procedure and try to establish some common RAN2 understanding on this objective.

# 2 Discussion

## 2.1 Supporting scenario for selective activation

According to the WID description, R18 mobility enhancement target to support selective activation of CG at least for SCG. We understand that the motivation of this objective comes from the frequent change of SCG (especially on FR2). By allowing subsequent SCG change, it can reduce signalling overhead and potentially reduced interruption time for SCG change. On other hand, the motivation of selective CG activation for MCG is not clear to us. The design for MCG could be more complicate as now there is no anchor communication with network during MCG change. Our preference is to discuss the procedure for supporting SCG selective CG activation first and MCG part could be discussed if there is additional time.

Proposal 1: For obj#2 in R18 mobility WI, RAN2 prioritizes the selective activation of cell groups for SCG.

## 2.2 Triggering of SCG activation

Assuming that we are going to activate one SCG (from multiple candidate SCG) dynamically, the first question is how to control/trigger the activation of the target SCG. Fundamentally, there could be two options:

- Option 1 – Based on NW signaling (e.g. via RRC signaling)

- Option 2 – Based on Pre-defined condition (CPC-like)

The WID description itself is not clear on whether the activation is based on NW signaling or UE predefined condition. Since one of the main motivations to have this objective is to reduce signaling overhead, we believe that option 1 may not be suitable for this enhancement. If RRC message is needed to trigger the activation of one SCG, there is no much difference compared to legacy SCG change procedure. As the WID also mentioned “without reconfiguration and re-initiation of CPC/CPA”, our preference is to use CPC-like triggering condition (option 2). We suggest RAN2 to go with this direction for obj#2.

Proposal 2: RAN2 understands that selective activation of cell groups is triggered by pre-defined condition (CPC-like behavior).

## 2.3 Subsequent SCG Change

With the understanding from P1 and P2, we think that the key discussion point for selective SCG activation is how to support the subsequent SCG change without RRC Reconfiguration message. Taking R16/R17 CPC/CPA as baseline, the UE could be configured with multiple candidate PSCells associated with corresponding triggering condition. PSCell add or change would be performed once the condition is met. In our view, the basic flow of subsequent SCG change after CPC could be illustrated as below.

Figure 1 – Subsequent SCG Change after CPC

Similar to CPC, we could also have subsequent SCG Change after CPA, the basic follow is shown below.

Figure 2 – Subsequent SCG Change after CPA

The procedure of subsequent SCG change is very similar to current CPC procedure. The NW will configure some candidate PSCell which is associated with some trigger condition and predefined reconfiguration. Once the trigger condition is met, the UE perform SCG change based on the predefined reconfiguration. The key difference is that the UE will keep all candidate configuration (the trigger condition and predefined configuration) after the SCG change.

RAN2 could further discuss the RRC model of the “predefined reconfiguration”. In CPC, it is a complete RRC Reconfiguration message. In selective SCG activation, whether we need whole RRC message could be further studied. In addition, the delta configuration aspect should be taken into account as now UE will keep the predefined reconfiguration even after applying the predefined configuration from other candidate. The trigger condition from CPC could be reused as baseline. RAN2 may have to study how to reuse the trigger condition in subsequent SCG change.

Therefore, we propose to confirm the CPC-like flow with some FFS point on predefined reconfiguration and triggering condition.

Proposal 3: RAN2 considers the basic flow of selective SCG activation as

- The NW configure one or more candidate PSCell. Each candidate PSCell is associated with some trigger condition and predefined reconfiguration.

- Once the trigger condition is met, the UE perform SCG change based on the predefined reconfiguration. The UE keep all candidate configurations after this SCG change.

- FFS on details of trigger condition

- FFS on how to model the predefined reconfiguration

# 3 Conclusions

Based on the discussion in section 2, we propose the following proposals:

Proposal 1: For obj#2 in R18 mobility WI, RAN2 prioritizes the selective activation of cell groups for SCG.

Proposal 2: RAN2 understands that selective activation of cell groups is triggered by pre-defined condition (CPC-like behavior).

Proposal 3: RAN2 considers the basic flow of selective SCG activation as

- The NW configure one or more candidate PSCell. Each candidate PSCell is associated with some trigger condition and predefined reconfiguration.

- Once the trigger condition is met, the UE perform SCG change based on the predefined reconfiguration. The UE keep all candidate configurations after this SCG change.

- FFS on details of trigger condition

- FFS on how to model the predefined reconfiguration

# 4 References

[1] RP-221799, “Revised WID on Further NR mobility enhancements”, MediaTek