3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2208493

e-Meeting, 17th August – 29th August 2022

Source:	vivo, ZTE, Ericsson, Huawei, Xiaomi Communications

Title:          Discussion on the format of on-demand PRS configuration

Agenda Item:	6.11.2.3

Document for:	Discussion and Decision

- Introduction

At RAN2#114e meeting, it was agreed that RAN1 was responsible to decide and provide the list of possible parameters for on-demand DL-PRS. RAN1 made the following agreement regarding the possible parameters at the RAN1#107e meeting:

Agreement - From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per TRP - Start/end time of DL PRS transmission - either per resource, or per resource set, or per TRP - ON/OFF indicator (for LMF initiated request only) Agreement - From RAN1 perspective, for UE-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signalled - per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - per FR - Number of DL PRS frequency layers - per UE - Start/end time of DL PRS transmission Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - UE recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - UE requests to provide the QCL information in the assistance data

In this contribution, we will provide our view on the on-demand PRS configuration in the LPP ProvideAssistanceData message to align with RAN1.

- Discussion

The procedure for on-demand PRS is as below (extracted from [1]):

Figure 1: Procedures for On-Demand PRS request.

0. The LMF may receive information on the possible On-Demand PRS configurations that the gNB can support during the TRP Information Exchange procedure. 1. In case of UE-initiated On-Demand PRS, the LMF may configure the UE with pre-defined PRS configurations via LPP Provide Assistance Data message or via posSI. 2a. In case of UE-initiated On-Demand PRS, the UE sends an On-Demand PRS request to the LMF via LPP Request Assistance Data message. The On-Demand PRS request can be a request for a pre-defined PRS configuration indicated with pre-defined PRS configuration ID or explicit parameter for PRS configuration and may be a request for PRS transmission or change to the PRS transmission characteristics for positioning measurements. NOTE 1: The LPP Request Assistance Data message for On-Demand PRS may also be sent in an MO-LR location service request message. NOTE 2: If the NW has provided the pre-defined On-Demand PRS configurations to the UE, the UE is allowed to request On-Demand PRS parameters based on pre-defined PRS configuration ID (index-based request) or explicit parameter requests that is within the scope of the received pre-defined On-Demand PRS configurations. Otherwise, the UE may blindly request On-Demand PRS parameters via an explicit request within the scope of the allowed parameter list, as specified in TS37.355 [42]. 2b. In case of LMF-initiated On-Demand PRS, the LMF and the UE may exchange LPP messages e.g., to obtain UE measurements or the DL-PRS positioning capabilities of the UE, etc. 3. The LMF determines the need for PRS transmission or change to the transmission characteristics of an ongoing PRS transmission. 4. The LMF requests the serving and non-serving gNBs/TRPs for new PRS transmission or PRS transmission with changes to the PRS configuration via NRPPa PRS CONFIGURATION REQUEST message. 5. The gNBs/TRPs provide the PRS transmission update in the NRPPa PRS CONFIGURATION RESPONSE message accordingly. 6. LMF may provide the updated PRS configuration used for PRS transmission or error cause via LPP Provide Assistance Data message to the UE. NOTE 3: If the LPP Request Assistance Data for On-Demand DL-PRS at Step 2a was sent in an MO-LR location service request message, the LMF provides a MO-LR response as described in clause 7.3.3. NOTE 4: It is up to Network (LMF) implementation on the steps to follow (accept/reject/ignore) on receiving UE-initiated On-Demand PRS request. NOTE 5: It is up to Network (TRP) implementation on the steps to follow (accept/reject/ignore) on receiving LMF-initiated On-Demand PRS requests.

In step 0, the possible On-Demand PRS configurations are provided in the On-demand PRS TRP Information of TRP Information IE as below:

- Allowed Resource Set Periodicity Values

- Allowed PRS Bandwidth Values

- Allowed Resource Repetition Factor Values

- Allowed Resource Number of Symbols Values

- Allowed Comb Size Values

- Number of Frequency Layers

- Start Time and Duration

- Off Indication

- QCL Information

| IE/Group Name | Semantics Description |
| --- | --- |
| On-demand PRS Request Allowed | Each position in the bitmap represents an on-demand PRS transmission parameter: first bit: Resource Set Periodicity second bit: PRS Bandwidth third bit: Resource Repetition Factor fourth bit: Resource Number of Symbols fifth bit: Comb Size sixth bit: Number of Frequency Layers seventh bit: Start Time and Duration eighth bit: Off Indication ninth bit: QCL Information Other bits reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. |
| Allowed Resource Set Periodicity Values | This IE applies only if the first bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Resource Set Periodicity IE defined in subclause 9.2.61, first bit = 4 and so on. Bits 21-24 are reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Resource Set Periodicity values are allowed to be requested. |
| Allowed PRS Bandwidth Values | This IE applies only if the second bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the PRS Bandwidth IE defined in subclause 9.2.61, first bit = 1 and so on. Bit 64 is reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all PRS Bandwidth values are allowed to be requested. |
| Allowed Resource Repetition Factor Values | This IE applies only if the third bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Resource Repetition Factor IE defined in subclause 9.2.61, first bit = rf1 and so on. Bit 8 is reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Resource Repetition Factor values are allowed to be requested. |
| Allowed Resource Number of Symbols Values | This IE applies only if the fourth bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Resource Number of Symbols IE defined in subclause 9.2.61, first bit = n2 and so on. Bits 5-8 are reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Resource Number of Symbols values are allowed to be requested. |
| Allowed Comb Size Values | This IE applies only if the fifth bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Comb Size IE defined in subclause 9.2.61, first bit = 2 and so on. Bits 5-8 are reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Comb Size values are allowed to be requested. |

In comparison, we found that the parameters of the possible on-demand PRS configuration in NRPPa are the same as those agreed by RAN1.

Observation 1: The parameters of the possible on-demand PRS configuration in NRPPa are the same as the parameters agreed by RAN1.

For the pre-defined on-demand PRS configuration from LMF to UE in step 1, the structure is as follows:

On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE { dl-prs-configuration-id-r17 DL-PRS-Configuration-ID-r17, nr-DL-PRS-PositioningFrequencyLayer-r17 NR-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-Info-r17 NR-DL-PRS-Info-r16, ... } NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE { dl-PRS-SubcarrierSpacing-r16 ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...}, dl-PRS-ResourceBandwidth-r16 INTEGER (1..63), dl-PRS-StartPRB-r16 INTEGER (0..2176), dl-PRS-PointA-r16 ARFCN-ValueNR-r15, dl-PRS-CombSizeN-r16 ENUMERATED {n2, n4, n6, n12, ...}, dl-PRS-CyclicPrefix-r16 ENUMERATED {normal, extended, ...}, ... } NR-DL-PRS-Info-r16 ::= SEQUENCE { nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF NR-DL-PRS-ResourceSet-r16, ... } NR-DL-PRS-ResourceSet-r16 ::= SEQUENCE { nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16, dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16, dl-PRS-ResourceRepetitionFactor-r16 ENUMERATED {n2, n4, n6, n8, n16, n32, ...} OPTIONAL, -- Need OP dl-PRS-ResourceTimeGap-r16 ENUMERATED {s1, s2, s4, s8, s16, s32, ...} OPTIONAL, -- Cond Rep dl-PRS-NumSymbols-r16 ENUMERATED {n2, n4, n6, n12, ...}, dl-PRS-MutingOption1-r16 DL-PRS-MutingOption1-r16 OPTIONAL, -- Need OP dl-PRS-MutingOption2-r16 DL-PRS-MutingOption2-r16 OPTIONAL, -- Need OP dl-PRS-ResourcePower-r16 INTEGER (-60..50), dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16, ... } NR-DL-PRS-Resource-r16 ::= SEQUENCE { nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16, dl-PRS-SequenceID-r16 INTEGER (0.. 4095), dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { n2-r16 INTEGER (0..1), n4-r16 INTEGER (0..3), n6-r16 INTEGER (0..5), n12-r16 INTEGER (0..11), ... },

The NR-DL-PRS-Info in the pre-defined on-demand PRS configuration includes three types of parameters:

- The green ones are the parameters agreed by RAN1, e.g., Periodicity, PRS Bandwidth, Repetition Factor, and Resource Number of Symbols.

- The yellow ones are not agreed by RAN1 but are optional, e.g., the resourceTimeGap

- The red ones are not agreed by RAN1 and are mandatory, e.g., the Resource Set Slot Offset, the Resource Power, Sequence ID

Observation 2: The parameters of the pre-defined on-demand PRS configuration in LPP are not aligned with RAN1.

For the parameters that are not agreed by RAN1, one issue is how LMF obtains and determines the values of these parameters. The on-demand PRS request may be a request for PRS transmission, which implies that there can be no ongoing PRS transmission. Thus the LMF cannot obtain the current values of these parameters. In our understanding, the LMF is not expected to determine the explicit value of the radio resource parameter.

During the RAN2 #118e meeting, it was a consensus that the pre-defined PRS configuration was not associated with any TRP. On top of that, it was agreed that predefined on-demand DL-PRS configuration did not include TRP-related information. Even if the LMF would like to determine the values, another issue is how could the time-domain offset and the sequence ID of different TRPs be the same, and the value of which TRP will be included in the pre-defined configuration.

Besides, all these parameters not decided by RAN1 are not in the requested transmission characteristics from LMF to TRPs. That is, the exchange between LMF and UE on these parameters is useless and will introduce unnecessary overhead.

To address the above issues, we propose to align with RAN1 on the possible parameters.

Observation 3: The LMF is not expected to determine the value of the parameters in the pre-defined on-demand PRS configuration that are not agreed by RAN1.

Proposal1: Align with RAN1 on the parameters of the pre-defined on-demand PRS configuration in LPP.

As to the UE-initiated on-demand PRS request, two types are allowed:

- Type 1: index-based request. The request is based on a pre-defined PRS configuration ID, which is associated with a list of pre-defined parameters.

- Type 2: explicit parameter request.

The IE NR-On-Demand-DL-PRS-Information was introduced for the Type 2 request and its parameters are the same as the parameters agreed by RAN1. In our understanding, Type 1 is used to reduce the signaling overhead of request, and the pre-defined parameters associated with the ID shall be the same as Type 2. Therefore, one straightforward way is reusing the NR-On-Demand-DL-PRS-Information in pre-defined on-demand PRS configuration for Type 1 requests. In this case, how to handle the nr-DL-PRS-PositioningFrequencyLayer-r17 and nr-DL-PRS-Info-r17 is another issue. There can be 2 alternatives: turn them into dummy IEs or remove them directly. Note that the ASN.1 is frozen, it will introduce an NBC issue if we remove them directly.

Another way is adding some clarification for the On-Demand-DL-PRS-Configuration, e.g., LMF may fill arbitrary values for these parameters not agreed by RAN1 and the UE shall ignore them.

Proposal 2: Discuss how to address the on-demand PRS configuration issue in LPP, the potential options include:

- Option 1: Reuse the nr-On-Demand-DL-PRS-Information IE to describe the pre-defined on-demand PRS configuration, turn the nr-DL-PRS-PositioningFrequencyLayer-r17 and nr-DL-PRS-Info-r17 into dummy IEs.

- Option 2: Reuse the nr-On-Demand-DL-PRS-Information IE to describe the pre-defined on-demand PRS configuration, remove the nr-DL-PRS-PositioningFrequencyLayer-r17 and nr-DL-PRS-Info-r17. (NBC change)

- Option 3: Add clarification in the field description of On-Demand-DL-PRS-Configuration, e.g., LMF may fill arbitrary values for these parameters not agreed by RAN1 and the UE shall ignore them.

As to the stage2 procedure, in steps 2a, 3 and 4, the LMF/UE could request for new PRS transmission or change to an ongoing PRS transmission. However, only the request to change the ongoing PRS transmission was covered in steps 5 and 6. Thus, we propose the following change request:

5. The gNBs/TRPs provide the successfully configured or updated PRS transmission in the NRPPa PRS CONFIGURATION RESPONSE message accordingly. 6. LMF may provide the PRS configuration used for PRS transmission or error cause via LPP Provide Assistance Data message to the UE.

Proposal 3: Add the case of new PRS transmission in the stage2 procedures of on-demand PRS response.

- Conclusion

Observation 1: The parameters of the possible on-demand PRS configuration in NRPPa are the same as the parameters agreed by RAN1.

Observation 2: The parameters of the pre-defined on-demand PRS configuration in LPP are not aligned with RAN1.

Observation 3: The LMF is not expected to determine the value of the parameters in the pre-defined on-demand PRS configuration that are not agreed by RAN1.

Proposal1: Align with RAN1 on the parameters of the pre-defined on-demand PRS configuration in LPP.

Proposal 2: Discuss how to address the on-demand PRS configuration issue in LPP, the potential options include:

- Option 1: Reuse the nr-On-Demand-DL-PRS-Information IE to describe the pre-defined on-demand PRS configuration, turn the nr-DL-PRS-PositioningFrequencyLayer-r17 and nr-DL-PRS-Info-r17 into dummy IEs.

- Option 2: Reuse the nr-On-Demand-DL-PRS-Information IE to describe the pre-defined on-demand PRS configuration, remove the nr-DL-PRS-PositioningFrequencyLayer-r17 and nr-DL-PRS-Info-r17. (NBC change)

- Option 3: Add clarification in the field description of On-Demand-DL-PRS-Configuration, e.g., LMF may fill arbitrary values for these parameters not agreed by RAN1 and the UE shall ignore them.

Proposal 3: Add the case of new PRS transmission in the stage2 procedures of on-demand PRS response.

The corresponding text proposals are attached in the annex.

- Reference

- TS 38305		Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN

- Annex

Option 1

–	NR-On-Demand-DL-PRS-Configurations

The IE NR-On-Demand-DL-PRS-Configurations provide a set of possible DL-PRS configurations which can be requested by the target device on-demand.

-- ASN1START

NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE {

on-demand-dl-prs-configuration-list-r17		SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF

On-Demand-DL-PRS-Configuration-r17,

...

}

On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE {

dl-prs-configuration-id-r17					DL-PRS-Configuration-ID-r17,

dummy1		NR-DL-PRS-PositioningFrequencyLayer-r16,

dummy2							NR-DL-PRS-Info-r16,

...

nr-on-demand-DL-PRS-Information-r17 			NR-On-Demand-DL-PRS-Information-r17,

}

DL-PRS-Configuration-ID-r17 ::= SEQUENCE {

nr-dl-prs-configuration-id-r17				INTEGER (1..maxOD-DL-PRS-Configs-r17),

...

}

-- ASN1STOP

NR-DL-PRS-On-Demand-Configurations field descriptions

dl-prs-configuration-id This field provides an identity for the On-Demand-DL-PRS-Configuration.

dummy1 This field is not used in the specification. If received it shall be ignored by the UE.

dummy2 This field is not used in the specification. If received it shall be ignored by the UE.

–	NR-On-Demand-DL-PRS-Information

The IE NR-On-Demand-DL-PRS-Information defines the possible or requested on-demand DL-PRS.

-- ASN1START

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-On-Demand-DL-PRS-PerFreqLayer-r17

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRange-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicity-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...}

OPTIONAL,

dl-prs-ResourceBandwidth-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactor-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbols-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-CombSizeN-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-Information-r17			DL-PRS-QCL-Information-r17			OPTIONAL,

...

}

DL-PRS-QCL-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

DL-PRS-QCL-InformationPerTRP-r17

DL-PRS-QCL-InformationPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17						INTEGER (0..255),

nr-PhysCellID-r17					NR-PhysCellID-r16							OPTIONAL,

nr-CellGlobalID-r17					NCGI-r15									OPTIONAL,

nr-ARFCN-r17						ARFCN-ValueNR-r15							OPTIONAL,

dl-prs-QCL-InformationSet-r17	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-QCL-Info-r17,

...

}

DL-PRS-QCL-Info-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r17			NR-DL-PRS-ResourceSetID-r16,

dl-prs-QCL-Information-r17		CHOICE {

nr-DL-PRS-QCL-Source-r17			DL-PRS-QCL-Info-r16,

dl-prs-QCL-Info-requested-r17		NULL

},

...

}

-- ASN1STOP

NR-On-Demand-DL-PRS-Information field descriptions

dl-prs-FrequencyRange This field specifies the frequency range for which the on-demand DL-PRS is possible or requested.

dl-prs-ResourceSetPeriodicity This field specifies the possible or requested periodicity of the DL-PRS Resource Set in slots. The periodicity depends on the subcarrier spacing (SCS) and takes values 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for SCS of 15, 30, 60 and 120 kHz respectively. μ refers to the target devices current primary cell.

dl-prs-ResourceBandwidth This field specifies the possible or requested number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-prs-ResourceRepetitionFactor This field specifies the possible or requested DL-PRS Resource repetition. Enumerated values n2, n4, n6, n8, n16, n32 correspond to 2, 4, 6, 8, 16, 32 resource repetitions, respectively.

dl-prs-NumSymbols This field specifies the possible or requested number of symbols per DL-PRS Resource within a slot.

dl-prs-CombSizeN This field specifies the possible or requested Resource Element spacing in each symbol of the DL-PRS Resource.

dl-prs-QCL-Information This field specifies the possible, recommended or requested QCL indication with other DL reference signals. - dl-PRS-ID indicates the DL-PRS ID of the TRP for which the QCL information is possible or recommended. - nr-PhysCellID indicates the physical Cell-ID of the TRP for which the QCL information is possible or recommended, as defined in TS 38.331 [35]. - nr-CellGlobalID indicates the NCGI, the globally unique identity of a cell in NR, of the TRP for which the QCL information is possible or recommended, as defined in TS 38.331 [35]. - nr-ARFCN indicates the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. - dl-prs-QCL-InformationSet indicates the possible or recommended QCL information per DL-PRS Resource Set. - nr-DL-PRS-ResourceSetID indicates the DL-PRS Resource Set ID for which the nr-DL-PRS-QCL-Source is possible or recommended - nr-DL-PRS-QCL-Source indicates the QCL source. - dl-prs-QCL-Info-requested indicates that the UE requests to provide the QCL information in the assistance data.

Option 2

–	NR-On-Demand-DL-PRS-Configurations

The IE NR-On-Demand-DL-PRS-Configurations provides a set of possible DL-PRS configurations which can be requested by the target device on-demand.

-- ASN1START

NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE {

on-demand-dl-prs-configuration-list-r17		SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF

On-Demand-DL-PRS-Configuration-r17,

...

}

On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE {

dl-prs-configuration-id-r17					DL-PRS-Configuration-ID-r17,

...	nr-on-demand-DL-PRS-Information-r17 			NR-On-Demand-DL-PRS-Information-r17

}

DL-PRS-Configuration-ID-r17 ::= SEQUENCE {

nr-dl-prs-configuration-id-r17				INTEGER (1..maxOD-DL-PRS-Configs-r17),

...

}

-- ASN1STOP

NR-DL-PRS-On-Demand-Configurations field descriptions

dl-prs-configuration-id This field provides an identity for the On-Demand-DL-PRS-Configuration.

–	NR-On-Demand-DL-PRS-Information

The IE NR-On-Demand-DL-PRS-Information defines the possible or requested on-demand DL-PRS.

-- ASN1START

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-On-Demand-DL-PRS-PerFreqLayer-r17

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRange-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicity-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...}

OPTIONAL,

dl-prs-ResourceBandwidth-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactor-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbols-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-CombSizeN-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-Information-r17			DL-PRS-QCL-Information-r17			OPTIONAL,

...

}

DL-PRS-QCL-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

DL-PRS-QCL-InformationPerTRP-r17

DL-PRS-QCL-InformationPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17						INTEGER (0..255),

nr-PhysCellID-r17					NR-PhysCellID-r16							OPTIONAL,

nr-CellGlobalID-r17					NCGI-r15									OPTIONAL,

nr-ARFCN-r17						ARFCN-ValueNR-r15							OPTIONAL,

dl-prs-QCL-InformationSet-r17	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-QCL-Info-r17,

...

}

DL-PRS-QCL-Info-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r17			NR-DL-PRS-ResourceSetID-r16,

dl-prs-QCL-Information-r17		CHOICE {

nr-DL-PRS-QCL-Source-r17			DL-PRS-QCL-Info-r16,

dl-prs-QCL-Info-requested-r17		NULL

},

...

}

-- ASN1STOP

NR-On-Demand-DL-PRS-Information field descriptions

dl-prs-FrequencyRange This field specifies the frequency range for which the on-demand DL-PRS is possible or requested.

dl-prs-ResourceSetPeriodicity This field specifies the possible or requested periodicity of the DL-PRS Resource Set in slots. The periodicity depends on the subcarrier spacing (SCS) and takes values 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for SCS of 15, 30, 60 and 120 kHz respectively. μ refers to the target devices current primary cell.

dl-prs-ResourceBandwidth This field specifies the possible or requested number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-prs-ResourceRepetitionFactor This field specifies the possible or requested DL-PRS Resource repetition. Enumerated values n2, n4, n6, n8, n16, n32 correspond to 2, 4, 6, 8, 16, 32 resource repetitions, respectively.

dl-prs-NumSymbols This field specifies the possible or requested number of symbols per DL-PRS Resource within a slot.

dl-prs-CombSizeN This field specifies the possible or requested Resource Element spacing in each symbol of the DL-PRS Resource.

dl-prs-QCL-Information This field specifies the possible, recommended or requested QCL indication with other DL reference signals. - dl-PRS-ID indicates the DL-PRS ID of the TRP for which the QCL information is possible or recommended. - nr-PhysCellID indicates the physical Cell-ID of the TRP for which the QCL information is possible or recommended, as defined in TS 38.331 [35]. - nr-CellGlobalID indicates the NCGI, the globally unique identity of a cell in NR, of the TRP for which the QCL information is possible or recommended, as defined in TS 38.331 [35]. - nr-ARFCN indicates the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. - dl-prs-QCL-InformationSet indicates the possible or recommended QCL information per DL-PRS Resource Set. - nr-DL-PRS-ResourceSetID indicates the DL-PRS Resource Set ID for which the nr-DL-PRS-QCL-Source is possible or recommended - nr-DL-PRS-QCL-Source indicates the QCL source. - dl-prs-QCL-Info-requested indicates that the UE requests to provide the QCL information in the assistance data.

Option 3

–	NR-On-Demand-DL-PRS-Configurations

The IE NR-On-Demand-DL-PRS-Configurations provides a set of possible DL-PRS configurations which can be requested by the target device on-demand.

-- ASN1START

NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE {

on-demand-dl-prs-configuration-list-r17		SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF

On-Demand-DL-PRS-Configuration-r17,

...

}

On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE {

dl-prs-configuration-id-r17					DL-PRS-Configuration-ID-r17,

nr-DL-PRS-PositioningFrequencyLayer-r17		NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-Info-r17							NR-DL-PRS-Info-r16,

...

}

DL-PRS-Configuration-ID-r17 ::= SEQUENCE {

nr-dl-prs-configuration-id-r17				INTEGER (1..maxOD-DL-PRS-Configs-r17),

...

}

-- ASN1STOP

NR-DL-PRS-On-Demand-Configurations field descriptions

dl-prs-configuration-id This field provides an identity for the On-Demand-DL-PRS-Configuration.

nr-DL-PRS-PositioningFrequencyLayer This field, in addition to the nr-DL-PRS-Info, provides the possible DL-PRS configuration. The UE shall ignore the dl-PRS-SubcarrierSpacing, dl-PRS-StartPRB, dl-PRS-PointA and dl-PRS-CyclicPrefix in this field.

nr-DL-PRS-Info This field, in addition to the nr-DL-PRS-PositioningFrequencyLayer, provides the possible DL-PRS configuration. The UE shall ignore the nr-DL-PRS-ResourceSetID, Resource Set Slot Offset of dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-ResourceTimeGap, dl-PRS-MutingOption1, dl-PRS-MutingOption2, dl-PRS-ResourcePower, nr-DL-PRS-ResourceIDand, dl-PRS-SequenceID and Resource offset of dl-PRS-CombSizeN-AndReOffset in this field.

Change request to the 38.305

7.6.2	On-Demand PRS transmission procedures

*<text omitted>*

3.	The LMF determines the need for PRS transmission or change to the transmission characteristics of an ongoing PRS transmission.

4.	The LMF requests the serving and non-serving gNBs/TRPs for new PRS transmission or PRS transmission with changes to the PRS configuration via NRPPa PRS CONFIGURATION REQUEST message.

5.	The gNBs/TRPs provide the successfully configured or updated PRS transmission in the NRPPa PRS CONFIGURATION RESPONSE message accordingly.

6.	LMF may provide the PRS configuration used for PRS transmission or error cause via LPP Provide Assistance Data message to the UE.