3GPP TSG-RAN WG2 Meeting #119-e	R2-2208500

# Electronic meeting, 17 – 29 August 2022

Agenda item:		6.1.3

Source:	Intel Corporation

Title:	Remaining MBS UE capability open issues

Document for:		Discussion and Decision

# Introduction

In RAN1 LS on UE feature list R1-2205609 [1], RAN1 asks RAN2 to decide the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. In addition, RAN1 leaves the implementation of FG 33-1-1 (DCI indicated slot-level repetition for broadcast) to RAN2.

In this contribution, we discuss remaining MBS UE capability open issues.

# Discussion

## Candidate values for the number of G-CS-RNTIs for SPS multicast in FG 33-5-1h

In RAN1 LS on UE feature list R1-2205609 [1], it was indicated that:

For NR_MBS, RAN1 reached the following agreement:

Agreement - There is no consensus in RAN1 for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. Ask RAN2 to decide the candidate values - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. - Note: from RAN1 perspective, the larger number of G-CS-RNTIs UE supports, the more impact to whether UE can report the capability of supporting other UE features under a given UE implement with a certain number of RNTI budget.

RAN2 has already decided that the configuration limit of G-CS-RNTI per UE is 8, as from following ASN.1 code:

maxG-CS-RNTI-r17     INTEGER ::= 8    -- Maximum number of G-CS-RNTI that can be configured for a UE.

Therefore, it is straightforward to confirm the candidate values for the number of G-CS-RNTIs are {2, 3, …, 8}.

Proposal 1: RAN2 to confirm that the candidate values for the number of G-CS-RNTIs for SPS multicast in FG 33-5-1h are {2, 3, …, 8}, and to clarify that RRC specification already captures the maximum number of G-CS-RNTIs allowed by ASN.1 as 8 (as defined by maxG-CS-RNTI-r17).

## RAN1 FG 33-1-1 DCI indicated slot-level repetition for broadcast

In the latest RAN1 UE feature list R1-2205607 [2], RAN1 indicates that how to implement FG 33-1-1 is up to RAN2.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33-1-1 | DCI indicated slot-level repetition up to 16 for broadcast MTCH | Support up to 16 times dynamic slot-level repetition for broadcast MTCH. | 33-1 | Up to RAN2 | Up to RAN2 | Up to RAN2 | Up to RAN2 | Up to RAN2 |

The prerequisite of FG 33-1-1 is FG 33-1, which is implemented in TS 38.306 clause 5.10 as optional feature without UE capability signalling, as copied below:

Definitions for feature

Broadcast reception It is optional for UE to support broadcast reception as specified in TS 38.331 [9]. A UE that supports the feature shall also support: - 4 broadcast MRBs as the minimum number; - PDCP 12 bits SN; - ROHC with profiles 0x0000, 0x0001 and 0x0002; - 8 ROHC context sessions; - RLC UM with 6 bits SN; - RLC UM with 12 bits SN; - DRX with long DRX cycle.

Since FG 33-1-1 is related to broadcast reception in IDLE/INACTIVE, it should be optional feature without UE capability signalling, just as FG 33-1. It should be noted that any optional broadcast feature (including FG 33-1-1) might not be used by gNB if it is not in the supported feature list of broadcast reception (FG 33-1). The reason is that gNB is not aware of whether UEs receiving a particular broadcast MRB support certain feature or not no matter whether the feature is explicitly signalled or not since gNB is not aware which UEs in IDLE/INACTIVE are receiving the broadcast MRB.

Proposal 2: FG 33-1-1 (DCI indicated slot-level repetition for broadcast) is implemented in TS 38.306 clause 5.10 as optional feature without UE capability signalling.

TP to implement FG 33-1-1 (as in Proposal 2) is provided in Annex.

## Upper limit for maxNumberRNTIs-MBS-r17

Currently it is still FFS regarding upper limit for maxNumberRNTIs-MBS-r17. Related UE capability definition and ASN.1 signalling is as below:

| maxNumberRNTIs-MBS-r17 Indicates the maximum number of simultaneous reception of PDCCH scrambled with G-RNTIs/G-CS-RNTIs for MBS multicast. | UE | No | No | No |
| --- | --- | --- | --- | --- |

MAC-ParametersCommon ::=    SEQUENCE {

...

maxNumberRNTIs-MBS-r17                   INTEGER (1..ffsUpperLimit)  OPTIONAL,       -- FFS

...

}

Configuration limit of G-RNTI and G-CS-RNTI per UE is 16 and 8 respectively, as from following ASN.1 code:

maxG-RNTI-r17      INTEGER ::= 16  -- Maximum number of G-RNTI that can be configured for a UE.

maxG-CS-RNTI-r17   INTEGER ::= 8   -- Maximum number of G-CS-RNTI that can be configured for a UE.

Therefore it is proposed that upper limit for maxNumberRNTIs-MBS-r17 is set to 24.

Proposal 3: Upper limit for maxNumberRNTIs-MBS-r17 (i.e. value captured as ffsUpperLimit in current ASN.1) is set to 24.

# Conclusion

In this contribution, we discuss remaining MBS UE capability open issues, and propose the following:

Proposal 1: RAN2 to confirm that the candidate values for the number of G-CS-RNTIs for SPS multicast in FG 33-5-1h are {2, 3, …, 8}, and to clarify that RRC specification already captures the maximum number of G-CS-RNTIs allowed by ASN.1 as 8 (as defined by maxG-CS-RNTI-r17).

Proposal 2: FG 33-1-1 (DCI indicated slot-level repetition for broadcast) is implemented in TS 38.306 clause 5.10 as optional feature without UE capability signalling.

Proposal 3: Upper limit for maxNumberRNTIs-MBS-r17 (i.e. value captured as ffsUpperLimit in current ASN.1) is set to 24.

Proposal 1 is already implemented in the mega CR for RAN1/RAN4 UE feature list in R2-2207277.

TP to implement FG 33-1-1 (as in Proposal 2) is provided in Annex.

# References

[1] R1-2205609, RAN1 LS, "LS on updated Rel-17 RAN1 UE features list for NR"

[2] R1-2205607, AT&T, NTT DOCOMO, "Updated RAN1 UE features list for Rel-17 NR after RAN1 #109-e Week2"

# Annex: TP for TS 38.306 to capture FG 33-1-1

## 5.10	MBS features

Definitions for feature

Broadcast reception It is optional for UE to support broadcast reception as specified in TS 38.331 [9]. A UE that supports the feature shall also support: - 4 broadcast MRBs as the minimum number; - PDCP 12 bits SN; - ROHC with profiles 0x0000, 0x0001 and 0x0002; - 8 ROHC context sessions; - RLC UM with 6 bits SN; - RLC UM with 12 bits SN; - DRX with long DRX cycle.

DCI indicated slot-level repetition for broadcast MTCH It is optional for UE to support up to 16 times dynamic slot-level repetition for broadcast MTCH as specified in TS 38.214 [12]. A UE that supports this feature shall also support broadcast reception.