3GPP TSG-RAN WG2 Meeting #119-e	R2-2208558

# Online, 17th Aug - 29th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3442 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.331 for TCI-UL-state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation,Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-8-9 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1: The decription of TCI-UL-state is ‘copy and paste’ from TCI-state which is not correct. 2: Regarding the field description of the ‘AdditionalPCI’ , the additionalPCI is an index which is associated with a serving cell, it shall be clarified in description what serving cell is associated with (e.g. the serving cell where the tci-ul-state is configured or the serving cell where the RS in TCI-UL-state is configured). 3: Regarding the field description of the servingCellId , it is just ‘copy and paste’ from the field description of the cell in TCI-State , it shall be modified for adapting to the TCI-UL-state. 4: Regarding the field description of the ul-PowerControl, there is a restriction for the presence of this IE in TCI-UL-state that such element shall be absent in any BWP-Uplink-Dedicated of the serving cell. Since the TCI-UL-State in one serving cell can be referenced to another serving cell, ‘this serving cell’ in the description is ambiguous for understanding what serving cell is referred to, the serving cell where this UL-TCI-state is configured or the serving cell where this TCI-UL-State is applied? In our understanding, it will confuse UE if the ul-PowerControl with different values is included in both BWP-Uplink-Dedicated and the referenced TCI-UL-State, so we think ‘this serving cell’ refer to ‘the serving cell where the UL-TCI-state is applied’. It shall be clarified in the field description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: Correct the description of TCI-UL-State as ‘The IE TCI-UL-State indicates the TCI state information for UL transmission’ 2: Correct the field description of addtionalPCI in TCI-UL-state as’ The SSB identified by the present ssb-Index-r17 refers to a cell identified by this field, this field is associated with the serving cell identified by seringCellId-r17.’ 3:Correct the field description of servingCellId as ‘The UE's serving cell in which the referenceSignal is configured. If the field is absent, it applies to the serving cell in which the TCI-UL-State is configured.’ 4: Correct the field description of the ‘ul-powerControl’ as ‘Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of the serving cell where the TCI-UL-State is applied.’ Impact analysis Impacted 5G architecture options: SA，NR-DC, NE-DC, (NG)EN-DC Impacted functionality: Untified TCI state Inter-operability: If NW implements according to this CR but UE not, for the second issue,UE and NW may have different understandings on what serving cell the AdditonalPCIIndex is associated with. For the fourth issue, no inter-operability issue can be found. If UE implements according to this CR but NW not, for the second issue,UE and NW may have different understandings on what serving cell the AdditonalPCIIndex is associated with. For the fourth issue, UE cannot understand what ul-PowerControl is available, the one configured in BWP-Uplink-Dedicated or TCI-UL-State. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and NW may have different understanding on what serving cell the AdditonalPCIIndex is associated with. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the Change

#### –	TCI-UL-State

The IE TCI-UL-State indicates the TCI state information for UL transmission...

TCI-UL-State information element

-- ASN1START

-- TAG-TCI-UL-STATE-START

TCI-UL-State-r17 ::=             SEQUENCE {

tci-UL-State-Id-r17              TCI-UL-State-Id-r17,

servingCellId-r17                ServCellIndex                                         OPTIONAL,   -- Need R

bwp-Id-r17                       BWP-Id                                                OPTIONAL,   -- Cond CSI-RSorSRS-Indicated

referenceSignal-r17              CHOICE {

ssb-Index-r17                    SSB-Index,

csi-RS-Index-r17                 NZP-CSI-RS-ResourceId,

srs-r17                          SRS-ResourceId

},

additionalPCI-r17                AdditionalPCIIndex-r17                                OPTIONAL,   -- Need R

ul-powerControl-r17              Uplink-powerControlId-r17                             OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17       PUSCH-PathlossReferenceRS-Id-r17                      OPTIONAL,   -- Need R

...

}

-- TAG-TCI-UL-STATE-STOP

-- ASN1STOP

TCI-UL-State field descriptions

additionalPCI The SSB identified by the present ssb-Index-r17 refers to a cell identified by this field, this field is associated with the serving cell identified by seringCellId-r17..

bwp-Id The DL BWP which the CSI-RS is located in or UL BWP where the SRS is located in.

servingCellId The UE's serving cell in which the referenceSignal-r17 is configured. If the field is absent, it applies to the serving cell in which the TCI-UL-State is configured.

pathlossReferenceRS-Id The ID of the reference Signal (e.g. a CSI-RS config or a SS block) used for PUSCH path loss estimation.

ul-powerControl Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of a serving cell where the TCI-UL-State is applied.

| Conditional Presence | Explanation |
| --- | --- |
| CSI-RSorSRS-Indicated | This field is mandatory present if csi-rs or srs is included, absent otherwise |

End of the Change