3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2208564

Elbonia, 17 – 26 August 2022

Agenda item:	7.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Issue on GNSS measurement during eMTC NTN handover

WID/SID:	LTE_NBIOT_eMTC_NTN - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In the RAN2-118 meeting the transmission of the GNSS validity duration from the UE to the network was discussed. The following was agreed with regards to when the UE transmits the GNSS validity duration.

P3: The new parameter for remaining GNSS validity duration is introduced in the following Msg5 messages: RRCConnectionResumeComplete, RRCConnectionSetupComplete, RRCreestablishmentComplete RRCConnectionResumeComplete-NB, RRCConnectionSetupComplete-NB, RRCreestablishmentComplete-NB. P4: The new parameter for remaining GNSS validity duration is introduced in RRCConnectionReconfigurationComplete for MTC Handover.

In this contribution we discuss issues related to providing the remaining GNSS validity duration as part of the handover procedure for eMTC.

# 2	Discussion

It is well known (see TR 36.763) that the time required to obtain a new GNSS fix depends on the GNSS state (e.g. hot or warm), the UE’s GNSS capability (e.g. number of antennas and processing speed) and the current GNSS channel state (e.g. high or low SNR).

Observation 1: The time to obtain a new GNSS fix depends on the GNSS state, UE capabilities, and GNSS channel state.

Therefore, the time required by a single UE to obtain a new GNSS fix can vary.

According to the agreement in the RAN2-118 meeting the UE must provide the remaining GNSS validity duration in the RRCConnectionReconfigurationComplete message of the eMTC handover. In this scenario, the network cannot be sure whether the UE will report a GNSS validity duration, based on an old GNSS fix, or attempt to obtain a new GNSS fix during the handover procedure, and in case the UE decides to do so, how long time the UE will spend obtaining the GNSS fix.

Observation 2: The network is not aware whether the UE will acquire a new GNSS fix during an eMTC handover, and how long time the UE may spend to obtain the new GNSS fix.

In one scenario, the UE may determine that the remaining GNSS validity duration (from the previous GNSS fix) is sufficiently long and just report that to the network as part of the RRCConnectionReconfigurationComplete while in another scenario, the UE may determine that the remaining GNSS validity is too short. In the latter case, the UE has to acquire the new GNSS fix before providing the updated GNSS validity duration in RRCConnectionReconfigurationComplete since simultaneous GNSS and NTN eMTC operation is not assumed.

The latter case is problematic for the network, because

- The network may not be able to configure the timer T304 appropriately. The timer is initiated when the UE receives the “RRCConnectionReconfiguration” message and stopped when the handover is completed. If the timer is shorter than the time required by the UE to obtain the new GNSS fix, the UE may experience a handover failure due to timer T304 expiry.

- The network may not be able to configure the RACH resources appropriately. The network may configure the resources at a time instance, where the UE has not acquired the new GNSS fix and thus the resources are wasted.

Observation 3: Without knowledge about the UE’s need for potential GNSS fix acquisition during an eMTC handover the network may not be able to configure the timer T304 and RACH resources appropriately.

Based on the above analysis, it is proposed that RAN2 discuss how to ensure the network can configure the timer T304 and RACH resources, to handle the scenario where the eMTC UE may need to acquire a new GNSS fix during a handover.

Proposal 1: RAN2 to discuss how the network can handle the eMTC UE potentially performing GNSS fix during a handover

It is evident that the network will benefit from knowledge about the UE’s intentions, i.e. is the UE expecting to perform a GNSS fix or not during a potential upcoming handover?

Observation 4: UE reporting the need for a GNSS fix and the corresponding time, required to obtain the GNSS fix, will be beneficial for the network.

# 3	Conclusion

This document has made the following observations:

Observation 1: The time to obtain a new GNSS fix depends on the GNSS state, UE capabilities, and GNSS channel state.

Observation 2: The network is not aware whether the UE will acquire a new GNSS fix during an eMTC handover, and how long time the UE may spend to obtain the new GNSS fix.

Observation 3: Without knowledge about the UE’s need for potential GNSS fix acquisition during an eMTC handover the network may not be able to configure the timer T304 and RACH resources appropriately.

Observation 4: UE reporting the need for a GNSS fix and the corresponding time, required to obtain the GNSS fix, will be beneficial for the network.

And proposed the following:

Proposal 1: RAN2 to discuss how the network can handle the eMTC UE potentially performing GNSS fix during a handover