3GPP TSG-RAN WG2 Meeting #119-e                             	R2-2208590

Online, August 17 – August 29, 2022

Agenda item:	6.1.5

Source:	Samsung

Title:	Correction for Initial Value of RX_DELIV for Multicast

Document for:	Discussion & Decision

# Introduction

This contribution proposes a correction for initial value of RX_DELIV for multicast.

# Discussion

In PDCP-Config of TS 38.331 [1], parameter name multicastHFN-AndRefSN and its field description say that it is a reference PDCP SN and its HFN. However, this parameter is used only for the initial value of RX_DELIV of PDCP operation. The current field description is not actually useful but confusing. The reason for this situation is that RAN2 initially separately agreed the initial value of HFN and reference SN in RAN2#116bis-e, and later agreed the combination of those two values is used as the initial value of RX_DELIV in TS 38.323 [2]. The relevant agreements are as follows:

< RAN2#116bis-e >

[027] For multicast, the initial value of HFN is indicated by the gNB via RRC.

[027] If the initial value of HFN is indicated by the gNB, a reference SN corresponding to the initial value of HFN can be indicated to the UE.

< RAN2#118-e >

Go for Option 2

- Option 2: Initial RX_DELIV is configured by RRC: SN(RX_DELIV) = SN_ref and HFN(RX_DELIV) = HFN_initial where HFN_initial and SN_ref are provided by RRC for multicast. (13/16)

Currently, separate handling of the initial HFN value does exist. This value is always used together with the RefSN. Thus, separate field description is not needed, and especially size relationship between SN and HFN is not needed at all but makes the spec complicated. We see a simple clarification is necessary. A possible changes would be as follows:

initialRXDELIV Indicates an initial value of RX_DELIV for multicast MRB PDCP window initialization as specified in TS 38.323 [5].

Proposal 1. multicastHFN-AndRefSN is renamed to initialRXDELIV.

Proposal 2. Adopt the corresponding TP in Section 5.

# Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. multicastHFN-AndRefSN is renamed to initialRXDELIV.

Proposal 2. Adopt the corresponding TP in Section 5.

# References

[1] TS 38.331 NR RRC specification, v17.1.0

[2] TS 38.323 NR PDCP specification, v17.1.0

# Text Proposals

38.331

PDCP-Config ::= SEQUENCE { drb SEQUENCE { discardTimer ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity} OPTIONAL, -- Cond Setup pdcp-SN-SizeUL ENUMERATED {len12bits, len18bits} OPTIONAL, -- Cond Setup1 pdcp-SN-SizeDL ENUMERATED {len12bits, len18bits} OPTIONAL, -- Cond Setup2 headerCompression CHOICE { notUsed NULL, rohc SEQUENCE { maxCID INTEGER (1..16383) DEFAULT 15, profiles SEQUENCE { profile0x0001 BOOLEAN, profile0x0002 BOOLEAN, profile0x0003 BOOLEAN, profile0x0004 BOOLEAN, profile0x0006 BOOLEAN, profile0x0101 BOOLEAN, profile0x0102 BOOLEAN, profile0x0103 BOOLEAN, profile0x0104 BOOLEAN }, drb-ContinueROHC ENUMERATED { true } OPTIONAL -- Need N }, uplinkOnlyROHC SEQUENCE { maxCID INTEGER (1..16383) DEFAULT 15, profiles SEQUENCE { profile0x0006 BOOLEAN }, drb-ContinueROHC ENUMERATED { true } OPTIONAL -- Need N }, ... }, integrityProtection ENUMERATED { enabled } OPTIONAL, -- Cond ConnectedTo5GC1 statusReportRequired ENUMERATED { true } OPTIONAL, -- Cond Rlc-AM-UM outOfOrderDelivery ENUMERATED { true } OPTIONAL -- Need R } OPTIONAL, -- Cond DRB moreThanOneRLC SEQUENCE { primaryPath SEQUENCE { cellGroup CellGroupId OPTIONAL, -- Need R logicalChannel LogicalChannelIdentity OPTIONAL -- Need R }, ul-DataSplitThreshold UL-DataSplitThreshold OPTIONAL, -- Cond SplitBearer pdcp-Duplication BOOLEAN OPTIONAL -- Need R } OPTIONAL, -- Cond MoreThanOneRLC t-Reordering ENUMERATED { ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220, ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750, ms3000, spare28, spare27, spare26, spare25, spare24, spare23, spare22, spare21, spare20, spare19, spare18, spare17, spare16, spare15, spare14, spare13, spare12, spare11, spare10, spare09, spare08, spare07, spare06, spare05, spare04, spare03, spare02, spare01 } OPTIONAL, -- Need S ..., [[ cipheringDisabled ENUMERATED {true} OPTIONAL -- Cond ConnectedTo5GC ]], [[ discardTimerExt-r16 SetupRelease { DiscardTimerExt-r16 } OPTIONAL, -- Cond DRB2 moreThanTwoRLC-DRB-r16 SEQUENCE { splitSecondaryPath-r16 LogicalChannelIdentity OPTIONAL, -- Cond SplitBearer2 duplicationState-r16 SEQUENCE (SIZE (3)) OF BOOLEAN OPTIONAL -- Need S } OPTIONAL, -- Cond MoreThanTwoRLC-DRB ethernetHeaderCompression-r16 SetupRelease { EthernetHeaderCompression-r16 } OPTIONAL -- Need M ]], [[ survivalTimeStateSupport-r17 ENUMERATED {true} OPTIONAL, -- Cond Drb-Duplication uplinkDataCompression-r17 SetupRelease { UplinkDataCompression-r17 } OPTIONAL, -- Cond Rlc-AM discardTimerExt2-r17 SetupRelease { DiscardTimerExt2-r17 } OPTIONAL, -- Need M initialRXDELIV-r17 BIT STRING (SIZE (32)) OPTIONAL -- Cond SetupOnlyMRB ]] } …

initialRXDELIV Indicates an initial value of RX_DELIV for multicast MRB PDCP window initialization as specified in TS 38.323 [5].

38.323

b) RX_DELIV This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for broadcast and groupcast based sidelink discovery, the initial value of the SN part of RX_DELIV is (x – 0.5 × 2[sl-PDCP-SN-Size–1]) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For broadcast MRBs, the initial value of the SN part of RX_DELIV is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs, the initial value of RX_DELIV is set by initialRXDELIV in TS 38.331 [3]. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB. NOTE 3: For broadcast MRBs, the initial value of the HFN part of RX_DELIV is set by UE implementation.