3GPP TSG RAN WG2 #119-e	R2-2208616

e-Meeting, August 15th – 26th, 2022

Agenda Item:	8.14.3

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Rel-17 leftover issues for QoE

Document for:	Discussion and Decision

Introduction

The new WID of enhancement on NR QoE management and optimizations for diverse services was approved in RAN#96[1]. In which, the following objective is included:

... - Left-over features from Rel-17, as well as the enhancements of existing features which are not included in Rel-17 normative phase, should be supported in Rel-18 if consensus on benefits are reached [RAN3, RAN2]. - Specify per-slice QoE measurement configuration enhancement. - Specify RAN visible QoE enhancements for QoE value, RAN visible QoE trigger event, RAN visible QoE Report over F1. - Specify QoE reporting handling enhancement for overload scenario.

In this contribution, we will discuss the remaining issues of per-slice QoE measurement configuration,  RAN visible QoE value and RAN visible QoE trigger event. Then we’ll give our proposals.

Discussion

Per-slice QoE measurement configuration enhancement

In Rel-17, RAN3 has agreed to support the QoE measurement per slice in management-based QoE and signaling-based QoE. For signaling-based QoE, RAN may further decide to only configure a specific UE for QoE measurements within selected slices, which are subset of slice list from OAM/CN. For management-based QoE, RAN will check the slice scope included in the QoE configuration from OAM/CN and based on this RAN will identify the UEs to configure with QoE. Wherein slice scope is a list of S-NSSAI. However, in a scenario where a service corresponds to multiple slice, if the RAN side simply indicates the need to perform QoE measurement through the service type, the UE can not avoid activating unnecessary slice to perform QoE measurement. Therefore, the slice list information should be provided to the UE. Further, the above problem can be solved in the following methods:

- Option 1: Slice information is included in the measurement configuration container

- Option 2: Slice information is included in the RRC signalling

In our opinion, both options can solve the above problems, but slice information cannot be included in both container and RRC signalling. e.g., in the case of Option 2, if the slice information is also included in the container, the configuration could be inconsistent with RAN configuration leading to improper UE behaviour.

Observation 1: If both Option 1 and 2 are supported, it may result in inconsistent configuration information and incorrect UE behavior.

Further, in Option 2, UE AS layer needs to forward slice information to UE APP layer. Therefore, the slice information in the measurement configuration container has minimal impact on existing the standard. We think it necessary to send LS to SA4 and suggest that SA4 include slice information in the measurement configuration container.

Proposal 1: The QoE measurement configuration container includes the slice information, and the gNB should not send the slice information by the RRC signalling.

RAN visible QoE enhancements

In Rel-17, in order to simplify the implementation of RAN Visible QoE (RVQoE), RAN3 has discussed an application method of RVQoE value. The RVQoE value is s value within an interval to indicate the degree of the RVQoE. After receiving this value, gNB evaluates whether it needs to be optimized accordingly. In our opinion, the main influence of this parameter on RAN2 is to transfer the relevant parameters. However, since it is up to RAN3 to decide whether or not to introduce this parameter, we suggest to postpone the discussion and wait for the conclusion of RAN3.

Observation 2: It is necessary to wait for the conclusion of whether or not to introduce RVQoE value into RAN3.

Conclusion

In previous sections, the following observations and proposals were made:

Observation 1: If both Option 1 and 2 are supported, it may result in inconsistent configuration information and incorrect UE behavior.

Observation 2: It is necessary to wait for the conclusion of whether or not to introduce RVQoE value into RAN3.

Proposal 1: The QoE measurement configuration container includes the slice information, and the gNB should not send the slice information by the RRC signalling.

# References

- RP-221803, Enhancement on NR QoE management and optimizations for diverse services, June, 2022