3GPP TSG-RAN WG2 Meeting #119-e	R2-2208657

Elbonia, 17th – 26th August, 2022

Agenda item:	6.10.3.2.3

Source: 	Sequans Communications

Title: 	Issues related to NR NTN epoch time

Document for:	Discussion and Decision

# Introduction

This contribution discusses two issues related to epoch time implementation.

# Discussion

From 38.331v17.1.0, epoch time is defined as follows:

NTN-Config field descriptions

epochTime Indicate the epoch time for assistance information (i.e. Serving satellite ephemeris in IE ephemerisInfo and Common TA parameters). When explicitly provided through SIB, or through dedicated signaling, EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. If this field is absent, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when provided in dedicated configuration. If this field is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses epoch time from the serving satellite ephemeris. In case of handover, this field is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the target cell. This field is excluded when determining changes in system information, i.e. changes of epochTime should neither result in system information change notifications nor in a modification of valueTag in SIB1.

## EpochTime SFN ambiguity

When explicitly signaled (yellow part of the description), the epochTime is ambiguous due to SFN wraparound every 10.24s. This issue has been discussed by RAN1 for several meetings, but without conclusion for now, even though it is considered a high priority issue.

We see the following main options to determine wich Epoch SFN/SF to consider depending of the SFN/SF at which the message is received:

- Consider nearest SFN(/subframe) occurrence in the future

- Consider nearest SFN(/subframe) occurrence in the past

- Consider nearest SFN(/subframe) (and one of future/past in case of equality)

There is some concern in RAN1 that

- having Epoch always in the past would waste useful UL synchronization time

- having Epoch in the future would require backward propagation of the orbit and common TA

The implicit Epoch time is already in the future (end of the SI window). The si-WindowLength ranges between 5ms and 2.56s. If backward propagation is not supported, this means a UE must wait some time before being able to access the cell. That time can be reduced by using short si-WindowLength, but this impacts the whole SIB scheduling (as it is common for all SI messages).

Hence it seems preferable to support Epoch in the future along with backward propagation. Epoch in the future without backward propagation would prevent UEs to timely access the NW and might be acceptable only for Epoch in a very near future.

Observation 1: It is beneficial to support Epoch in the future along with backward propagation

Observation 2: Epoch in the future without backward propagation introduces latency

Ultimately, the decision about backward propagation should be left to RAN1.

However, from RAN2 point of view, it may be good to keep the flexibility to support both Epoch in the future and in the past (e.g., Option 3). Indeed, the ntn-Config with explicit epochTime might be sent either via broadcast or dedicated message (within RRC Reconfiguration). In case of dedicated message, the actual reception of the message can be delayed by retransmissions.

If it is finally decided that backward propagation is a different feature that may not be supported by UEs, then a conservative NW approach would be to use the current SFN or a SFN in a very near future, to limit the transmission latency.

We also assume that a typical NW implementation, in case there is a need to send SIB19 via dedicatedSystemInformationDelivery (no CSS in active BWP case), may want to send the SIB19 as it is broadcasted, just adding the SFN/SF if it was not present (that would correspond to the end of SI-window of the broadcast SIB19). This de-facto corresponds to using Epoch in a very near future.

This comes with the drawback that any retransmission delay may turn such future Epoch into a past Epoch at the time of UE reception of the message.

Observation 3: Retransmission delays may turn a "Future Epoch" into a "Past Epoch"

Given above mentioned use cases, we think it might be better from RAN2 point of view to support both Future and Past Epochs, e.g. Option 3.

The Option 3 would be robust to retransmissions delays, while still enabling up to 5.12s future Epochs. It is not clear why up to 10.24s future Epochs would be required instead. If that was really a must, then we assume even larger future Epochs would be preferable and the best solution would be to add some HFN signaling to allow it.

Moreover, there was a similar issue for delivering ReferenceTimeInfo, and the solution implemented in specification corresponds to choosing the nearest frame.

Hence, it seems to us the best approach at this point would be to go for Option 3.

Proposal 1: When explicitly signaled, the EpochTime SFN occurrence is the nearest from the SFN at which the message is received

There is still an ambiguity in case of past/future equality (EpochTime SFN and message SFN spaced out by 512). It can be decided arbitrarily to choose either past or future Epoch to clear up that case. It could also be decided to not handle that case (it was not handled for ReferenceTimeInfo delivery) and let the NW implementation avoid it.

We propose a 38.331 TP in Annex based on ReferenceTimeInfo delivery wording, with additional handling of past/future equality case.

## Validity timer started at SF indicated by epochTime at UE/RP

The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point.

In our understanding, this sentence means that the timing for epoch time is defined at the "uplink time synchronization reference point". That is to say, the epoch time is "the starting time of a DL sub-frame […]" when that DL sub-frame is transmitted from the "uplink time synchronization reference point". At the UE, the corresponding DL sub-frame is received with a delay, equal to 1/2 Common TA(at tRP=epoch time) + service link delay (at tSAT= 1/2  Common TA).

This is essential for the calculations of timing (common TA, service link delay). In our view, the validity duration should (in theory) also be impacted, i.e. "start or restart T430 with the duration ntn-UlSyncValidityDuration from the subframe indicated by epochTime;" should consider the delay already elapsed upon receiving that subframe at the UE from the RP.

However, given the coarse granularity of ntn-UlSyncValidityDuration, and the fact that the transmission delay from RP to the UE would be much smaller, we assume it is also acceptable to consider starting the timer at the subframe indicated by epochTime at the UE (not at the RP). In our understanding, this is already implied by the specification, but it might be good to have a common understanding on this.

Proposal 2: Confirm that the validity timer is started from the subframe indicated by epochTime at the UE (not at the RP) (no change in specification)

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: It is beneficial to support Epoch in the future along with backward propagation

Observation 2: Epoch in the future without backward propagation introduces latency

Observation 3: Retransmission delays may turn a "Future Epoch" into a "Past Epoch"

Proposal 1: When explicitly signaled, the EpochTime SFN occurrence is the nearest from the SFN at which the message is received

Proposal 2: Confirm that the validity timer is started from the subframe indicated by epochTime at the UE (not at the RP) (no change in specification)

# Annex – 38.331 TP

NTN-Config field descriptions

epochTime Indicate the epoch time for assistance information (i.e. Serving satellite ephemeris in IE ephemerisInfo and Common TA parameters). When explicitly provided through SIB, or through dedicated signaling, EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. The UE considers the EpochTime frame (indicated by SFN) to be the frame which is nearest to the frame where the message is received (which can be either in the past or in the future), or the future frame in case EpochTime SFN and message SFN spaced out by 512. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. If this field is absent, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when provided in dedicated configuration. If this field is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses epoch time from the serving satellite ephemeris. In case of handover, this field is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the target cell. This field is excluded when determining changes in system information, i.e. changes of epochTime should neither result in system information change notifications nor in a modification of valueTag in SIB1.