3GPP TSG-RAN2#119-e	R2-2208745

# e-meeting, 17-29 August

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1856 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Capability CR for HO from E-UTRA to FR2-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 26/08/2022 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier elease) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM measurements for FR2-2 cell have been introduced in 38.331. The common understanding is that the UE supporting the HO to FR2-2 should also support the FR2-2 measurement framework. This is currently not clear in 36.306. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in the field descriptions of eutra-5GC-HO-ToNR-TDD-FR2-2-r17 and eutra-EPC-HO-ToNR-TDD-FR2-2-r17 that the UE supporting these features shall also support the corresponding RRM measurements. Impact analysis Impacted 5G architecture options: SA, ENDC Impacted functionality: FR2-2 Inter-operability: - If the UE is implemented according to this CR while the network is not, the network doesn’t know whether the UE supports the FR2-2 measurements or not. However in this case there is no interoperability issue since the UE can be configured with measurements if the network chooses to do this. - If the network is implemented according to this CR while the UE is not, the network may configure measurements for FR2-2 which may result in invalid (unsupported) configuration on the UE side if the UE doesn’t support this configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear whether the UE supporting HO to FR2-2 also supports the measurements or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.34.25, 4.3.34.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.331 CR 4869 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

#### 4.3.34.25	eutra-5GC-HO-ToNR-TDD-FR2-2-r17

This field indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR2-2 as specified in TS 38.101-x[xx]. A UE that indicates this field also supports eutra-5GC-r15. A UE supporting handover from E-UTRA/5GC to NR TDD FR2-2 shall also support the RRM measurements for FR2-2 as specified in TS 36.331 [5].

#### Next Change4.3.34.26	eutra-EPC-HO-ToNR-TDD-FR2-2-r17

This field indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR2-2 as specified in TS 38.101-x[xx]. A UE supporting handover from E-UTRA/EPC to NR TDD FR2-2 shall also support the RRM measurements for FR2-2 as specified in TS 36.331 [5].

End of Changes