3GPP TSG-RAN WG2 Meeting #119 electronic                 	R2-2208905

Online, August, 2022

Source:	OPPO

Title:	Cell Identity Issue for NPN during RRC Resume Procedure

Agenda Item:	5.1.3.1.1

Document for:	Discussion and Decision

- Introduction

In RAN#94_e meeting, RAN2 has received a LS from RAN3 on cell identity issue for NPN during RRC resume procedure [1], in this contribution, we will analyze this issue and give our suggestion.

- Discussion

RAN3 raised an issue for NPN-only cell during RRC resume procedure [1]:

In case the UE resumes in an NPN-only cell in an inter-gNB mobility scenario RAN3 has discovered the following:

The UE, while calculating the VarResumeMAC-Input would use the targetCellIdentity pointing to the cellIdentity of the first PLMN-Identity included in the PLMN-IdentityInfoList. Meanwhile, the old/anchor gNB would use the PCI and ARFCN information of the targetCellIdentity to derive the Key NG-RAN Star in the RETRIEVE UE CONTEXT RESPONSE message.

The targetCellIdentity is associated to the first entry in the PLMN-IdentityInfoList, which the old/anchor gNB is not aware of, as it doesn’t correspond to an actual cell in NPN-only case and therefore this information is not exchanged over Xn.

RAN3 has figured out that configuring the cellIdentity of the first entry of the plmn-IdentityInfoList with the same value as the cellIdentity-r16 of the first entry in the npn-IdentityInfoList-r16 for NPN only cell would enable resumption into NPN-only cells in inter-gNB mobility cases.

RAN3 would like to ask RAN2 to consider updating TS 38.331 for including such “configuration rule” in TS 38.331.

Before agreeing the suggestion from RAN3 LS, we’d like to review what current spec looks like:

From RAN2 TS 38.331 spec, IE VarResumeMAC-Input is defined like the following:

#### –	VarResumeMAC-Input

The UE variable VarResumeMAC-Input specifies the input used to generate the resumeMAC-I during RRC Connection Resume procedure.

VarResumeMAC-Input variable

-- ASN1START

-- TAG-VARRESUMEMAC-INPUT-START

VarResumeMAC-Input  ::=     SEQUENCE {

sourcePhysCellId                        PhysCellId,

targetCellIdentity                      CellIdentity,

source-c-RNTI                           RNTI-Value

}

-- TAG-VARRESUMEMAC-INPUT-STOP

-- ASN1STOP

VarResumeMAC-Input field descriptions

targetCellIdentity An input variable used to calculate the resumeMAC-I. Set to the cellIdentity of the first PLMN-Identity included in the PLMN-IdentityInfoList broadcasted in SIB1 of the target cell i.e. the cell the UE is trying to resume.

source-c-RNTI Set to C-RNTI that the UE had in the PCell it was connected to prior to suspension of the RRC connection.

sourcePhysCellId Set to the physical cell identity of the PCell the UE was connected to prior to suspension of the RRC connection.

It’s quite clear that UE will always use the cellIdentity of the first PLMN-Identity included in the PLMN-IdentityInfoList broadcasted in SIB1 of the target, i.e. the cell the UE is trying to resume, no matter the cell is an NPN-only cell or not.

Observation1: From RAN2 spec perspective, UE shall always use the cellIdentity of the first PLMN-Identity included in the PLMN-IdentityInfoList broadcasted in SIB1 of the target, i.e. the cell the UE is trying to resume, when calculating the VarResumeMAC-Input no matter the cell is an NPN-only cell or not.

From RAN3 spec perspective, the relevant part is described below [2]:

#### 9.1.1.8	RETRIEVE UE CONTEXT REQUEST

This message is sent by the new NG-RAN node to request the old NG-RAN node to transfer the UE Context to the new NG-RAN.

Direction: new NG-RAN node  old NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | reject |
| UE Context ID | M |  | 9.2.3.40 |  | YES | reject |
| Integrity protection | M |  | BIT STRING (SIZE (16)) | RRC Resume: ResumeMAC-I either contained in the RRC ResumeRequest or the RRCResumeRequest1 message as defined in TS 38.331 [10]) or the ShortResumeMAC-I in the RRCConnection ResumeRequest message as defined in TS 36.331 [14]) RRC Reestablishment: ShortMAC-I contained in the RRCReestablishmentRequest as defined in TS 38.331 [10]) or the ShortMAC-I in the RRCConnection ReestablishmentRequest message as defined in TS 36.331 [14]). RRC Resume for UP CIoT Optimization: ShortResumeMAC-I in the RRCConnection ResumeRequest message or RRCConnection ResumeRequest-NB message as defined in TS 36.331 [14]. | YES | reject |
| New Cell Identifier | M |  | NG-RAN Cell Identity 9.2.2.9 | RRC Resume: Corresponds to the targetCellIdentity within the VarResumeMAC-Input as specified in TS 38.331 [10] or the cellIdentity within the VarShortINACTIVE-MAC-Input as specified in TS 36.331 [14]. RRC Reestablishment: Corresponds to the targetCellIdentity within the VarShortMAC-Input as specified in TS 38.331 [10] or the cellIdentity within the VarShortMAC-Input as specified in TS 36.331 [14]. RRC Resume for UP CIoT Optimization: Corresponds to the cellIdentity within the VarShortResumeMAC-Input or VarShortResumeMAC-Input-NB as specified in TS 36.331 [14]. | YES | reject |
| RRC Resume Cause | O |  | 9.2.3.61 | In case of RNA Update, contains the cause value provided by the UE in the RRCResumeRequest or the RRCResumeRequest1 message, as defined in TS 38.331 [10], or in the RRCConnection ResumeRequest message, as defined in TS 36.331 [14]. | YES | ignore |
| SDT Support Request | O |  | 9.2.3.163 |  | YES | ignore |

9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

Omit something here

| Broadcast PLMN Identity Info List NR |  | 0..<maxnoofBPLMNs> |  | This IE corresponds to the PLMN-IdentityInfoList IE and the NPN-IdentityInfoList IE (if available) in SIB1 as specified in TS 38.331 [10]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE and NPN identities and associated information contained in the NPN-IdentityInfoList IE (if available) are included and provided in the same order as broadcast in SIB1. NOTE: In case of NPN-only cell, the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are not included. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

After target cell receives the RRCResumeRequest message from UE, target NG-RAN will send RETRIEVE UE CONTEXT REQUEST message to source/anchor NG-RAN, if you look at the content of RETRIEVE UE CONTEXT REQUEST message, i.e. the highlighted yellow part above, you can easily find that targetCellIdentity associated to target cell is already transferred to source/anchor NG-RAN, but the problem is that the source/anchor NG-RAN will have no idea which PCI and ARFCN the targetCellIdentity is associated with if the target cell is a NPN-only cell, because the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE for NPN-only cell are not exchanged between target NG-RAN and source NG-RAN according to the highlighted green part above defined in [2]; for Non-NPN only cell, there is no problem as all the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE will be exchanged between target NG-RAN and source NG-RAN according to the highlighted pink part above defined in [2]  .

Observation2: From RAN3 spec perspective, targetCellIdentity within the VarResumeMAC-Input as specified in TS 38.331 is already transferred from target NG-RAN to source/anchor NG-RAN, but the source/anchor NG-RAN will have no idea which PCI and ARFCN the targetCellIdentity is associated with when the target cell is a NPN-only cell, because for NPN-only cell, the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are not exchanged between target NG-RAN and source NG-RAN according to RAN3 spec TS 38.423.

Based on observation1 and observation2, we can find that RAN2 spec is not aligned with RAN3 spec, so the next question is that which spec should be changed to follow the another. From RAN3 LS [1], RAN3 prefers RAN2 to solve this misalignment, the suggested solution is to configure the cellIdentity of the first entry of the plmn-IdentityInfoList with the same value as the cellIdentity-r16 of the first entry in the npn-IdentityInfoList-r16 for NPN only cell, this solution can work but it’s too late for R16, our view is to solve this problem based on network-based solution, the simplest solution is to remove the note in TS 38.423, i.e. the highlighted green part above.

Proposal: From RAN2 perspective, RAN2 prefers RAN3 to solve the problem raised in RAN3 LS by network-based solution, for instance, remove the note included in Broadcast PLMN Identity Info List NR defined in clause 9.2.2.11 of TS 38.423.

- Conclusion

In conclusion, we propose the following:

Observation1: From RAN2 spec perspective, UE shall always use the cellIdentity of the first PLMN-Identity included in the PLMN-IdentityInfoList broadcasted in SIB1 of the target, i.e. the cell the UE is trying to resume, when calculating the VarResumeMAC-Input no matter the cell is an NPN-only cell or not.

Observation2: From RAN3 spec perspective, targetCellIdentity within the VarResumeMAC-Input as specified in TS 38.331 is already transferred from target NG-RAN to source/anchor NG-RAN, but the source/anchor NG-RAN will have no idea which PCI and ARFCN the targetCellIdentity is associated with when the target cell is a NPN-only cell, because for NPN-only cell, the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are not exchanged between target NG-RAN and source NG-RAN according to RAN3 spec TS 38.423.

Proposal: From RAN2 perspective, RAN2 prefers RAN3 to solve the problem raised in RAN3 LS by network-based solution, for instance, remove the note included in Broadcast PLMN Identity Info List NR defined in clause 9.2.2.11 of TS 38.423.

- References

[1] 3GPP R2-2206930 LS on NPN only cell

[2] 3GPP TS38.423 h00

- Drafted Reply LS to RAN3

3GPP TSG-RAN WG2 Meeting #119 electronic                 	R2-22

Online, August, 2022

Title:	Reply LS on NPN only cell

Response to:

Release:	Rel-16

Work Item:	NG_RAN_PRN-Core

Source:	RAN2

To:	RAN3

Cc:

Contact Person:

Name:	Jiangsheng Fan

E-mail Address:fanjiangsheng@oppo.com

Attachments:

1. Overall Description:

RAN2 would like to thank RAN3 for the LS on NPN only cell, after discussion in RAN2, RAN2 made the following agreement:

=> RAN2 prefers RAN3 to solve the problem raised in RAN3 LS by network-based solution, for instance, remove the note included in Broadcast PLMN Identity Info List NR defined in clause 9.2.2.11 of TS 38.423.

2. Actions:

To RAN3 group:

ACTION: RAN2 respectfully asks RAN3 to take the above into consideration and give the feedback if any.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 meeting #119bis         10th – 19th October 2022		                          eMeeting

TSG RAN WG2 Meeting #120              14th – 18th November 2022			CA, Canada