3GPP TSG-RAN WG2 Meeting #119-e	R2-2208932

Online, 15th – 26th August, 2022

Agenda Item:	6.12.2.2

Source:	MediaTek Inc.

Title:	On PUCCH configuration in initial UL BWP

Document for:	Discussion and decision

# 1 Introduction

RAN1 has introduced the option to configure different PUCCH resources for RedCap UEs. In this paper, we clarify when such a configuration is expected to be present and which set of PUCCH resources are to be used by RedCap UEs.

# 2 Discussion

The PUCCH resources to be used by a UE during the initial access procedure up until a dedicated configuration is available is provided by the pucch-ResourceCommon IE in the initial UL BWP. For this reason, this field is mandatorily present in an initial BWP, as highlighted in the excerpt from TS38.331 [1] below.

PUCCH-ConfigCommon information element

-- ASN1START

-- TAG-PUCCH-CONFIGCOMMON-START

PUCCH-ConfigCommon ::=            SEQUENCE {

pucch-ResourceCommon             INTEGER (0..15)                                OPTIONAL,   -- Cond InitialBWP-Only

pucch-GroupHopping               ENUMERATED { neither, enable, disable },

hoppingId                        INTEGER (0..1023)                              OPTIONAL,   -- Need R

p0-nominal                       INTEGER (-202..24)                             OPTIONAL,   -- Need R

...,

[[

nrofPRBs                         INTEGER (1..16)                                OPTIONAL,   -- Need R

intra-SlotFH-r17                 ENUMERATED {fromLowerEdge, fromUpperEdge}      OPTIONAL,   -- Need R

pucch-ResourceCommon-RedCap-r17  INTEGER (0..15)                                OPTIONAL,   -- Need R

additionalPRBOffset-r17          ENUMERATED {n2, n3, n4, n6, n8, n9, n10, n12}  OPTIONAL    -- Need S

]]

}

-- TAG-PUCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUCCH-ConfigCommon field descriptions

pucch-ResourceCommon An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters. The UE uses those PUCCH resources until it is provided with a dedicated PUCCH-Config (e.g. during initial access) on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field (see TS 38.213 [13], clause 9.2).

pucch-ResourceCommon-RedCap An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters for RedCap UEs. The UE uses those PUCCH resources until it is provided with a dedicated PUCCH-Config (e.g. during initial access) on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field (see TS 38.213 [13], clause 9.2).

Excerpt 1: PUCCH resource configuration in 38.331 [1]

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | The field is mandatory present in the PUCCH-ConfigCommon of the initial BWP (BWP#0) in SIB1. It is absent in other BWPs. |

RAN1 have now introduced a PUCCH resource configuration to be used by RedCap UEs as highlighted in Excerpt 1 above. It is unclear from the current specification what the expected behaviour if there isn’t a RedCap-specific PUCCH resource configuration. Which set of PUCCH resources should a RedCap UE operating in the legacy initial UL BWP use if pucch-ResourceCommon-RedCap-r17 is absent?

PUCCH resources are always needed in an initial UL BWP for use up until a dedicated PUCCH configuration is provided (i.e. during initial access). This is the reason why the conditional flag was introduced for pucch-ResourceCommon. To ensure that RedCap UEs operating in an initial UL BWP can correctly select PUCCH resources during and after initial access,  we should either:

- Require the NW to always configure pucch-ResourceCommon-RedCap in an initial UL BWP, or

- RedCap UE should use pucch-ResourceCommon if pucch-ResourceCommon-RedCap is absent in an initial UL BWP.

If the PUCCH resources are shared between legacy UEs and RedCap UEs, then the first option results in an unnecessary increase in signalling overhead. If the PUCCH resources are not shared, there are no differences between the two options. Therefore we propose to clarify that if pucch-ResourceCommon-RedCap is absent, a RedCap UE uses the PUCCH resources configured in pucch-ResourceCommon instead.

Proposal 1: If pucch-ResourceCommon-RedCap is absent from an initial UL BWP, a RedCap UE operating in this BWP uses the PUCCH resources configured in pucch-ResourceCommon instead.

If we agree to this proposal, we should inform RAN1 of these agreements in case any RAN1 specification changes are needed.

Proposal 2: Send an LS to RAN1 informing them of our conclusions.

# 3 Conclusion

In this contribution we propose the following:

Proposal 1: If pucch-ResourceCommon-RedCap is absent from an initial UL BWP, a RedCap UE operating in this BWP uses the PUCCH resources configured in pucch-ResourceCommon instead.

Proposal 2: Send an LS to RAN1 informing them of our conclusions.

A TP and a draft LS is provided in the Annex.

# 4 References

- 3GPP TS 38.331 V17.1.0 - NR Radio Resource Control (RRC) protocol specification

# 5 Annex

## Text Proposal

START OF CHANGE

#### PUCCH-ConfigCommon

The IE PUCCH-ConfigCommon is used to configure the cell specific PUCCH parameters.

PUCCH-ConfigCommon information element

-- ASN1START

-- TAG-PUCCH-CONFIGCOMMON-START

PUCCH-ConfigCommon ::=              SEQUENCE {

pucch-ResourceCommon                INTEGER (0..15)                                      OPTIONAL,   -- Cond InitialBWP-Only

pucch-GroupHopping                  ENUMERATED { neither, enable, disable },

hoppingId                           INTEGER (0..1023)                                    OPTIONAL,   -- Need R

p0-nominal                          INTEGER (-202..24)                                   OPTIONAL,   -- Need R

...,

[[

nrofPRBs                            INTEGER (1..16)                                      OPTIONAL,   -- Need R

intra-SlotFH-r17                    ENUMERATED {fromLowerEdge, fromUpperEdge}            OPTIONAL,   -- Need R

pucch-ResourceCommon-RedCap-r17     INTEGER (0..15)                                      OPTIONAL,   -- Need S

additionalPRBOffset-r17                 ENUMERATED {n2, n3, n4, n6, n8, n9, n10, n12}        OPTIONAL    -- Need S

]]

}

-- TAG-PUCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUCCH-ConfigCommon field descriptions

additionalPRBOffset When intra-slot PUCCH frequency hopping within RedCap-specific initial UL BWP is disabled, each common PUCCH resource is mapped to a single PRB on one side of the UL BWP. This parameter determines an additional PRB offset in the PRB mapping for the PUCCH resource. If the field is not configured, the UE shall assume an additional PRB offset of zero.

hoppingId Cell-specific scrambling ID for group hopping and sequence hopping if enabled, see TS 38.211 [16], clause 6.3.2.2.

intra-SlotFH-r17 In case a separate initial UL BWP is configured for RedCap UEs, the presence of this parameter indicates whether intra-slot PUCCH frequency hopping within the separate initial UL BWP in the common PUCCH resource is enabled for RedCap UEs. If this field is absent, intra-slot PUCCH frequency hopping within RedCap-specific initial UL BWP is enabled. If this field is present, intra-slot PUCCH frequency hopping within RedCap-specific initial UL BWP is disabled and each PUCCH resource is mapped to a single PRB on one side of the UL BWP and this parameter determines whether the PRB index in the PRB mapping is counted in increasing order from the lower edge or in decreasing order from the upper edge of the UL BWP.

nrofPRBs Indicates the number of PRBs used per PUCCH resource for PUCCH format 0 and format 1 in FR2-2, see TS 38.213 [13], clause 9.2.1.

p0-nominal Power control parameter P0 for PUCCH transmissions. Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.2).

pucch-GroupHopping Configuration of group- and sequence hopping for all the PUCCH formats 0, 1, 3 and 4. Value neither implies neither group or sequence hopping is enabled. Value enable enables group hopping and disables sequence hopping. Value disable disables group hopping and enables sequence hopping (see TS 38.211 [16], clause 6.3.2.2).

pucch-ResourceCommon An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters. The UE uses those PUCCH resources until it is provided with a dedicated PUCCH-Config (e.g. during initial access) on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field (see TS 38.213 [13], clause 9.2).

pucch-ResourceCommon-RedCap An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters for RedCap UEs. The UE uses those PUCCH resources until it is provided with a dedicated PUCCH-Config (e.g. during initial access) on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field (see TS 38.213 [13], clause 9.2). If the field is absent, a RedCap UE uses pucch-ResourceCommon instead.

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | The field is mandatory present in the PUCCH-ConfigCommon of the initial BWP (BWP#0) in SIB1. It is absent in other BWPs. |

END OF CHANGE

## Draft LS to RAN1

START OF DRAFT LS

Title:	Draft LS on PUCCH configuration for RedCap UEs in initial UL BWP

Response to:

Release:	Rel-17

Work Item:	Support of reduced capability NR devices (NR_redcap-Core)

Source:	MediaTek (To be RAN2)

To:	RAN1

Cc:

Contact person:	Pradeep Jose, pradeep dot jose at mediatek dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 discussed the configuration of PUCCH resources for RedCap UEs and have reached the following agreement:

If pucch-ResourceCommon-RedCap is absent from an initial UL BWP, a RedCap UE operating in this BWP uses the PUCCH resources configured in pucch-ResourceCommon instead.

RAN2 respectfully requests RAN1 to take the above agreement into consideration and update their specifications if necessary.

# 2	Actions

To RAN1:

ACTION: 	RAN2 respectfully requests RAN1 to take the above agreements into consideration and update their specifications if necessary.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG2 Meeting #119-bis-e	10th – 19th October 2022	Online

TSG-RAN WG2 Meeting #120	14th – 18th November 2022	Canada

END OF DRAFT LS