3GPP TSG RAN WG2#119-e	R2-2209129

e-Meeting, 17th - 29th August, 2022

3GPP TSG WG RAN1 #110	R1-2208224

Toulouse, France, August 22nd – 26th 2022

# Title:	Reply LS on LS on further questions on feMIMO RRC parameters

# Response to:	LS (R2-2208964) on further questions on feMIMO RRC parameters

# Release:	Rel-17

# Work Item:	NR_feMIMO-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Claes Tidestav

E-mail Address:	claes.tidestav@ericsson.com

1. Overall Description:

RAN1 thanks RAN2 for the LS on further questions on feMIMO RRC parameters and would like to provide the following response.

RAN2 required input on a set of RRC parameters for Rel-17, and asked the following questions:

Question 1

RAN2 would like to ask RAN1

- RAN2 would like to ask what is the relation between servingcellindex configured in qcl-Type1/qcl-Type2 and the additionalPCI, when additionalPCI is configured? That is, is it correct understanding that additionalPCI is an index referring to a PCI value configured in a list additionalPCI-ToAddModList under a serving cell configuration and thus depending which serving cell it refers to, the exact PCI value may be different?

- RAN2 assumes additionalPCI is per TCI-state and refers to the configured reference signal in case of SSB is configured as reference signal in qcl-Type1 and/or qcl-Type2. That is, there is no such case where qcl-Type1 and qcl-Type2 for the same TCI-state associate with different additionalPCI values. Please confirm whether this is also RAN1’s understanding.

- If additionalPCI is configured, can cell be configured for any of the qcl-Type1 or qcl-Type2?

- if c) is confirmed, would there be need to state that cell cannot be two different values for qcl-Type1 and for qcl-Type2?

Answer to question 1:

RAN1 confirms the following understanding

- The additionalPCI is an index referring to a PCI value configured in a list additionalPCI-ToAddModList under a serving cell configuration and thus depending on which serving cell it refers to, the exact PCI value may be different.

- For a given TCI-state, there is no such case where qcl-Type1 and qcl-Type2 for the same TCI-state associate with different additionalPCI values.

- It is not precluded that cell be configured for any of the qcl-Type1 or qcl-Type2.

- If configured, cell cannot be two different values for qcl-Type1 and for qcl-Type2

Question 2

RAN2 considers the case where a serving cell uses the TCI states defined in another cell, i.e. dl-OrJoint-TCIStateList is set to unifiedTCI-StateRef. and would like to ask RAN1:

- When “cell” is absent in QCL-info, is the referenceSignal located in the serving cell where the TCI-state is configured (dl-orJoint-TCI-State-ToAddModList is in IE PDSCH-Config of this serving cell) or in the serving cell where the TCI-state is used ( unifiedTCI-StateRef is in IE PDSCH-Config of this serving cell)? And is the above limited to certain qcl-Type?

- Is the configuration of the TCI state of the serving cell indicated by unifiedTCI-StateRef still applicable for the serving cell configured with unifiedTCI-StateRef when the serving cell (e.g. SCell) indicated by unifiedTCI-StateRef is deactivated?

Answer to question 2:

RAN1 confirms the following understanding

- The referenceSignal is located in the serving cell where TCI-state is used (unifiedTCI-StateRef is in IE PDSCH-Config of this serving cell). The above is not limited to any qcl-Type.

- Yes, the RRC configuration of the TCI state of the serving cell indicated by unifiedTCI-StateRef is still applicable for the serving cell configured with unifiedTCI-StateRef when the serving cell (e.g. SCell) indicated by unifiedTCI-StateRef is deactivated. UE is not required to track the source RS for QCL/pathloss indication if the source RS is from a deactivated serving cell.

Question 3

RAN2 would like to ask RAN1

- in case the servingCellId is present, does the additionalPCI in IE TCI-UL-State refer to one of additional PCIs configured in the serving cell indicated by the field servingCellId?

- is it correct that there is no qcl-Type field in IE TCI-UL-State as the parameter list excel file in R1-2202759 did not advice to include QCL Type for UL TCI state(row4)?

- If b) is correct, it is assumed that QCL related limitations should be deleted from the field description of the servingCellId? That is, should. "The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeC or typeD. See TS 38.214 [19] clause 5.1.5." in the field description of servingCellId" be deleted?

Answer to Question 3:

RAN1 confirms the following understanding

- Yes. In case the servingCellId is present, the additionalPCI in IE TCI-UL-State refers to one of additional PCIs configured in the serving cell indicated by the field servingCellId.

- Correct that there is no qcl-Type field in IE TCI-UL-State as the parameter list excel file in R1-2202759 did not advice to include QCL Type for UL TCI state (row4).

- Yes, it should be removed.

Question 4

RAN2 considers the case where a serving cell uses the UL TCI states defined in another cell, i.e. ul-TCIStateList is set to unifiedTCI-StateRef. and would like to ask RAN1:

- When ‘servingCellId’ is absent in TCI-UL-State, is the referenceSignal configured in the serving cell where the TCI-UL-state is configured or in the serving cell where the TCI-ULstate is used (in case this serving cell is not directly configured with UL TCI states but is configured with parameter unifiedTCI-StateRef )?

- Is the configuration of the UL TCI state of the serving cell indicated by unifiedTCI-StateRef still applicable for the serving cell configured with unifiedTCI-StateRef when the serving cell (e.g. SCell) indicated by unifiedTCI-StateRef is deactivated?

Answer to Question 4:

RAN1 confirms the following understanding

- When ‘servingCellId’ is absent in TCI-UL-State, the referenceSignal is configured in the serving cell where the TCI-ULstate is used (in case this serving cell is not directly configured with UL TCI states but is configured with parameter unifiedTCI-StateRef).

- Yes, the configuration can be applicable. UE is not required to track the source RS for QCL/pathloss indication if the source RS is from a deactivated serving cell.

RAN2 also discussed about the configuration flexibility of the UL power control.

In Rel-17 unified TCI framework, TCI-State (joint type) and TCI-UL-State-r17 (UL-only type) can be optionally configured with a set of power control parameters (ul-powerControl-r17). According to TS 38.331 V17.1.0, there are two possible configuration cases: a) ul-powerControl-r17 is present in BWP-UplinkDedicated and it is absent in all joint TCI states used together with this BWP-UplinkDedicated and UL TCI states used in this BWP-UplinkDedicated, b) ul-powerControl-r17 is absent in BWP-UplinkDedicated and it is present in all joint TCI states used together with this BWP-UplinkDedicated and UL TCI states used together with this BWP.

RAN1’s agreements do not exclude the case that ul-powerControl-r17 is present in some TCI states and is absent in other TCI states (case c)). In case c), ul-powerControl-r17 can be configured in both BWP-UplinkDedicated and joint TCI-State/TCI-UL-State-r17 and the UE uses ul-powerControl-r17 in BWP-UplinkDedicated only when the TCI state used is not configured with ul-powerControl-r17. However, this case is currently excluded by RAN2 specifications

Question 5

RAN2 would like to ask RAN1 whether current specification is sufficient for UL power control or whether further flexibility, such as case c), should be supported?

Answer to question 5:

There is no consensus in RAN1 if the current specification is sufficient for UL power control and if further flexibility, such as case c, is needed.

Question 6

- Does RAN2 have correct understanding for PH report, i.e.:

  - the UE provides two Type 1 PH value for the serving cell if there is actual or reference PUSCH transmission on both TRP for slot n.

  - the UE provides one Type 3 PH value for the serving cell if there is actual or reference SRS transmission for slot n.

- If a) is correct, in which case will the UE report type 3 PH value for this serving cell?

Answer to question 6:

(a).  Yes, RAN2 understanding is correct.

(b).  For type 3 PH value determination, legacy procedure applies.

2. Actions:

To RAN2 group:

ACTION:

RAN1 respectfully asks RAN2 to take the above conclusions into consideration.

3. Date of Next RAN1 Meetings:

TSG WG RAN1 #110-bis-e 	10th – 19th October 2022	Online

TSG WG RAN1 #111		14th – 18th November 2022	Europe