3GPP TSG RAN WG2#119-e	R2-2209158

e-Meeting, 17th - 29th August, 2022

3GPP TSG-RAN WG4 Meeting #104-e	R4-2214411

Electronic Meeting, August 15- 26, 2022

Title:	LS on intraBandENDC-Support

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: 	Hu Dan

E-mail Address: 	hudan11@huawei.com

Attachments:	N/A

1. Overall Description:

In Rel-15 and Rel-16, RAN4 introduced intra-band contiguous and non-contiguous ENDC configurations with more than 2 carriers, as illustrated in Table 1.

Table 1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_41A_n41A |
| DC_(n)48CA | DC_(n)48AA DC_48A_n48A |
| DC_(n)48DA | DC_(n)48AA DC_48A_n48A |

For the configuration of intra-band ENDC combination with more than 2 carriers, it is possible that

- All CCs are contiguous in DL but neither carrier is contiguous to each other in UL, e.g., DL DC_(n)41AB with UL DC_41A_n41A

- LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, e.g., DL DC_48A_(n)48AA with UL DC_(n)48AA, DL DC_48A_(n)48AA with UL DC_48A_n48A

The IE intraBandENDC-Support in UE capability can indicate three states {contiguous, non-contiguous, both} per band combination. Whether the Band combination is contiguous or non-contiguous is unclear if UL and DL configurations are different or if LTE and NR carriers are in mixed contiguous and non-contiguous configuration. Thus there could be confusion when the UE indicates either of the 2 cases above with the capability of intraBandENDC-Support.

For example, if the UE reports ‘contiguous’ of Band combination DC_(n)41AB, it may not support DC_41A_n41A(non-contiguous) in uplink. But considering the table above defined in TS 38.101-3, the BS would get confused that whether the UE supports DC_41A_n41A(non-contiguous) in uplink or not.

For the problems of ambiguity on some intra-band ENDC band combinations with more than 2 carriers, new signalling could be a possible solution. Though the ambiguity issue for intra-band EN-DC combinations still exists from Rel-16, considering the backward compatibility issue, RAN4 agrees that new signalling if needed could be introduced from Rel-18.

RAN4 also discussed the possible solutions to address the issue, but no consensus yet [1][2][3]. Contributions listed below are just for RAN2's reference.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to consider a solution to solve the problems of ambiguity on some intra-band ENDC band combinations with more than 2 carriers.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#104bis-e	Oct 2022	E-meeting

TSG-RAN4 Meeting#105	Nov 2022	TBD

# References

[1] R4-2212852, “Draft CR for 38.101-3 Rel-16 intra-band contiguous EN-DC band combination”, Google Inc., Comcast, CableLabs, Federated Wireless, RAN4#104e, August 15-26, 2022

[2] R4-2212854, “Draft CR for 38.101-3 Rel-17 intra-band contiguous EN-DC band combination”, Google Inc., Comcast, CableLabs, Federated Wireless, RAN4#104e, August 15-26, 2022

[3] R4-2212582, “Draft CR for 38.101-3 Rel-16 to correct band combination for intra-band ENDC”, Xiaomi, RAN4#104e, August 15-26, 2022