3GPP TSG-RAN WG2 Meeting #119	R2-2209276

# Electronic, 17 - 29 Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3359 | rev | 3 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MINT - applicableDisasterInfoList [MINT] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
|  | e |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SIB1 indicates the PLMNs that share a cell in plmn-IdentityInfoList in cellAccessRelatedInfo. plmn-IdentityInfoList is a list of lists: each entry in plmn-IdentityInfoList has a plmn-IdentityList. This allows this example scenario: Entry 1 of plmn-IdentityInfoList: plmn-IdentityInfoList{PLMN A; PLMN B} Entry 2 of plmn-IdentityInfoList: plmn-IdentityInfoList{PLMN C} Entry 3 of plmn-IdentityInfoList: plmn-IdentityInfoList{PLMN D, PLMN E} The motivation for having a list of lists like this is that the PLMNs sharing an entry of plmn-IdentityInfoList (e.g. PLMN A and PLMN B above) can share cell identity, tracking area code, etc. Note therefore that there can be more than one plmn-IdentityList. When MINT was introduced in RRC, it was added signalling of so called disaster information (sent in applicableDisasterInfoList). Disaster information is indicated as a list, the intention was that the network should be able to signal different disaster information for the different networks that share the cell, similar to how different networks sharing the cell could have different cell identity, tracking area code, etc. The field description of applicableDisasterInfoList has an error though. The error is that it says that applicableDisasterInfoList has one entry for each entry of plmn-IdentityList. But the intention was to have one entry for each entry of plmn-IdentityInfoList. This error makes it impossible to provide disaster information for the scenario shown above as it is unclear for which of the (more than one) plmn-IdentityList the disaster roaming information applies. The above error applies also to NPNs where the intention was to refer to npn-IdentityInfoList-r16 instead of npn-IdentityList-r16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the field desciption of applicableDisasterInfoList it is changed to say that there is one entry per entry of plmn-IdentityInfoList and npn-IdentityInfoList-r16 rather than one entry per entry of plmn-IdentityList and npn-IdentityList-r16. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: MINT Inter-operability: If the network is implemented according to the CR and the UE is not or vice versa, the UE will not be able to decode applicableDisasterInfoList correctly and may therefore do (or not do) disaster roaming erroneously. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to provide disaster related information in case there are more than one entry in plmn-IdentityInfoList or more than one entry in npn-IdentityInfoList-r16. And it is ambigious which disaster information applies to which network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.1	System information blocks

[Omitted unchanged parts]

#### –	SIB15

SIB15 contains configurations of disaster roaming information.

SIB15 information element

-- ASN1START

-- TAG-SIB15-START

SIB15-r17 ::=                          SEQUENCE {

commonPLMNsWithDisasterCondition-r17   SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity                   OPTIONAL,  -- Need R

applicableDisasterInfoList-r17         SEQUENCE (SIZE (1..maxPLMN)) OF ApplicableDisasterInfo-r17      OPTIONAL,  -- Need R

lateNonCriticalExtension               OCTET STRING                                                    OPTIONAL,

...

}

ApplicableDisasterInfo-r17    ::= CHOICE {

noDisasterRoaming-r17             NULL,

disasterRoamingFromAnyPLMN-r17    NULL,

commonPLMNs-r17                   NULL,

dedicatedPLMNs-r17                SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity

}

-- TAG-SIB15-STOP

-- ASN1STOP

SIB15 field descriptions

commonPLMNsWithDisasterCondition A list of PLMN(s) for which disaster condition applies and that disaster inbound roaming is accepted, which can be commonly applicable to the PLMNs sharing the cell.

applicableDisasterInfoList A list indicating the applicable disaster roaming information for the networks indicated in plmn-IdentityInfoList and npn-IdentityInfoList-r16. The network indicates in this list one entry for each entry of plmn-IdentityInfoList, followed by one entry for each entry of npn-IdentityInfoList-r16, meaning that this list will have as many entries as the number of entries of the combination of plmn-IdentityInfoList and npn-IdentityInfoList-r16. The first entry in this list indicates the disaster roaming information applicable for the network(s) in the first entry of plmn-IdentityInfoList/npn-IdentityInfoList-r16, the second entry in this list indicates the disaster roaming information applicable for the network(s) in the second entry of plmn-IdentityInfoList/npn-IdentityInfoList-r16, and so on. Each entry in this list can either be having the value noDisasterRoaming, disasterRoamingFromAnyPLMN, commonPLMNs, or dedicatedPLMNs. If an entry in this list takes the value noDisasterRoaming, disaster inbound roaming is not allowed in this network(s). If an entry in this list takes the value disasterRoamingFromAnyPLMN, this network(s) accepts disaster inbound roamers from any other PLMN (except those indicated in SIB1). If an entry in this list takes the value commonPLMNs, the PLMN(s) with disaster conditions indicated in the field commonPLMNsWithDisasterCondition apply for this network(s). If an entry in this list contains the value dedicatedPLMNs, the listed PLMN(s) are the PLMN(s) with disaster conditions that the network(s) corresponding to this entry accepts disaster inbound roamers from. For SNPNs, the network indicates the value noDisasterRoaming.