3GPP TSG-RAN WG2 Meeting #119-bis electronic	R2-2209560

e-Meeting, 10th October – 19th October 2022

Source:	vivo

Title:          Discussion on sidelink positioning

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#119 meeting, RAN2 discussed the sidelink positioning topic and achieved the following agreements:

Agreements:

Proposal 1 (modified): Confirm that for sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios shall be supported.  FFS if partial coverage case assumes anything about which UEs are in coverage.

Proposal 2: Study the architecture and signaling procedures to enable at least the following two operation scenarios:

-	Operation Scenario 1: PC5-only-based positioning.

-	Operation Scenario 2: Combination of Uu- and PC5-based positioning.

Agreement:

RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server.  FFS from RAN2 perspective if there are cases without a UE in the location server role.

Agreement:

Proposal 4 (modified): Align with SA2/RAN1 on the terms for sidelink positioning, and introduce the following terms of UE role as the baseline for further discussion:

-	Target UE: UE to be positioned

-	Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface.  FFS: clarification of the knowledge of the anchor UE.

Additional roles can be considered.

Agreements:

Introduce a new protocol for sidelink positioning procedures between UEs (name FFS, e.g., RSPP, SLPP).  FFS where it is specified.

The new protocol is a separate ASN.1 module from LPP (this does not necessarily imply whether it is included in 37.355).

Agreement:

Study the potential impact to LPP for support of sidelink positioning procedures between UE and LMF.  FFS how much impact (if any), e.g., only to carry the new protocol, and if the PC5-only and hybrid PC5+Uu cases are the same or different.

Agreement:

RAN2 wait for SA2 on the triggering of the positioning procedures from upper layers.

Agreement:

RAN2 will study the question of cast type for positioning signalling.  For SL-PRS, follow RAN1 decision and consider cast type if something arises in RAN2 scope.

In this contribution, we will continue to discuss the following aspects of sidelink positioning based on the above agreements from RAN2’s perspective:

- Partial coverage scenario

- UE roles

- Protocol stack

- Cast types

- General signaling procedure

- Latency and efficiency requirements

- Discussion

- 2.1 Partial coverage scenario

In the last RAN2 meeting, RAN2 confirmed that three network coverage scenarios (in-coverage, partial coverage, and out-of-coverage scenarios) shall be supported. There is an FFS if partial coverage case assumes anything about which UEs are in coverage. For sidelink positioning in the partial coverage scenario, which UEs being in coverage is related to the definition of UE roles. RAN2 has defined the “Target UE” and “Anchor UE” roles and RAN2 also confirmed that additional roles can be considered. Since the partial coverage issue is related to role definition, we understand that RAN2 should first complete the work of role definition, and then discuss the partial coverage issue. Otherwise, for example, if RAN2 defines the partial coverage only based on “Target UE” and “Anchor UE”, and then a new role(s) is introduced again, RAN2 needs to revisit to define the partial coverage.

Proposal 1: RAN2 to discuss the partial coverage issue (i.e., which UEs are in coverage) only after completing the work of role definition.

In the last meeting, some companies considered using the UE-to-NW relay to cope with the partial coverage scenario. In Rel-17, the sidelink relay is introduced to provide connectivity to the network for U2N Remote UE(s). Both L2 and L3 U2N Relay architectures are supported. For L3 U2N Relay, the architecture is transparent to the serving RAN of the U2N Relay UE, except for controlling sidelink resources. According to TS23.304 [2], the control plane protocol stack is as below:

Observation 1: For L3 U2N Relay, the transmission path for NAS messages is quite long (Remote UE -> Relay UE -> RAN -> UPF -> N3IWF -> AMF).

Connection establishment for the L3 U2N relay is also needed:

For L2 U2N Relay, the following high-level connection establishment procedure is applied (extracted from TS 38.300 [3]), which includes relay connection setup, relay SRB/DRB setup, relay security activation:

Figure 16.12.5.1-1: Procedure for L2 U2N Remote UE connection establishment

Observation 2: For L2/L3 U2N Relay, an (AS or NAS) connection establishment procedure is needed before relay transmission.

To our understanding, the U2N relay feature was developed mainly for (plenty of) user data relay. If the U2N relay is used for sidelink positioning in the partial coverage scenario, there are the following drawbacks:

- It requires that the related UEs must support the relay feature and also the operator must deploy the relay in the network (RAN and core network). Otherwise, the SL positioning in the partial coverage scenario can not work. This greatly restricts the applicable scenario.

- The relay connection establishment procedure increases the signaling overhead and the positioning time delay.

- For L3 U2N relay, the transmission path for NAS messages is quite long (Remote UE -> Relay UE -> RAN -> UPF -> N3IWF -> AMF), which increases the positioning time delay.

The sidelink positioning procedure is only related to control signaling interaction. The amount of exchanged messages between LMF and UE is quite limited. It is completely unnecessary to use the U2N relay feature. We understand that another solution is message forwarding via control signaling. Figure 1 shows an example that LMF sends the message 1 to UE2 via UE1:

Figure 1: message forwarding via control signaling

Proposal 2: For the partial coverage scenario, do not consider using the U2N relay feature.

Proposal 3: Message forwarding via control signaling is adopted for partial coverage scenario.

- 2.2 UE roles

In the last RAN2 meeting, RAN2 introduced two roles: target UE and anchor UE. The location server UE was discussed, but some companies thought that it is SA2 scope and the server UE has security implications that need to be evaluated. We have a different understanding. Firstly, SA2 does not care about the detailed positioning execution procedure, but RAN2 does. In a detailed PC5-only positioning execution procedure, from the RAN2’s view, there should be an entity that determines the positioning method(s), how many and which UEs act as anchor UEs, who executes the location calculation, and maybe also the SL PRS configuration, to fulfill the positioning QoS. Therefore, the definition of location server UE role is not only related to SA2 but more related to RAN2. In other words, it is more of RAN2’s need to define the location server UE.

Observation 3: From the RAN2’s view, in a detailed PC5-only positioning execution procedure, there should be an entity that determines the positioning method(s), how many and which UEs act as anchor UEs, who executes the location calculation, and maybe also the SL PRS configuration, to fulfil the positioning QoS.

In the traditional Uu-based positioning, the participants include target UE, gNB, and LMF, all of which are reliable. For sidelink positioning we also need to ensure the participants are reliable, e.g., the participants need to be authenticated. Otherwise, e.g., if some of the anchor UEs provides fake measurement result, fake SL PRS configurations, or fake positioning results, the positioning result may have quite an offset far from the real location.

Observation 4: For sidelink positioning, need to ensure all of the participants including anchor UE are reliable, though it is SA2’s scope.

In Rel-17, positioning reference units (PRUs) were discussed. PRUs with known locations can enhance positioning performance. PRU needs to be reliable, similar to anchor UE. SA2 discussed potential solutions in [4]. We understand that a similar scheme can be applied to anchor UE.

If location server UE is introduced, the same scheme can be also applied to location server UE, though it is SA2’s scope. Compared with anchor UE, no additional security issues are foreseen for location server UE.

Observation 5: The similar security scheme can be applied to anchor UE and location server UE. Compared with anchor UE, no additional security issues are foreseen for location server UE.

In the last RAN2 meeting, one company commented “having a server UE could mean that all UEs have to be connected to the server”. We understand that there is no such restriction, e.g., the location server UE can communicate with anchor UE via target UE, i.e., the target UE can forward messages via control signaling if there is no connection between location server UE and anchor UE.

In SA2#152 meeting, SA2 sent the LS [7] to RAN1/2/3 to ask RAN WGs to evaluate if the terminologies defined by SA2 are aligned and if any further updates are needed, and gives a feedback to SA2. In the attached paper [8], there is the following change to the definition of the location server UE:

As we know, LMF determines who to execute the location calculation function in Rel-17. If it is the target UE to calculate the location, the positioning is UE-based positioning; if it is the LMF to calculate the location, the positioning is LMF-based positioning. For sidelink positioning, we understand that it should be still location server functionality to determine who to execute the location calculation function, e.g., target UE, anchor UE, or location server functionality itself. The essential functions of the location server should be according to positioning QoS determining the positioning method(s), how many and which UEs act as anchor UEs, who executes the location calculation, etc, but not including the location calculation. We think that it is unnecessary to fix an individual role in the specification which executes the location calculation function.

Based on the above analysis, we propose:

Proposal 4: From the RAN2’s view, introduce the location server UE role, and send an LS to SA2 to align the definition of location server UE:

- Location Server UE: A UE offering location server functionality (e.g., according to positioning QoS, determining the positioning method(s), how many and which UEs act as anchor UEs, who executes the location calculation, etc), for sidelink Positioning and Ranging over PC5.

We understand that if having the related capability, target UE and anchor UE can act as location server UE, as below:

Figure 2

The sidelink positioning WI [5] needs to support multiple use cases, including V2X, public safety, commercial and IIOT. Therefore, there may be various UE capabilities. Depending on UE capability, the above 3 cases may be used. For example, for the IIOT case, the capability of the target UE may be low and Case 2 may be not applicable.

Proposal 5: With specific capability, target UE or anchor UE can act as location server UE.

- 2.3	Protocol stack

In the last RAN2 meeting, RAN2 agreed to introduce a new protocol for sidelink positioning procedures between UEs. There are two FFS: the name is FFS, e.g., RSPP (Ranging/Sidelink Positioning Protocol), SLPP (Sidelink Positioning Protocol), and FFS where it is specified.

According to SA2 TR [6], sidelink positioning includes relative positioning, ranging and absolute positioning.

Sidelink Positioning: Positioning UE using PC5 to obtain absolute position, relative position, or ranging information.

Ranging is one of 3 positioning modes and is contained in sidelink positioning. The Ranging and Sidelink Positioning are at different levels. Therefore, we prefer the name SLPP.

Proposal 6: Use the SLPP (Sidelink Positioning Protocol) as the name of the new protocol between UEs.

For LMF-independent positioning, SA2 had some discussions about the protocol stack over PC5 in SA2 TR [6]. SA2 thinks that the position of SLPP in the protocol stack should be negotiated with RAN Working Groups.

TR 23.700-86: # 4 Architecture assumptions and requirements <ignore unrelated part> - SR5: The reference point between the Sidelink (SL) Positioning and Ranging function in UEs. It is carried over the PC5 reference point. Editor's note: Whether SR5 will be over PC5-RRC, PC5-D, PC5-U and/or PC5-S will be aligned with RAN WGs.

Regarding the position, two alternatives were captured in SA2 TR [6]:

-	Alt 1: (Solution #4) CP-based solution, with new protocol over V2X/ProSe layer, e.g., PC5-D, PC5-S;

-	Alt 2: (Solution #14) UP-based solution, with new protocol over PC5-U.

Figure 3: new protocol over V2X/ProSe Layer

Figure 4: new protocol over PC5-U

From the view of RAN2, in addition to Alt 1, the following CP-based alternatives should be considered:

-	Alt 3: CP-based solution, with new protocol over PC5-RRC.

-	Alt 4: CP-based solution, with new protocol over PDCP.

Figure 5: SLPP over PC5 RRC

Figure 6: SLPP over PDCP

For the UP-based solution, one issue is that the transmission priority of SLPP messages cannot be guaranteed since the priority of SL DRB is lower than that of SL SRB. Especially, when the UE has plenty of data to transfer and/or the sidelink resources are scarce, the time delay may be quite significant. However, the time delay is very important for positioning service since it is one of the positioning QoS parameters. Also, SLPP messages are most likely defined by RAN2. It is a little strange that SLPP messages defined by RAN2 are transmitted as user data. By comparison, the transmission priority of SLPP messages can be guaranteed in the CP-based solutions.

Among the CP-based solutions, the lower layer the SLPP is located on, the less the signaling overhead, processing overhead and positioning time delay will be. Therefore, we prefer Alt 4.

Proposal 7: SLPP is located over PDCP, similar to PC5-S.

For LMF-dependent sidelink positioning, there are three different options with respect to how to carry the sidelink positioning related signaling between the LMF and the UE:

- Option 1: Using SLPP which is carried over NAS.

- Option 2: Extension of LPP, i.e., introducing new IEs for sidelink positioning related parameters in the LPP messages.

- Option 3: Enhancement of LPP, i.e., using SLPP which is carried over LPP.

There are two cases within the research scope: PC5-only case and hybrid PC5 + Uu case. We understand that PC5-only based positioning is the subset of hybrid positioning. They should take the same option to reduce the specification complexity.

Proposal 8: For LMF-dependent sidelink positioning, PC5-only case and hybrid case take the same option to carry the sidelink positioning related signaling between the LMF and the UE.

With Option 1, the sidelink positioning is completely isolated from Uu positioning even for the hybrid case, which seems that they belong to different positioning sessions. However, in the email discussion “[Post119-e][406][POS] Sidelink positioning protocol issues”, the majority thought that the Uu and PC5 based positioning procedures belong to the same positioning session. Moreover, Option 1 has specification impacts on SA2, e.g., differentiating SLPP message from LPP and LCS message in NAS message. Option 2 is a straightforward way to extend LPP to further support the SL positioning as a new positioning method, which benefits for hybrid positioning.

Proposal 9: Reuse and extend the LPP for sidelink positioning procedures between UE and LMF.

Option 3 is more like a compromise between Option 1 and Option 2. Option 3 reduces the burden to maintain the LPP specification and is acceptable to us if preferred by the majority.

- 2.4	Cast types

During the last meeting, there was some discussion on the support of different cast types for positioning signaling over PC5. For sidelink positioning, RAN1 has agreed to support multiple positioning methods:

Agreement With regards to the Positioning methods supported using SL measurements study further the following methods: - RTT-type solutions using SL - Study both single-sided (also known as one-way) and double-sided (also known as two-way) RTT - SL-AoA - Include both Azimuth of arrival (AoA) and zenith of arrival (ZoA) in the study - SL-TDOA - SL-AoD - Corresponds to a method where RSRP and/or RSRPP measurements similar to the DL-AoD method in Uu. - Include both Azimuth of departure (AoD) and zenith of departure (ZoD) in the study

It is likely that one anchor UE supports some methods, but not supports the others. Anchor UE’s capability should be obtained before being selected. If the capability exchange is only via a sidelink unicast connection, the sidelink unicast connection will be set up first. If the anchor UE’s capability does not fulfil the requirements, the anchor UE cannot be selected and the unicast connection may be released. Broadcast/groupcast of positioning capability benefits for anchor UE selection to avoid unnecessary sidelink unicast connection setup. Therefore, we propose to support the broadcast/groupcast for UE sidelink positioning capability.

If anchor UE can autonomously determine its SL PRS configuration, broadcast/groupcast of SL positioning assistance data is similar to posSIB and benefits for signaling overhead reduction, e.g., avoiding sidelink unicast connection setup.

Proposal 10: support the broadcast/groupcast for SL positioning assistance data and UE sidelink positioning capability.

- 2.5	General signaling procedures

- 2.5.1 LMF-dependent sidelink positioning signaling procedures

The following Figure 8 illustrates a typical signaling procedure for LMF-dependent sidelink positioning.

Figure 7: LMF-dependent sidelink positioning signaling procedures

1.	The LMF receives a Location Service Request from the AMF.

2.	The LMF obtains the sidelink positioning capability of the target UE from the target UE, e.g., via the LPP Request Capabilities message.

3.	The LMF determines to perform the sidelink positioning, e.g., when the UE is not able to perform Uu-based positioning (not support, or not able to measure on enough TRP to perform Uu-based positioning).

4.	The LMF obtains the information of Anchor UE(s), e.g., the target UE may initiate the discovery procedure over the PC5 interface to find the Anchor UE(s) and sends the information of Anchor UE(s) to the LMF.

5.	The LMF may perform the exchange of LPP messages for sidelink positioning between the LMF and Anchor UE(s), e.g., Assistance Data Transfer and Location Information Transfer.

6.	The LMF performs the exchange of LPP messages for sidelink positioning between the LMF and the target UE, e.g., Assistance Data Transfer and Location Information Transfer.

7.	The LMF obtains the location information of the target UE based on the information from Step 5 or 6.

8.	The LMF sends Location Service Response to the AMF.

Proposal 11: For LMF-dependent sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Potential standard impact:

-	New or enhanced existing signaling procedure to exchange capability for sidelink positioning.

-	New or enhanced existing discovery procedure to obtain the information of Anchor UEs. (in SA2 scope, if reuse the ProSe discovery procedure)

-	New or enhanced existing signaling procedure to exchange the assistant information and measurement result/position information for sidelink positioning.

Proposal 12: For LMF-dependent sidelink positioning, the potential standard impact includes enhancing the LPP messages about Capability transfer, Assistance Data transfer, and Location Information Transfer.

- 2.5.2 LMF-independent sidelink positioning signaling procedures

The following Figure 9 illustrates a typical signaling procedure for LMF-independent sidelink positioning.

Figure 8: LMF-independent sidelink positioning signaling procedures

1.	The Location Server UE receives a Sidelink Location Request message, or the Location Server UE itself determines the need for some location service.

2.	The Location Server UE may initiate the discovery and/or sidelink unicast link establishment with target UE and Anchor UEs.

3.	The Location Server UE initiates the sidelink positioning capability exchange procedure with the target UE and Anchor UEs.

4.	The Location Server UE performs the assistance data transfer procedure with target UE and Anchor UEs.

5.	The target UE transmits the SL PRS and the Anchor UEs measure the SL PRS, or the Anchor UEs transmit the SL PRS and the target UE measures the SL PRS.

6.	The Location Server UE performs the location information transfer procedure with the target UE or Anchor UEs.

7.	The LMF obtains the location information of the target UE based on the information from Step 6.

8.	The LMF provides the location estimate via Sidelink Location Response.

Proposal 13: For LMF-independent sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Potential standard impact:

-	New protocol for sidelink positioning may need to be introduced, including new signaling procedures for sidelink positioning capability exchange, assistance data transfer and location information transfer.

-	New signaling procedure for Sidelink Location Request/Response. (in SA2 scope, if the request/response are LCS messages)

-	New or enhanced existing discovery procedure. (in SA2 scope, if reuse the ProSe discovery procedure)

Proposal 14: For LMF-independent sidelink positioning, the potential standard impact includes new SLPP messages for the procedures between UEs, including Capability transfer, Assistance Data transfer, and Location Information Transfer.

Note that there are some potential standard impacts on SA2 side, RAN2 shall LS to SA2 if needed.

Proposal 15: When RAN2 concludes on the standard impact, LS to SA2 if needed, e.g., on protocol stack and discovery enhancement.

- 2.6	Latency and efficiency requirements

To reduce the latency and improve efficiency, some features were introduced for Uu-based positioning in Rel-17, e.g.:

-	Preconfigured assistance data: Multiple DL-PRS configurations can be provided to the UE before or during an ongoing LPP positioning session, to be then utilized for potential positioning measurements at a future time.

-	Preconfigured measurement gap: A set of measurement gap patterns can be pre-configured to UE and the UE can request activation and deactivation of the pre-configured MG via MAC CE.

-	On-Demand PRS: The LMF may control and decide whether PRS is transmitted or not and change the characteristics of an ongoing PRS transmission. The on-demand PRS transmission procedure can be initiated either by the UE or LMF.

For the sidelink positioning at specific scenarios, e.g., V2X, the latency and efficiency requirement will be critical. Therefore, we need to investigate how to reduce signaling overhead and latency for sidelink positioning, the potential aspects include：

-	Aspect 1: reducing the interaction and latency of configuration signaling,

-	Aspect 2: reducing unnecessary PC5 and/or Uu connection establishment,

-	Aspect 3: reducing the probability of positioning failure due to inappropriate configuration.

Proposal 16: Study essential solutions to support the latency and efficiency requirements for sidelink positioning, the potential optimization aspects include:

- reducing the interaction and latency of configuration signaling,

- reducing unnecessary PC5 and/or Uu connection establishment,

- reducing the probability of positioning failure due to inappropriate configuration.

- Conclusion

Partial coverage scenario

Proposal 1: RAN2 to discuss the partial coverage issue (i.e., which UEs are in coverage) only after completing the work of role definition.

Observation 1: For L3 U2N Relay, the transmission path for NAS messages is quite long (Remote UE -> Relay UE -> RAN -> UPF -> N3IWF -> AMF).

Observation 2: For L2/L3 U2N Relay, an (AS or NAS) connection establishment procedure is needed before relay transmission.

Proposal 2: For the partial coverage scenario, do not consider using the U2N relay feature.

Proposal 3: Message forwarding via control signaling is adopted for partial coverage scenario.

UE roles

Observation 3: From the RAN2’s view, in a detailed PC5-only positioning execution procedure, there should be an entity that determines the positioning method(s), how many and which UEs act as anchor UEs, who executes the location calculation, and maybe also the SL PRS configuration, to fulfil the positioning QoS.

Observation 4: For sidelink positioning, need to ensure all of the participants including anchor UE are reliable, though it is SA2’s scope.

Observation 5: The similar security scheme can be applied to anchor UE and location server UE. Compared with anchor UE, no additional security issues are foreseen for location server UE.

Proposal 4: From the RAN2’s view, introduce the location server UE role:

- Location Server UE: A UE offering location server functionality (e.g., according to positioning QoS, determining the positioning method(s), how many and which UEs act as anchor UEs, who executes the location calculation, etc), for sidelink Positioning and Ranging over PC5.

Proposal 5: With specific capability, target UE or anchor UE can act as location server UE.

Protocol stack

Proposal 6: Use the SLPP (Sidelink Positioning Protocol) as the name of the new protocol between UEs.

Proposal 7: SLPP is located over PDCP, similar to PC5-S.

Proposal 8: For LMF-dependent sidelink positioning, PC5-only case and hybrid case take the same option to carry the sidelink positioning related signaling between the LMF and the UE.

Proposal 9: Reuse and extend the LPP for sidelink positioning procedures between UE and LMF.

Cast types

Proposal 10: support the broadcast/groupcast for SL positioning assistance data and UE sidelink positioning capability.

General signaling procedures

Proposal 11: For LMF-dependent sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Proposal 12: For LMF-dependent sidelink positioning, the potential standard impact includes enhancing the LPP messages about Capability transfer, Assistance Data transfer, and Location Information Transfer.

Proposal 13: For LMF-independent sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Proposal 14: For LMF-independent sidelink positioning, the potential standard impact includes new SLPP messages for the procedures between UEs, including Capability transfer, Assistance Data transfer, and Location Information Transfer.

Proposal 15: When RAN2 concludes on the standard impact, LS to SA2 if needed, e.g., on protocol stack and discovery enhancement.

Latency and efficiency requirements

Proposal 16: Study essential solutions to support the latency and efficiency requirements for sidelink positioning, the potential optimization aspects include:

- reducing the interaction and latency of configuration signaling,

- reducing unnecessary PC5 and/or Uu connection establishment,

- reducing the probability of positioning failure due to inappropriate configuration.

- Reference

- R2-2207081 Discussion on sidelink positioning, vivo

- 3GPP TS 23.304 v17.4.0

- 3GPP TS 38.300 v17.1.0

- 3GPP TR 23.700-71 V0.4.0

- RP-213588		Revised SID on Study on expanded and improved NR positioning

- 3GPP TR 23.700-86 Study on Architecture Enhancement to support Ranging based services and sidelink positioning

- S2-2207129 LS on Terminology Alignment for Ranging/Sidelink Positioning SA2

- S2-2207117 General: Clarification on Location Server UE and peer-to-peer operation, MediaTek Inc.