3GPP TSG-RAN WG2 Meeting #119bis electronic					      R2-2209567

Online, Oct. 10th- Oct. 19th, 2022

Source: 	vivo

Title:  	Discussion on RACH report enhancement for RACH partitioning

Agenda Item:	8.13.6

Document for:	Discussion and Decision

Introduction

At RAN2#119e, the following agreements were achieved regarding RACH report enhancement:

Agreements

RACH report

1	RAN2 to discuss RACH partitioning for RACH report enhancements.

2	RAN2 is asked to discuss the support of (NG)EN-DC and NE-DC scenarios for SN RACH report. Only focus on the leftover issues for completing the whole work which partly done in R17 in RAN3. Draft LS to RAN3 ask for clarification. (Ericsson)

In this contribution, we will provide our analyses on the function and content of RACH report enhancement for RACH partitioning.

Discussion

In Rel-17, several features were introduced to enable early indication to the network by dedicated RACH resources.

| Feature | Reason for RACH indication |
| --- | --- |
| RedCap | To indicate reduced capabilities to the network so that the network can adapt subsequent transmissions |
| SDT | To indicate a larger MSG3 size request (or MSGA size in case of 2-step RA) |
| Coverage enhancement | To indicate the need for coverage enhancement (e.g., request for MSG3 repetition) |
| Slicing | To indicate high priority slice to the network and to achieve slice isolation also for RACH |

Due to limited RACH resources, RACH partitioning was introduced to enable shared RACH resources for multiple features.

SIB1-v1700-IEs ::= SEQUENCE { hsdn-Cell-r17 ENUMERATED {true} uac-BarringInfo-v1700 SEQUENCE { uac-BarringInfoSetList-v1700 UAC-BarringInfoSetList-v1700 } sdt-ConfigCommon-r17 SDT-ConfigCommonSIB-r17 redCap-ConfigCommon-r17 RedCap-ConfigCommonSIB-r17 featurePriorities-r17 SEQUENCE { redCapPriority-r17 FeaturePriority-r17 slicingPriority-r17 FeaturePriority-r17 msg3-Repetitions-Priority-r17 FeaturePriority-r17 sdt-Priority-r17 FeaturePriority-r17 } si-SchedulingInfo-v1700 SI-SchedulingInfo-v1700 hyperSFN-r17 BIT STRING (SIZE (10)) eDRX-AllowedIdle-r17 ENUMERATED {true} eDRX-AllowedInactive-r17 ENUMERATED {true} intraFreqReselectionRedCap-r17 ENUMERATED {allowed, notAllowed} cellBarredNTN-r17 ENUMERATED {barred, notBarred} nonCriticalExtension SEQUENCE {} }

The FeaturePriority IE is used to indicate the priority for each feature, such as RedCap, Slicing, SDT and MSG3-Repetitions for Coverage Enhancements. These priorities are used to determine which FeatureCombinationPreambles the UE shall use when a feature maps to more than one FeatureCombinationPreambles and a lower value means a higher priority.

FeatureCombination-r17 ::= SEQUENCE { redCap-r17 ENUMERATED {true} OPTIONAL, -- Need R smallData-r17 ENUMERATED {true} OPTIONAL, -- Need R nsag-r17 NSAG-List-r17 OPTIONAL, -- Need R msg3-Repetitions-r17 ENUMERATED {true} OPTIONAL, -- Need R spare4 ENUMERATED {true} OPTIONAL, -- Need R spare3 ENUMERATED {true} OPTIONAL, -- Need R spare2 ENUMERATED {true} OPTIONAL, -- Need R spare1 ENUMERATED {true} OPTIONAL -- Need R }

According to the current mechanism, when multiple features are to be indicated while no exact matching RACH resource is available for the feature combination, the UE will select one of the available combinations with the highest priority. For instance, the intended combination is RedCap+Slicing+MSG3 repetition, while only RedCap+Slicing and Slicing+MSG3 are available. If the RedCap is configured with the highest priority, the UE will select the resource for the RedCap+Slicing combination to perform RACH. In this case, the intended feature combination is not the same as the used feature combination, and the network shall be notified of the RACH partitioning-related information for further optimization.

Proposal 1: Introduce the RACH partitioning-related information in the RACH report to enable optimization of feature combination and/or feature priority.

- As to the essential information to be introduced, it’s straightforward to include the intended feature combination in the RACH report. The network can analyze the number of each intended feature combination to optimize RACH resource allocation, e.g., allocate the RACH in order of intended feature combinations. In addition, if one feature is triggered multiple times but with a low priority, the network can also reconfigure it with a higher priority. Besides, the gNB may switch the RACH resource allocation upon resource status changes, thus the used feature combination shall be notified to the network as well.

Proposal 2: For the content of the RACH report enhancement for RACH partitioning, the following information can be considered:

- Intended feature combination;

- Priority of each intended feature;

- Used feature combination.

Conclusions

Proposal 1: Introduce the RACH partitioning-related information in the RACH report to enable optimization of feature combination and/or feature priority.

Proposal 2: For the content of the RACH report enhancement for RACH partitioning, the following information can be considered:

- Intended feature combination;

- Priority of each intended feature;

- Used feature combination.

Reference