3GPP TSG-RAN WG2 Meeting #119bis electronic	                     R2-2209626

Online, Oct, 2022

Agenda Item:	8.4.2.4

Source: 	OPPO

Title:  	Discussion on measurement related issues of L1/L2 mobility

Document for:	Discussion and Decision

Introduction

In RAN#94 meeting, WID [1] has been agreed to further study mobility enhancement, and one of the objectives is to study L1/L2 based mobility, which aiming to reduce the HO latency as well as interruption. During last RAN2 meeting, the following measurement related agreements have been achieved：

Measurement delay can/may be considered in this work

Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS)

In this contribution, we further discuss the measurement related issues for L1/L2 mobility.

Discussion

Measurement configuration for candidate cell monitoring

L1/L2 mobility supports dynamic switching among multiple pre-configured candidate cells. Similar as legacy handover preparation, L1/L2 HO decision relies on UE measurement of the candidate cells. In RAN2#119e meeting, it has been agreed that L1/L2 mobility relies on L1 measurements for target cell selection.

R17 FeMIMO introduced inter-cell measurement mechanism in CSI framework to support L1-RSRP measurement and reporting on SSBs associated with a PCI different from the serving cell PCI. To support L1/L2 mobility, R17 inter-cell L1-RSRP measurement and reporting frame can be used as a baseline for candidate cell monitoring and evaluation.

R17 CSI framework supports inter-cell L1-RSRP measurement.

L1/L2 mobility relies on R17 CSI framework to support L1-RSRP measurement and reporting for L1/L2 candidate cell/beam monitoring.

In R17 CSI framework, the CSI measurement and report configuration including SSB resources associated with PCIs different from the PCI of a serving cell is provided by serving gNB. And the configuration is unchanged upon TCI state switching. For R18 L1/L2 mobility, there are two ways to provide the candidate cell related measurement configuration

- Alt1: common measurement configuration

In this option, UE is provided with L1 measurement configuration for L1/L2 mobility once the candidate configuration is received. For example, the beams of candidate cells can be associated to the CSI resources of additional PCIs in CSI measurement configuration. And the L1 measurement configuration is common for all configured candidate cells. Upon cell switching, UE keeps the L1 measurement configuration and continues monitoring candidate cells.

This alternative is similar as R17 inter-cell L1-RSRP measurement, CSI configuration is kept when UE is indicated with a TCI state that associate with an SSB of a PCI different from the serving cell PCI. And alt 1 is advantageous from the perspective of signalling overhead reduction, since the measurement configuration may be redundant for each candidate cells.

- Alt2: candidate cell specific configuration

Another alternative is to follow the measurement configuration approach in R16 conditional handover, i.e., each candidate cell is associated with a L3 measurement config. For L1/L2 mobility, each candidate cell configuration includes a CSI measurement/report config. UE apply CSI measurement configuration of each candidate cell configuration for cell monitoring and evaluation.

RAN2 study the measurement configuration granularity of L1/L2 candidate cell monitoring.

Furthermore, as defined in TS 38.133, to support L1-RSRP for cell with different PCI from serving cell, the SSB of the cell with different PCI from serving cell has the same SCS, sfn-SSB-Offset and center frequency as the SSB of the serving cell. That is, inter-cell L1-RSRP measurement only support intra-frequency measurement. For inter-frequency L1/L2 mobility, L1-based measurement enhancement to support inter-frequency is required.

R17 inter-cell L1-RSRP measurement is only applicable for intra-frequency scenario.

Send LS to RAN1/4 on the feasibility of inter-frequency inter-cell L1-RSRP measurement.

Trigger of measurement reporting

NW-initiated:

Legacy L1 measurement reporting, i.e. CSI reporting, is configured and triggered by NW. NW can configure a periodic or semi-persistent report sent on PUCCH, or configure a semi-persistent or aperiodic report sent on PUSCH. For L1/L2 mobility candidate cell evaluation/measurement reporting, one can follow the existing CSI reporting way, i.e. rely on NW to configure the reporting periodicity.

NW-initiated measurement reporting can be reused as baseline for candidate cell evaluation for L1/L2 mobility.

UE-initiated

Considering the purpose of L1/L2 mobility measurement reporting is to assist NW for HO decision, NW does not need constant measurements of the candidate cells, which causes large signalling overheads.

Legacy L3-based handover utilizes event-based measurement reporting triggering, UE reports measurement results once the events are fulfilled. Similar way can be applied for L1/L2 mobility measurement reporting for signalling overheads reduction. That is, once the L1/L2 mobility candidate cell configuration is provided, UE keeps monitoring the configured candidate cells, and the measurement result is reported when L1/L2 mobility measurement event is fulfilled.

UE-initiated measurement reporting helps on signalling overheads reduction.

UE-initiated L1 measurement reporting can be also considered for L1/L2 mobility.

Robustness improvement of measurement reporting

In traditional handover, L3 measurements is a key criterion for target cell selection, UE evaluates the neighbour cells according to the measurement configuration provided by NW. To derive the cell quality, UE measures multiple beams of a cell and averages the consolidated measurements results. Besides, UE applies layer 3 filtering for the cell measurement results before using them for the evaluation of reporting criteria to avoid ping-pang handover.

Filtering takes place at L3 to derive cell quality from multiple beams.

Existing L1 measurement focus on beam level result, UE reports beam level measurements to NW through CSI report. The beam measurement result may be less reliable for a cell switching decision by comparing to L3 cell measurements. If L1-based measurement is used for L1/L2 candidate cell monitoring, how to improve the robustness of L1 measurement should be further studied.

- One straightforward way is to derive a L1-based cell quality for target cell selection. Considering the purpose of L1/L2 based mobility is cell switching, a cell level quality is more convincing for a cell switch decision. And from UE movement perspective, it may be instable and unreliable to take the beam level measurement only into consideration for handover decision.

- Measurement event mentioned in section 2.2 not only helps on the frequently measurement reporting issue, but also benefit for improving the robustness of reported measurement result. For example, UE reports the candidate cell’s measurement to NW if at least n SSBs with SS-RSRP above the configured threshold.

- Another way is to introduce L1-based time to trigger to improve the robustness of L1 measurement.

Enhancements of L1 measurement to improve robustness and reliability should be considered for L1/L2 mobility.

Candidate cell preparation

L1/L2 based mobility relies on a set of pre-configured candidate cells, one FFS from last meeting is whether to use L3 measurement for candidate cell preparation. From our understanding, the candidate cell preparation is similar as CHO/CPAC, candidate cell is selected according to L3 measurement and reporting.

L3 measurement is applied for candidate cell preparation for L1/L2 based mobility.

Conclusion

Based on the discussion in section 2 we have following observations:

Observation 1	R17 CSI framework supports inter-cell L1-RSRP measurement.

Observation 2	R17 inter-cell L1-RSRP measurement is only applicable for intra-frequency scenario.

Observation 3	UE-initiated measurement reporting helps on signalling overheads reduction.

Observation 4	Filtering takes place at L3 to derive cell quality from multiple beams.

Based on the discussion above, we have the following proposals:

Proposal 1	L1/L2 mobility relies on R17 CSI framework to support L1-RSRP measurement and reporting for L1/L2 candidate cell/beam monitoring.

Proposal 2	RAN2 study the measurement configuration granularity of L1/L2 candidate cell monitoring.

Proposal 3	Send LS to RAN1/4 on the feasibility of inter-frequency inter-cell L1-RSRP measurement.

Proposal 4	NW-initiated measurement reporting can be reused as baseline for candidate cell evaluation for L1/L2 mobility.

Proposal 5	UE-initiated L1 measurement reporting can be also considered for L1/L2 mobility.

Proposal 6	Enhancements of L1 measurement to improve robustness and reliability should be considered for L1/L2 mobility.

Proposal 7	L3 measurement is applied for candidate cell preparation for L1/L2 based mobility.

Reference

- RP-222332  Further NR Mobility Enhancements    MediaTek Inc