3GPP TSG-RAN WG2 Meeting #119bis                                                 	R2-2209720

Electronic, 10th - 19th October, 2022

Source:	CATT

Title:	Consideration on General Aspects of AI/ML for NR Air-interface

Agenda Item:	8.16.2

Document for:	Discussion and Decision

Introduction

In RAN#94 e-meeting, The SID of AI/ML for NR Air Interface [1] was approved. In the WID, it is mentioned that the RAN2 only starts the work (e.g. consider the Protocol aspects) after there is sufficient progress on the use case study in RAN1:

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

Since RAN1 has already discussed the AI/ML item for 2 group meetings and some progress is made, in this document, we share our views about the AI/ML for NR air-interface.

Discussion

  - Work split between RAN 1/2/3

As per the SID, the study on AI/ML for air-interface is based on the current RAN architecture and new interfaces shall not be introduced. Therefore there is no RAN3 work expected during SI phase, maybe some information exchange based on the existing inter-node interface, and it can be discussed in normative phase in the future.

Observation 1: There is no RAN3 work expected during SI phase.

RAN1 has already discussed the AI/ML for two group meetings, and is responsible for the (sub) use cases evaluations/selection, general AI/ML framework (LCM, inference operation, data sets handling), various levels of collaboration between UE and gNB per use case, common notation and terminology, and RAN1 related spec impact;

It is the SID requirement to finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98. RAN2 needs to start the work after there is sufficient progress on the use case study in RAN1, and is responsible for the general framework or for specific (sub) use cases selected by RAN1, identify protocol impact and complexity from RAN2 point of view (on capability, LCM, data, UE and gNB collaboration).

Proposal 1: RAN2 may consider both general AI/ML aspects, and the protocol impact and complexity for the potential (sub) use cases.

  - Terminology

In RAN1#109 and RAN1#110 meetings, RAN1 agreed to include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion in the TR38.843 (The highlight definitions are added/modified in RAN1#110):

Table 1 Terminology until RAN1#110

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model by learning the input/output relationship in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing do not assume subsequent tuning of the model. |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |
| On-UE training | Online/offline training at the UE |
| On-network training | Online/offline training at the network |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| Model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Model deployment | Delivery of a fully developed and tested model runtime image to a target UE/gNB where inference is to be performed. |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple model exchanges, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online (field) data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Model update | Retraining or fine tuning of an AI/ML model, via online/offline training, to improve the model inference performance. |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data e.g., clustering is a common example of this. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |

Now the main framework and the steps of Life Cycle Management are not finally decided, more Terminology may be introduced in the future RAN1 meeting.

Since most of the terminologies are closely relative to the whole procedure of performing the AI/ML operation and should be shared between RAN1 and RAN2, and could also be used for RAN2 AI/ML air interface SI discussion.

Proposal 2: The terminologies defined by RAN1 should also be used for RAN2 AI/ML air-interface discussion.

  - Functional framework

It is suggest in the SID that for the study on AI/ML for air-interface, the basic framework and principles agreed for RAN3 FS_NR_ENDC_data_collect, as captured in section 4 of TR 37.817 [2], should be taken into consideration for possible applicability [1].

If an AI/ML model is only known and used by the network, it should follow the RAN3 framework in general. Since the training/inference may involve the UE and/or the network, the framework of AI/ML application in air interface should embody how AI/ML model is trained, deployed and interactive with other modules for wireless communication. Thus Model management can be useful and should be considered as one of the building blocks besides the existing modules defined in the RAN3 framework. The framework shall at least include function blocks for: data collection (maybe for AI/ML model training and inference, respectively), model training, model management, AI/ML model and actor. Figure 1 illustrates how these function blocks are interactive with each other:

Figure 1 Functional framework of AI/ML-based approach

Compared to the framework in TR37.817 [2], model management becomes much more complicated and should be considered as a module in the framework.

Observation 2: The AI/ML framework in this SID should include the same sets of moduals/functionanlities as the framework captured in TR37.817, taking into account the different roles of the UE and the network in these moduals/functionanlities, as well as necessary collaborations, signaling.

Proposal 3: Compared to the framework in TR37.817, model management is an important module to be taken into account in the general AI/ML framework.

  - Collaboration between UE and Network, with our without model transfer

As required by the SID, several collaboration levels between UE and gNB should be identified. According to the interaction degree between UE and gNB from view of signaling, three collaboration levels were agreed as Level x, Level y and Level z in RAN1#109 meeting [3].

Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels 1. Level x: No collaboration 2. Level y: Signaling-based collaboration without model transfer 3. Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary

It can be generally assumed in RAN2 that different collaboration levels require different signaling and procedures that involve information/signaling exchange between the UE and the gNB. Since collaboration level x have no signaling-based collaboration between UE and network, only collaboration level y and z can have RAN2 impact. The specific use case may only apply one of the collaboration levels, or apply both collaboration level y and z for different sub use cases or solutions.

Based on the current classification of collaboration levels by RAN1, the main specification impact from RAN2 perspective may be focus on the partial/full model transfer:

Figure 2 Classification of levels from RAN2 impact perspective

If the model needs to be transferred within 3GPP, then it is RAN2’s job to discuss the detailed solutions, e.g. whether and how the model transfers in a CP/UP format.

Proposal 4: If the AI/ML model needs to be transferred within 3GPP, it is RAN2’s job to discuss the detailed solutions, e.g. whether and how the model transfers in a CP/UP format.

Some issues may also be needed to assist the final decision of the collaboration levels by RAN1 per use case:

1) The requirements of the model and input/output data training/inference, e.g., data size, how frequent it is transferred, reliability, latency, etc;

2) Whether SA needs to be involved.

Since there is no more discussion in RAN1#110 meeting on how to define Level y/z boundary (e.g. whether the “without model transfer” means ‘no model delivery at all’ or ‘model delivery outside 3GPP’), as well as whether finer levels can be introduced, the analysis about RAN2 impact of different collaboration levels should be deferred. RAN2 can consider such categorization of different UE/gNB collaboration levels, also take into account future extension of the definitions from RAN1, and discuss per each use cases.

Proposal 5: RAN2 impact discussion involving collaboration levels should wait a clearer definition of the collaboration levels made by RAN1.

  - Life Cycle Management (LCM)

In last RAN1 meeting, RAN1 agreed an initial procedure of Life Cycle Management (LCM) as follows, to control and manage the whole life of the AI/ML based function enhancement. But the steps and the necessity of some components are not clearly defined now. In our understanding, the management of data and AI/ML model can be generally mapped to the LCM steps of ‘Data collection’ and ‘Model deployment /selection/activation/deactivation/switching/fallback / monitoring/update’.

Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses.

In this procedure, some steps are tentative and some steps may have not been well defined. Therefore generally speaking, RAN2 could first discuss which steps can have some discussion in RAN2 for the specification impact aspect, and the detailed content can be further discussed after more RAN1 progress made.

We analyses the potential RAN2 work for the LCM steps in the table below:

Table 2 Potential RAN2’s discussion point based on current LCM

| LCM components | Potential RAN2’s further discussion point |
| --- | --- |
| Data collection | RAN2 assumes that there are cases when gNB or UE needs to provide assistant information for data collection to each other. The exact protocol impact can be further discussed for more progresses are made for the specific use cases. |
| Model training | Depending how/where training is done. From view of simplicity, RAN2 could use offline training as the starting point. |
| Model registration | RAN2 assumes model registration is useful if one-sided model at UE is used, and can further discuss protocol impact based on RAN1’s further progress. |
| Model deployment/ configuration/ transfer | FFS in RAN1. Depending how/where model is located/used. RAN2 initially discuss the requirements/ways to transfer AI model. |
| Model inference operation | Depending how/where inference is done, can discuss in a later stage for specific use cases. |
| Model selection, activation, deactivation, switching, and fallback operation | Likely involve coordination/signaling exchange. RAN2 assumes mode select .etc. involves signaling exchange. Details can be further discussed based on RAN1’s progress. |
| Model monitoring/ update | Likely involve coordination/signaling exchange. Can discuss in a later stage when other aspects are clearer |
| UE capability | Can discuss in a later stage when other aspects are clearer. |

Therefore, from the analysis we can see that all steps of LCM may more or less have some impact on the air-interface.

Proposal 6: RAN2 impact could be initially considered based on the Life Cycle Management steps defined by RAN1.

The terminology of One-sided (AI/ML) model and Two-sided (AI/ML) model are defined to indicate whether a joint inference is performed by both the network and the UE, e.g. for CSI compression, a two-sided model is used. And besides the inference type, whether a joint training is performed by both sides is also discussed for specific use case. By deciding the model training/inference side(s), together with the collaboration levels the main structure and the assistance information transmitted between UE and gNB can be identified for different LCM steps for different sub-use cases.

Observation 3: The model training/inference side(s) may largely impact the RAN2 specification, together with the collaboration levels for different sub-use cases.

Proposal 7: Detailed RAN2 impact of assistance information transmitted between UE and gNB for different LCM steps could be considered after RAN1 deciding the model training/inference side(s) per sub-use case.

  - UE Capability Indication

In the last 2 group meetings, RAN1 had limited (initial discussion) on UE capability, e.g., as part of the studies for sub use cases of AI/ML for positioning accuracy enhancement, RAN1 will further discuss aspects including UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring).

Therefore, we think it is too early to decide the UE capability framework for AI/ML in RAN2, e.g., many aspects are not very clear so far:

  - Whether operations/functionalities related to data set, model, and inference require separate UE capabilities or not;

  - Whether different level of UE/gNB collaboration require separate UE capabilities or not;

  - What UE capabilities are needed per each (sub) use case.

However some general principle of UE capability could still apply (e.g., optional capability, network configures AI/ML based on UE’s capabilities, etc.)

Proposal 8: RAN2 further discuss whether UE capability framework for AI/ML is based on the following two-level assumption:

a) General AI/ML capability framework is not restricted any specific use cases;

b) UE capabilities that are needed for a specific use case can be discussed on a case by case basis.

Conclusion

According to the analysis in section 2, we propose:

For work split between RAN 1/2/3:

Observation 1: There is no RAN3 work expected during SI phase.

Proposal 1: RAN2 may consider both general AI/ML aspects, and the protocol impact and complexity for the potential (sub) use cases.

For terminology:

Proposal 2: The terminologies defined by RAN1 should also be used for RAN2 AI/ML air-interface discussion.

For functional framework:

Observation 2: The AI/ML framework in this SID should include the same sets of moduals/functionanlities as the framework captured in TR37.817, taking into account the different roles of the UE and the network in these moduals/functionanlities, as well as necessary collaborations, signaling.

Proposal 3: Compared to the framework in TR37.817, model management is an important module to be taken into account in the general AI/ML framework.

For collaboration between UE and Network, with our without model transfer:

Proposal 4: If the AI/ML model needs to be transferred within 3GPP, it is RAN2’s job to discuss the detailed solutions, e.g. whether and how the model transfers in a CP/UP format.

Proposal 5: RAN2 impact discussion involving collaboration levels should wait a clearer definition of the collaboration levels made by RAN1.

For Life Cycle Management (LCM):

Proposal 6: RAN2 impact could be initially considered based on the Life Cycle Management steps defined by RAN1.

Observation 3: The model training/inference side(s) may largely impact the RAN2 specification, together with the collaboration levels for different sub-use cases.

Proposal 7: Detailed RAN2 impact of assistance information transmitted between UE and gNB for different LCM steps could be considered after RAN1 deciding the model training/inference side(s) per sub-use case.

For UE Capability Indication:

Proposal 8: RAN2 further discuss whether UE capability framework for AI/ML is based on the following two-level assumption:

a) General AI/ML capability framework is not restricted any specific use cases;

b) UE capabilities that are needed for a specific use case can be discussed on a case by case basis.

Reference

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #96

- TR 37.817 Study on enhancement for Data Collection for NR and EN-DC h00

- Chairman’s Notes RAN1#109-e, RAN1#109-e