3GPP TSG-RAN WG2 Meeting #119bis-e	R2-2209739

# Electronic meeting, 10th - 19th Oct, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3502 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on TS 38.331 for SL DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In subclause 5.3.5.14, the procedure of UE receiving the sl-DRX-Config-GC-BC included in sl-ConfigDedicatedNR is missed. - In subclause 5.8.3.1, the condition of gNB supporting SL DRX for Tx UE reporting the sidelink DRX assistance information, sidelink DRX on/off indication, sidelink DRX configuration reject information is missed. - In subclause 5.8.3.1, the condition of gNB supporting SL DRX for Rx UE reporting the sidelink DRX configuration information, Destination Layer-2 ID and QoS profile(s) information associated with its interested services is missed. - In subclause 5.8.9.1.3, the Rx UE behaviour of accepting the SL DRX configuration or rejecting the SL DRX configuration need to be put in the same place. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 5.3.5.14, add the procedure of UE applying the related parameters upon receiving the sl-DRX-Config-GC-BC included in sl-ConfigDedicatedNR. - In subclause 5.8.3.1, add the the condition of gNB supporting SL DRX for Tx UE reporting the sidelink DRX assistance information, sidelink DRX on/off indication, sidelink DRX configuration reject information. - In subclause 5.8.3.1, add the condition of gNB supporting SL DRX for Rx UE reporting the sidelink DRX configuration information, Destination Layer-2 ID and QoS profile(s) information associated with its interested services. - In subclause 5.8.9.1.3, move the procedure that Rx UE accepts the SL DRX configuration to the procedure where UE rejects the SL DRX configuration. Impact analysis Impacted functionality: NR sidelink communication Inter-operability: For change 1, If the NW implements the CR but the UE does not, there is inter-operability issue since the UE will not apply the parameters in sl-DRX-Config-GC-BC, the UE could not work in SL DRX mode for GC/BC communication. If the UE implements the CR but the NW does not, there is inter-operability issue since the NW cannot allocate the SL grant for the enabled SL DRX. For change2 and change 3, If the NW implements the CR but the UE does not, there is inter-operability issue since the TX/RX UE will report the SL DRX related information to network that does not support SL DRX, a mistake on accepting the SidelinkUEinformationNR message in NW will be generated, the UE will not receive the desired configuration information from the network. If the UE implements the CR but the NW does not, there is no inter-operability issue in NW and UE since the network is just responsible to receive the information from SL UE and the information is correct based on UE implementation. For chang 4, If the NW implements the CR but the UE does not, there is no inter-operability issue since this is used in PC5 link. If the UE implements the CR but the NW does not, there is no inter-operability issue since this is used in PC5 link. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For change1, the network and UE will have not common understanding to the the parameters in sl-DRX-Config-GC-BC, the SL active time computed in UE and network will be different, it will lead to the data loss, the SL communication will be low qualified. For change 2 and change 3, Tx UE and Rx UE will report the SL DRX related information to a gNB not supporting SL DRX, a mistake on accepting the SidelinkUEinformationNR message in NW will be generated. For change 4, the current procedure will make UE determine reject the SL DRX configuration or accept the SL DRX configuration in two different steps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.14, 5.8.3.1, 5.8.9.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.3.5.14	Sidelink dedicated configuration

Upon initiating the procedure, the UE shall:

1>	if sl-FreqInfoToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each entry included in the received sl-FreqInfoToReleaseList that is part of the current UE configuration:

3>	release the related configurations from the stored NR sidelink communication configurations;

1>	if sl-FreqInfoToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	if configured to receive NR sidelink communication:

3>	use the resource pool(s) indicated by sl-RxPool for NR sidelink communication reception, as specified in 5.8.7;

2>	if configured to transmit NR sidelink communication:

3>	use the resource pool(s) indicated by sl-TxPoolSelectedNormal, sl-TxPoolScheduling or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.8.8;

2>	if configured to receive NR sidelink discovery:

3>	use the resource pool(s) indicated by sl-DiscRxPool or sl-RxPool for NR sidelink discovery reception, as specified in 5.8.13.2;

2>	if configured to transmit NR sidelink discovery:

3>	use the resource pool(s) indicated by sl-DiscTxPoolSelected, sl-DiscTxPoolScheduling, sl-TxPoolSelectedNormal, sl-TxPoolScheduling or sl-TxPoolExceptional for NR sidelink discovery transmission, as specified in 5.8.13.3;

2>	perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal, sl-TxPoolScheduling, sl-DiscTxPoolSelected, sl-DiscTxPoolScheduling or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.5.3;

2>	use the synchronization configuration parameters for NR sidelink communication on frequencies included in sl-FreqInfoToAddModList, as specified in 5.8.5;

1>	if sl-RadioBearerToReleaseList or sl-RLC-BearerToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform sidelink DRB release as specified in 5.8.9.1a.1;

1>	if sl-RadioBearerToAddModList or sl-RLC-BearerToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform sidelink DRB addition/modification as specified in 5.8.9.1a.2;

1>	if sl-ScheduledConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	configure the MAC entity parameters, which are to be used for NR sidelink communication, in accordance with the received sl-ScheduledConfig;

1>	if sl-UE-SelectedConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	configure the parameters, which are to be used for NR sidelink communication, in accordance with the received sl-UE-SelectedConfig;

1>	if sl-MeasConfigInfoToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each SL-DestinationIndex included in the received sl-MeasConfigInfoToReleaseList that is part of the current UE configuration:

3>	remove the entry with the matching SL-DestinationIndex from the stored NR sidelink measurement configuration information;

1>	if sl-MeasConfigInfoToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each sl-DestinationIndex included in the received sl-MeasConfigInfoToAddModList that is part of the current stored NR sidelink measurement configuration:

3>	reconfigure the entry according to the value received for this sl-DestinationIndex from the stored NR sidelink measurement configuration information;

2>	for each sl-DestinationIndex included in the received sl-MeasConfigInfoToAddModList that is not part of the current stored NR sidelink measurement configuration:

3>	add a new entry for this sl-DestinationIndex to the stored NR sidelink measurement configuration.

1>	if sl-DRX-ConfigUC-ToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each SL-DestinationIndex included in the received sl-DRX-ConfigUC-ToReleaseList that is part of the current UE configuration:

3>	remove the entry with the matching SL-DestinationIndex from the stored NR sidelink DRX configuration information;

1>	if sl-DRX-ConfigUC-ToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each sl-DestinationIndex included in the received sl-DRX-ConfigUC-ToAddModList that is part of the current stored NR sidelink DRX configuration:

3>	reconfigure the entry according to the value received for this sl-DestinationIndex from the stored NR sidelink DRX configuration information;

2>	for each sl-DestinationIndex included in the received sl-DRX-ConfigUC-ToAddModList that is not part of the current stored NR sidelink DRX configuration:

3>	add a new entry for this sl-DestinationIndex to the stored NR sidelink DRX configuration.

1>	if sl-DRX-Config-GC-BC is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2> configure the DRX related parameters, which are to be used for NR sidelink groupcast and broadcast communication and unicast/broadcast based communication of Direct Link Establishment Request, in accordance with the received sl-DRX-Config-GC-BC;

1>	if sl-RLC-ChannelToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform PC5 Relay RLC channel release as specified in 5.8.9.7.1;

1>	if sl-RLC-ChannelToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform PC5 Relay RLC channel addition/modification as specified in 5.8.9.7.2;

NEXT OF CHANGE

5.8.3	Sidelink UE information for NR sidelink communication

5.8.3.1	General

Figure 5.8.3.1-1: Sidelink UE information for NR sidelink communication

The purpose of this procedure is to inform the network that the UE:

-	is interested or no longer interested to receive or transmit NR sidelink communication,

-	is requesting assignment or release of transmission resource for NR sidelink communication,

-	is reporting QoS parameters and QoS profile(s) related to NR sidelink communication,

-	is reporting that a sidelink radio link failure or sidelink RRC reconfiguration failure has been detected,

-	is reporting the sidelink UE capability information of the associated peer UE for unicast communication,

-	is reporting the RLC mode information of the sidelink data radio bearer(s) received from the associated peer UE for unicast communication,

-	is reporting the accepted sidelink DRX configuration received from the associated peer UE for NR sidelink unicast reception when sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs,

-	is reporting the sidelink DRX assistance information received from the associated peer UE for NR sidelink unicast transmission, when the UE is configured with sl-ScheduledConfig and sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs,

-	is reporting, for NR sidelink groupcast transmission, the sidelink DRX on/off indication for the associated Destination Layer-2 ID, when UE is configured with sl-ScheduledConfig and sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs,

-	is reporting, for NR sidelink groupcast or broadcast reception, the Destination Layer-2 ID and QoS profile(s) associated with its interested services to which sidelink DRX is applied when sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs,

-	is reporting sidelink DRX configuration reject information from its associated peer UE for NR sidelink unicast transmission, when the UE is configured with sl-ScheduledConfig and sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs,

-	is reporting parameters related to U2N relay operation.

NEXT OF CHANGE

5.8.9.1.3	Reception of an RRCReconfigurationSidelink by the UE

The UE shall perform the following actions upon reception of the RRCReconfigurationSidelink

<skip>

1>	if the RRCReconfigurationSidelink message includes the sl-LatencyBoundIUC-Report:

2>	apply the configured sidelink IUC report latency bound;

1>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfigurationSidelink (i.e. sidelink RRC reconfiguration failure):

2>	continue using the configuration used prior to the reception of the RRCReconfigurationSidelink message;

2>	set the content of the RRCReconfigurationFailureSidelink message;

3>	submit the RRCReconfigurationFailureSidelink message to lower layers for transmission;

1>	else:

2> if the RRCReconfigurationSidelink message includes the sl-DRX-ConfigUC-PC5, and

3> if the UE rejects the sl-DRX-ConfigUC-PC5:

4> include the sl-DRX-ConfigReject in the RRCReconfigurationCompleteSidelink message;

4> consider no sidelink DRX to be applied for the corresponding sidelink unicast communication;

3> else if the UE accepts the sl-DRX-ConfigUC-PC5:

4> configure lower layers to perform sidelink DRX operation according to sl-DRX-ConfigUC-PC5 for the associated destination as defined in TS 38.321 [3];

2>	set the content of the RRCReconfigurationCompleteSidelink message;

3>	submit the RRCReconfigurationCompleteSidelink message to lower layers for transmission;

NOTE 1:	When the same logical channel is configured with different RLC mode by another UE, the UE handles the case as sidelink RRC reconfiguration failure.

NOTE 2: It is up to the UE whether or not to indicate the rejection to the peer UE for a received sidelink DRX configuration

END OF CHANGE