3GPP TSG-RAN WG2 Meeting #119bis-e                        R2-2209808

Online, October 10-19, 2022

Agenda item:	8.13.3

Source:	Samsung

Title:	Inter-RAT signaling based logged MDT override protection

Document for:	Discussion & Decision

# Introduction

We have an objective in Rel-18 SON/MDT [1] to handle Inter-RAT scenarios for signaling based logged MDT override protection as below.

Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

In RAN2#119-e, we discussed various aspects of the above objective and the following agreements are made [2].

Agreement:

1	RAN2 confirms the valid scenario for Rel-18 inter-RAT scenario for signalling based logged MDT override protection is set by the WID:

a.	Logged MDT is configured in E-UTRAN, the UE reselects to NR.

2	Rel-17 mechanism for signalling based logged MDT override protection in intra-NR scenario is the baseline for Rel-18 inter-RAT scenario.

We discuss further on the objective based on the agreements, in this paper.

# Discussion

As we agreed to consider Rel-17 mechanism as the baseline for Rel-18 inter-RAT scenario, eNB can inform the UE with Logged MDT type indication by extending LTE LoggedMeasurementConfiguration

Proposal 1: Extend LTE LoggedMeasurementConfiguration with Logged MDT type indication.

As in NR, we can also consider UE assistance support to assist the network in preventing management based logged MDT overwriting signaling based logged MDT for inter-RAT case. A UE which has been configured with signaling based logged MDT by eNB and has moved to NR can indicate to gNB whether signaling based MDT has been configured in RRC messages like RRCSetupComplete,RRCReconfigurationComplete etc.

Proposal 2: UE informs gNB whether signaling based MDT is configured when it is configured by E-UTRA.

Since WI objective (also confirmed by RAN2) doesn’t require any RAT based prioritization, we think that it would be possible to reuse the existing NR signaling introduced in Rel 17 for the UE to inform the network whether it is configured for signaling based MDT in LTE. gNB can prevent overwrite of signalling based MDT irrespective of whether it was configured by LTE or NR.

Proposal 3: R17 NR signaling can be reused by the UE to inform gNB whether signaling based MDT is configured even when it is configured by E-UTRA.

# Conclusion

In the previous sections, we have discussed Inter-RAT signalling based logged MDT override protection and have made following proposals.

Proposal 1: Extend LTE LoggedMeasurementConfiguration with Logged MDT type indication.

Proposal 2: UE informs gNB whether signaling based MDT is configured when it is configured by E-UTRA.

Proposal 3: R17 NR signaling can be reused by the UE to inform gNB whether signaling based MDT is configured even when it is configured by E-UTRA.

# References

- RP-221825	Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity).

- RAN2#119-e meeting report.