3GPP TSG-RAN WG2 Meeting #119bis electronic	R2-2209816

E-Meeting, 10th – 19th October, 2022

Source:	vivo

Title:	Discussion on NR SL communication transmission using exception pool during D2I path switch

Agenda Item:	6.7.2.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss whether/how to capture the agreement below [1], including whether this is related to an inexistent case.

RAN2#119e Agreements: Proposal 29 (modified): RAN2 confirms during path switch (T420 is running), UE can use exceptional pool for sidelink communication in mode 1 (same as Rel-16 V2X).

- Discussion

During the post email for Relay 38.331 CR [2], the RRC rapporteur expressed their concern on whether and how to capture the above agreement, as shown below. Thus, the issue is postponed to give companies enough time for further consideration

Our understanding is also that, when performing D2I path switch with T420 running, the UE can only be configured with Mode-2 on the target path (i.e. indirect path) by the serving gNB, irrespective of what SL mode was configured previously on the source path (i.e. direct path). Therefore, the above agreement is somewhat misleading to confirm it is for mode 1 operation as in the legacy NR sidelink communication.

Observation 1	When performing D2I path switch with T420 running, the UE can only be configured with Mode-2 on the target path (i.e. indirect path), irrespective of what SL mode was configured previously on the source path (i.e. direct path).

Moreover, the use of exceptional pool for sidelink communication when T420 is running, strictly speaking, is executed during the transition period before the UE can use the mode 2 normal pools configured for the target path. For this short transition period, the UE is most likely having no available sensing result in the mode-2 TX pool(s) on the indirect link. According to current RRC specification, the condition to use exceptional pool for sidelink communication transmission can be automatically fulfilled by the condition of not having available sensing results for the UE.

Observation 2	As per the current procedure, the UE will autonomously enter into the case that it has no available sensing result in the mode-2 TX pool(s) on indirect link, and thus enter the loop for transmission using exceptional pool. No extra Spec impact is needed.

Given the above observations, we think the previous agreement should be revised into the correct form and thus have the following proposal to close this issue due to the earlier agreement.

Proposal 1	RAN2 to revise the previous agreement as “RAN2 confirms during path switch (T420 is running), UE can use exceptional pool for sidelink communication in mode 1 (same as Rel-16 V2X).”, and confirm that this agreement can already be realized via existing NR SL communication procedure with no extra Spec impact.

- Conclusion

This contribution discussed the RAN2#119e agreement on NR SL communication transmission using exception pool during D2I path switch. The contribution concludes with:

Observation 1	When performing D2I path switch with T420 running, the UE can only be configured with Mode-2 on the target path (i.e. indirect path), irrespective of what SL mode was configured previously on the source path (i.e. direct path).

Observation 2	As per the current procedure, the UE will autonomously enter into the case that it has no available sensing result in the mode-2 TX pool(s) on indirect link, and thus enter the loop for transmission using exceptional pool. No extra Spec impact is needed.

Proposal 1	RAN2 to revise the previous agreement as “RAN2 confirms during path switch (T420 is running), UE can use exceptional pool for sidelink communication in mode 1 (same as Rel-16 V2X).”, and confirm that this agreement can already be realized via existing NR SL communication procedure with no extra Spec impact.

- References

- RAN2#119e Chairman Notes.

- [Post119-e][404][Relay] Relay 38.331 CR (Huawei).