3GPP TSG-RAN WG2 Meeting #119bis electronic	R2-2209849

Online, October 10-19, 2022

Agenda Item:		6.10.3

Source:				ASUSTeK

Title:		Discussion on reported value for event-triggered TA report

Document for: 	Discussion and Decision

- Introduction

In RAN2#118 and #119 meetings, the TA value included in TAR MAC CE has been discussed. It has been clarified that the TAR MAC CE is generated based on the latest available estimate of the UE’s TA value. In this contribution, we further discuss the reported TA value for the case of event-triggered TA report.

- Discussion

In NTN, a TA report could be triggered by following events:

- Upon indication from upper layers (i.e. ta-Report is enabled for RA due to RRC connection establishment, RRC connection reestablishment or RRC connection resume);

- Upon receiving configuration of offsetThresholdTA, if the UE has not previously reported TA value to current serving cell;

- When the variation between current TA and the last reported TA is equal to or larger than offsetThresholdTA, if configured.

If the TA report has been triggered, the UE would generate a TAR MAC CE if there are available UL resources to transmit the TAR MAC CE. Otherwise, the UE would determine whether to trigger a SR and wait for available UL resources for TA report.

In previous meetings, the case was discussed that more than one TA report could be triggered before the TAR MAC CE is transmitted. As mentioned in [1], this situation has been addressed in TS 38.321 V17.1.0 [2] as:“a MAC PDU shall contain at most one Timing Advance Report MAC CE, even when multiple events have triggered a Timing Advance report.” And following clarification (with underline) [3] was added in last meeting to specify that the latest TA should be reported even multiple events for TA report have been triggered:

### 5.4.8 Timing Advance Reporting […] A MAC PDU shall contain at most one Timing Advance Report MAC CE, even when multiple events have triggered a Timing Advance report. The Timing Advance Report MAC CE shall be generated based on the latest available estimate of the UE’s Timing Advance value prior to the MAC PDU assembly.

On the other hand, since timingAdvanceSR in TAR-Config could be configured as not enabled, when a TA report is triggered, the UE may not transmit a SR even when there is no UL resources for the TA report. And thus the time between a TA report is triggered and the TA report is actually generated may be long. For the case that the TA report is triggered due to the variation between current TA and the last reported TA becoming larger than offsetThresholdTA, it may result in that the TA value to be reported doesn’t fulfil the condition of triggering the TA report.

For example, as shown in Figure 1, at timing t1, the UE generates and transmits a first TA report with the first TA value. At timing t2, the variation between current TA (i.e. the second TA) at the time and the last reported TA (i.e. the first TA) becomes larger than offsetThresholdTA, the UE thus triggers a second TA report. It is assumed that the UE does not have UL resources for TAR MAC CE at timing t2, and the UE does not transmit a SR since timingAdvanceSR is not enabled. The UE then has available UL resources for TAR MAC CE at timing t3. However, the variation between the latest TA (i.e. the third TA) and the last reported TA (i.e. the first TA) becomes smaller than offsetThresholdTA. It is not clear whether the UE should transmit a TA report with the third TA.

Figure 1: An example for event-triggered TA reporting

Since the intention of the TA report triggered by offsetThresholdTA is to inform the network when the variation of TA becomes large, it may be less useful to report a TA value which doesn’t fulfil the condition of triggering the TA report. In this point of view, it may make sense to re-evaluate the offsetThresholdTA when the TAR MAC CE is to be generated if corresponding TA report is triggered by offsetThresholdTA. And if the TA value to be reported doesn’t fulfil the condition of the trigger, the TA report is cancelled. On the other hand, it may be simpler to have a unified behaviour regardless of whether the variation between the TA to be reported and the last reported TA is larger or smaller than offsetThresholdTA once the TA report has been triggered. It is proposed to discuss which option is preferred.

Proposal 1: For the TA reporting based on offsetThresholdTA, discuss whether to cancel the TA report if the variation between the latest TA value prior to the MAC PDU assembly and the last reported TA becomes below offsetThresholdTA.

For option 1, if RAN2 agrees to cancel the TA report if the variation between the latest TA value prior to the MAC PDU assembly and the last reported TA becomes below offsetThresholdTA, the following text proposal should be adopted:

### 5.4.8 Timing Advance Reporting […] A MAC PDU shall contain at most one Timing Advance Report MAC CE, even when multiple events have triggered a Timing Advance report. The Timing Advance Report MAC CE shall be generated based on the latest available estimate of the UE’s Timing Advance value prior to the MAC PDU assembly. The Timing Advance Report MAC CE shall be cancelled if the latest available estimate of the UE’s Timing Advance value prior to the MAC PDU assembly doesn’t fulfill the condition of triggering the Timing Advance report.

For option 2, if RAN2 agrees to generate and transmit a TAR MAC CE regardless of whether the variation between the TA to be reported and the last reported TA is larger or smaller than offsetThresholdTA once the TA report has been triggered, the following text proposal should be adopted:

### 5.4.8 Timing Advance Reporting […] A MAC PDU shall contain at most one Timing Advance Report MAC CE, even when multiple events have triggered a Timing Advance report. The Timing Advance Report MAC CE shall be generated based on the latest available estimate of the UE’s Timing Advance value prior to the MAC PDU assembly, even when the latest available estimate of the UE’s Timing Advance value prior to the MAC PDU assembly doesn’t fulfill the condition of triggering the Timing Advance report.

Proposal 2: Adopt one of the above text proposals to TS 38.321.

- Conclusion

In this contribution, we discuss the reported TA value for the case of event-triggered TA report and have following proposals:

Proposal 1: For the TA reporting based on offsetThresholdTA, discuss whether to cancel the TA report if the variation between the latest TA value prior to the MAC PDU assembly and the last reported TA becomes below offsetThresholdTA.

Proposal 2: Adopt one of the above text proposals to TS 38.321.

- Reference

- R2-2208275, “Clarifications to the Timing Advance reporting procedure.”

- TS 38.321 V17.1.0 NR MAC specification.

- R2-2209044, “Corrections to Release-17 Non-Terrestrial Networks (NTN) for TS 38.321.”