3GPP TSG-RAN WG2 Meeting #119bis-e	R2-2209865

E-meeting, 10 – 19 October 2022

Title: 	SHR and SPCR

Source: 	Huawei, HiSilicon

Agenda item:	8.13.4

Document Type:	discussion

Introduction

During the RAN2 discussion, we agreed to wait for the progress on RAN3 for the SPCR (Successful PSCell Change Report) and SHR for inter-RAT.

=>	RAN2 should wait for RAN3 progress for one meeting (e.g. the scenarios and requirements) for the following features:

-	(2) Successful Pscell change report

-	(3) SHR for inter-RAT

There was an LS from RAN3 to inform RAN2 of the agreed scenarios of SHR for intra-system inter-RAT and SPCR for NR-DC. The detailed information is listed in the following:

- SHR (Successful HO Report) for intra-system inter-RAT, HO from NR to LTE will be treated first

- SPCR (Successful PSCell Change Report) for NR-DC, including:

  - SN- and MN-initiated classic PSCell change / CPC

  - intra-SN classic PSCell change / CPC

  - classic Addition / CPA

  - HO with SN change are not prohibited, but possibly addressed once the basic solution for SPCR is known.

In this paper, for the agreed scenarios, we will give our considerations on more potential scenarios for intra-system inter-RAT SHR and the required information provided by the UE to support inter-RAT SHR and NR-DC SPCR.

Discussion

Intra-system inter-RAT SHR

In the LS [1] from the RAN3, it suggested to consider SHR for intra-system inter-RAT and first to treat the SHR from NR to LTE.

R17 intra-NR SHR scheme

As discussed in R17 SHR, the UE is configured with SHR triggering condition(s) via the handover command message from the source node. There are four SHR triggering condition. The timer T310 and T312 related triggering conditions are decided and configured by the source cell of the handover. For the timer T304 related triggering condition, it is decided by the target cell of the handover, delivered to the source cell from the target cell and finally configured to the UE by the source cell. There is also the DAPS related triggering condition configured by the source cell in case of DAPS HO. For inter-RAT HO, DAPS is not supported.

Observation 1: SHR triggering conditions are configured by the source and/or target cell of the HO.

When the UE successfully completes the Random Access procedure with the target cell, and at least one of the SHR triggering conditions have been met, the UE stores the successful handover information into the varSuccessHO-Report.

Observation 2: The UE stores the successful handover information in the variable upon successfully completing the Random Access procedure with the target cell of the handover if at least one of the SHR triggering conditions are met.

Case 1: HO from NR to LTE within NG-RAN

In the LS [1], it was preferred to firstly discuss this direction for SHR for intra-system inter-RAT. For this direction, the source cell belongs to the NR while the target one belongs to the LTE. As mentioned in the aforesaid section, the NR source node sends the MobilityFromNRCommand message to configure the UE with all the SHR triggering conditions.

Proposal 1: For the SHR from NR to LTE, introduce the T310 and T312 related triggering condition in the MobilityFromNRCommand message.

For the intra-system inter-RAT HO from NR to LTE, the target ng-eNB decides the T304 related triggering condition and delivers it to the source NR node. Technically speaking, all the configuration information of the target ng-eNB is included in the Target NG-RAN node To Source NG-RAN node Transparent Container as showed in the below table. The detailed information in the container refers to the HandoverCommand defined in RAN2, which contains the RRCConnectionReconfiguration message in LTE.

| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

–	HandoverCommand

This message is used to transfer the handover command generated by the target eNB.

Direction: target eNB to source eNB/ source RAN

HandoverCommand message

-- ASN1START

HandoverCommand ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverCommand-r8					HandoverCommand-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverCommand-r8-IEs ::=			SEQUENCE {

handoverCommandMessage				OCTET STRING (CONTAINING DL-DCCH-Message),

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

HandoverCommand field descriptions

handoverCommandMessage Contains the entire DL-DCCH-Message including the RRCConnectionReconfiguration message used to perform handover within E-UTRAN or handover to E-UTRAN, generated (entirely) by the target eNB.

In R17 SHR, it is only applied to the mobility within NR and only introduces the SHR triggering condition in the NR specification. Therefore, if we want to support the SHR for the handover from NR to LTE, one potential solution is to include the T310/T312/T304 related triggering condition in the handover command message from NR to LTE. Nevertheless, based on the above HandoverCommand message from LTE to NR, it contains the RRCConnectionReconfiguration message. There is no T304 related triggering condition in the RRCConnectionReconfiguration message. If T304 related triggering condition is supported for HO from NR to LTE, it implies that the related enhancement should be captured in LTE RAN2 specification.

However, the reason to firstly treat this case 1 is to reduce the impact on LTE as much as possible. From less impact on LTE point of view, RAN2 can discuss whether to introduce the T304 related SHR triggering condition for the HO from NR to LTE.

Observation 3: For the SHR from NR to LTE, the introduction of the T304 related triggering condition in the MobilityFromNRCommand message will have impact on the RRCConnectionReconfiguration message.

Proposal 2: For the SHR from NR to LTE, whether to introduce the T304 related triggering condition in the MobilityFromNRCommand message is FFS.

In addition, as described above, the UE performs the determination of the successful handover information upon successful random access procedure with the target. It is worth noting that for the handover case from NR to LTE, the UE completes the handover in the LTE target cell. If the SHR triggering condition(s) is triggered, the UE should set the handover information into the intra-system inter-RAT SHR related variable. Obviously, there is no such variable defined in LTE. To support the SHR function for the HO from NR to LTE, it is desirable for RAN2 to define the UE actions in LTE to store the successful handover information for underlying failure during the handover procedure.

For the successful information, we prefer to take the information in the R17 SHR as baseline.

SuccessHO-Report-r17 ::=                 SEQUENCE {

sourceCellInfo-r17                       SEQUENCE {

sourcePCellId-r17                        CGI-Info-Logging-r16,

sourceCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL,

rlf-InSourceDAPS-r17                     ENUMERATED {true}                               OPTIONAL

},

targetCellInfo-r17                       SEQUENCE {

targetPCellId-r17                        CGI-Info-Logging-r16,

targetCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL

},

measResultNeighCells-r17                 SEQUENCE {

measResultListNR-r17                     MeasResultList2NR-r16                           OPTIONAL,

measResultListEUTRA-r17                  MeasResultList2EUTRA-r16                        OPTIONAL

}                                                                                            OPTIONAL,

locationInfo-r17                         LocationInfo-r16                                    OPTIONAL,

timeSinceCHO-Reconfig-r17                TimeSinceCHO-Reconfig-r17                           OPTIONAL,

shr-Cause-r17                            SHR-Cause-r17                                       OPTIONAL,

ra-InformationCommon-r17                 RA-InformationCommon-r16                            OPTIONAL,

upInterruptionTimeAtHO-r17               UPInterruptionTimeAtHO-r17                          OPTIONAL,

c-RNTI-r17                               RNTI-Value                                          OPTIONAL,

...

}

As listed above, there are source/target cell information, measurement results for source, target and neighbours, Shr-cause to indicate which triggering condition was met, C-RNTI, CHO and DAPS HO related time information in the SHR. For inter-RAT handover, both CHO and DAPS HO are not supported. Consequently, the CHO or DAPS HO related information is not needed for inter-RAT SHR. If we take the NR SHR as baseline, it is desirable for RAN2 to include the similar information in the inter-RAT SHR.

Proposal 3: RAN2 to include the following information in inter-RAT SHR from NR to LTE:

- Source NR cell info and target LTE cell info

- Measurement results for source, target and neighbours (including LTE/NR neighbouring cell measurements if available)

- Shr-cause to indicate which triggering condition was met

- UE information, e.g. C-RNTI

For the reporting of the inter-RAT SHR stored in the LTE variable, one solution is to introduce the whole reporting procedures in LTE RAN2, including the available indicator, the request from the network and the reporting flows. This requires the introduction of the complete reporting procedures.

The other is to consider the container reporting in NR, similar as the reporting of the LTE RLF report scheme. For the reporting of LTE RLF report, it was introduced when discussing the reporting of NR RLF report. The LTE RLF report shares the available and request indicators with the NR one. The reporting is finally defined as one of choice for the RLF report reported by the UE.

RLF-Report-r16 ::=                   CHOICE {

nr-RLF-Report-r16                    SEQUENCE {

},

eutra-RLF-Report-r16                 SEQUENCE {

}

}

However, for the intra-NR SHR, it will be reported as the mandatory for the R17 SHR reporting. It seems difficult to reuse the available and request indicators of the NR SHR. That is, if we want to support the reporting of inter-RAT SHR stored in LTE, we need to introduce the complete reporting procedures in NR.

Based on the above discussion, there will be three potential reporting schemes for the inter-RAT SHR from NR to LTE:

- Alt 1: reporting in LTE only

- Alt 2: reporting in NR only

- Alt 3: reporting in both LTE and NR

As we all know, the forwarding scheme of the SHR is still under discussion. It seems a bit earlier for RAN2 to start the discussion of the reporting of SHR. In our understanding, where to report the inter-RAT SHR from NR to LTE can wait for the decision on the forwarding scheme in RAN3. With the requirement in RAN3, RAN2 can avoid unnecessary reporting and reduce the potential delivery signalling at network side. For example, if the SHR is first sent to the target, it is beneficial to report the inter-RAT SHR from NR to LTE only in LTE. If the SHR is first send to the source, it seems more reasonable to report it when the UE moves back to NR. This will reduce the unnecessary delivery signalling of inter-system at network side.

Proposal 4: For the SHR from NR to LTE, RAN2 waits for the forwarding scheme in RAN3 to decide where to report the SHR, e.g., LTE only, NR only or both LTE and NR.

Case 2: HO from LTE to NR within NG-RAN

Although RAN3 suggested to consider case 1 firstly, obviously, the HO from LTE to NR will occur as frequently as the case 1. If the HO from LTE to NR is about to failure, this will also downgrade the capacity and performance of whole system. It is beneficial to consider the SHR function to ensure the robustness of the handover from LTE to NR as well.

Proposal 5: For intra-system inter-RAT SHR, HO from LTE to NR is supported.

NR-DC SPCR

Cases

Based on the LS, there are cases to consider for successful PSCell change report (SPCR) for NR-DC.

- SPCR for NR-DC, including:

  - SN- and MN-initiated classic PSCell change / CPC

  - intra-SN classic PSCell change / CPC

  - classic Addition / CPA

  - HO with SN change are not prohibited, but possibly addressed once the basic solution for SPCR is known.

As showed in the LS, it has been decided to start the SPCR for NR-DC.

Observation 4: NR-DC is prioritized for SPCR in RAN3.

Besides, there are seven cases suggested by RAN3 for RAN2 to start the SPCR discussion. The cases can be classified as:

- Scenario 1(classic PSCell change):

  - 1a. SN/ MN-initiated classic PSCell change;

  - 1b. Intra-SN classic PSCell change

- Scenario 2(conditional PSCell change):

  - 2a. SN/ MN-intimated CPC

  - 2b. Intra-SN CPC

- Scenario 3(PSCell addition)

  - 3a. Classic addition

  - 3b. CPA

- Scenario 4: HO with SN change

Technically speaking, SPCR is mainly introduced for the PSCell change optimization. Therefore, we prefer to consider the scenario 1 as highest priority. In addition, it is expected to first consider the classic PSCell change related scenario 1 and related solutions. For cases of scenario 2, they are related to CPC and we can take the solutions for the classic PSCell change as baseline to support the scenario 2.

As for scenario 3, these are related to PSCell addition. The PSCell addition related MRO is not included in the SPCR scope. From the other aspect, if there will not be too much new information needed based on the soluiton for PSCell change, it is acceptable for us to consider the PSCell addition this release. Otherwise, it should be postponed to next release.

For scenario 4, according to the LS from RAN3, we agree with RAN3 to deprioritize it for the time being.

Proposal 6a: RAN2 to focus on the following cases of SPCR:

- Scenario 1(classic PSCell change): 1a. SN/ MN-initiated classic PSCell change; 1b. Intra-SN classic PSCell change;

- Scenario 2(conditional PSCell change): 2a. SN/ MN-intimated CPC; 2b. Intra-SN CPC;

Proposal 6b: RAN2 for further discuss whether the following scenarios should be considered under the SPCR:

- Scenario 3: 3a. Classic addition; 3b. CPA;

Proposal 6c: RAN2 to deprioritize the following scenario under the SPCR:

- Scenario 4: HO with SN change;

Triggering and reporting

SPCR is used for the UE to generate and report the successful PSCell change information to detect the underlying failures. Similarly, SPCR triggering conditions should be considered to avoid unnecessary generation and reporting of the successful PSCell change information. With the SPCR triggering condition, only the successful PSCell change procedure with underlying failure can be recorded. This kind of SPCR can be beneficial for the network to perform the root cause analysis and make corresponding optimization.

Proposal 7: Introduce T304, T310 and T312 related triggering conditions of SCG for SPCR.

For the reporting of SHR, the UE is not limited to immediately report it to the target PCell but can store the SHR for at most 48 hours. Subsequently, the UE can indicate the availability of the SHR to any cell where the UE enters into connected state. As for the reporting scheme of SPCR, it is preferred to apply the similar principle. That is, The UE can store the SPCR for at most 48 hours. The UE can inform the network of the availability of the SPCR and report it the the network. The UE may discard the SPCR 48 hours after the last successful PSCell change information is stored or until the SPCR is fetched by the network.

Proposal 8: The UE stores the SPCR and discard the SPCR if not fetched within 48 hours.

Potential information in SPCR

Here, with the above assumption, we prefer to give our consideration on the potential information requesting the UE to provide for scenario 1 and 2.

- Scenario 1: 1a. SN- and MN-initiated classic PSCell change; and 1b. intra-SN classic PSCell change

If we take the information provided in the SHR as baseline, it is worth noting that for classic PSCell change, the UE should provide the following information in the SPCR at least: the source PSCell information, the target PSCell information, the cause to trigger the SPCR and the latest measurement results. These information is related to the successful PSCell change and can be used for the potential optimization.

Besides, for the MN initiated classic PSCell, it is also reasonable for the UE to provide the PCell information. If the PCell is responsible for the SPCR, with the PCell information, it is useful for the reception node to deliver the SPCR to the MN based on the PCell information.

For the CPC, it is also desirable for the UE to provide the time elapsed between the CPC execution and the reception of the CPC configuration.

As a small summary, we can conclude that the following information should be provided by the UE in the SPCR for scenario 1:

- Source and target PSCell information

- SPCR cause

- Latest measurement results

- PCell information, in case of MN initiated classic PSCell change

  - Scenario 2: 2a. SN- and MN-initiated CPC; and 2b.intra-SN CPC

With the analysis of scenario 1 in mind, we consider whether additional information is needed to support scenario 2. FOR CPC, the UE will perform the PSCell change upon the CPC triggering condition is met. The time between the reception of CPC configuration and the CPC execution may be beneficial for the network to perform the CPC optimization. Therefore, it is desirable for the UE to report this time information in addition to the information needed for scenario 1.

Compared with scenario 1, the UE needs to provide the following additional information:

- Time elapsed between the CPC execution and the reception of the CPC configuration in case of CPC

As a summary of the aforesaid considerations, we prefer RAN2 to consider that:

Proposal 9: RAN2 to include the following information in the SPCR for scenario 1 and 2:

-	Source PSCell information

-	target PSCell information

-	SPCR cause

-	Latest measurement results

-	PCell information, in case of MN initiated PSCell change/CPC

-	Time elapsed between the CPAC execution and the reception of the CPC configuration in case of CPC

Conclusion

In this paper, we discuss miscellaneous enhancements for intra-system inter-RAT SHR and SPCR, and we have the following observations and proposals:

Observation 1: SHR triggering conditions are configured by the source and/or target cell of the HO.

Observation 2: The UE stores the successful handover information in the variable upon successfully completing the Random Access procedure with the target cell of the handover if at least one of the SHR triggering conditions are met.

Observation 3: For the SHR from NR to LTE, the introduction of the T304 related triggering condition in the MobilityFromNRCommand message will have impact on the RRCConnectionReconfiguration message.

Observation 4: NR-DC is prioritized for SPCR in RAN3.

Proposal 1: For the SHR from NR to LTE, introduce the T310 and T312 related triggering condition in the MobilityFromNRCommand message.

Proposal 2: For the SHR from NR to LTE, whether to introduce the T304 related triggering condition in the MobilityFromNRCommand message is FFS.

Proposal 3: RAN2 to include the following information in inter-RAT SHR from NR to LTE:

- Source NR cell info and target LTE cell info

- Measurement results for source, target and neighbours (including LTE/NR neighbouring cell measurements if available)

- Shr-cause to indicate which triggering condition was met

- UE information, e.g. C-RNTI

Proposal 4: For the SHR from NR to LTE, RAN2 waits for the forwarding scheme in RAN3 to decide where to report the SHR, e.g., LTE only, NR only or both LTE and NR.

Proposal 5: For intra-system inter-RAT SHR, HO from LTE to NR is supported.

Proposal 6a: RAN2 to focus on the following cases of SPCR:

- Scenario 1(classic PSCell change): 1a. SN/ MN-initiated classic PSCell change; 1b. Intra-SN classic PSCell change;

- Scenario 2(conditional PSCell change): 2a. SN/ MN-intimated CPC; 2b. Intra-SN CPC;

Proposal 6b: RAN2 for further discuss whether the following scenarios should be considered under the SPCR:

- Scenario 3: 3a. Classic addition; 3b. CPA;

Proposal 6c: RAN2 to deprioritize the following scenario under the SPCR:

- Scenario 4: HO with SN change;

Proposal 7: Introduce T304, T310 and T312 related triggering conditions of SCG for SPCR.

Proposal 8: The UE stores the SPCR and discard the SPCR if not fetched within 48 hours.

Proposal 9: RAN2 to include the following information in the SPCR for scenario 1 and 2:

-	Source PSCell information

-	target PSCell information

-	SPCR cause

-	Latest measurement results

-	PCell information, in case of MN initiated PSCell change/CPC

-	Time elapsed between the CPAC execution and the reception of the CPC configuration in case of CPC

Reference

- R2-2209104, LS on the scope for the support of SON/MDT enhancements