3GPP TSG-RAN WG2 Meeting #119bis electronic	R2-2209870

E-Meeting, 10th – 19th Oct., 2022

Agenda item:	8.4.2.3

Source:	Samsung

Title:	L1/L2 signalling for inter-cell mobility

Document for:	Discussion & Decision

# Introduction

According to WID on Further NR mobility enhancements [1], the detail objectives to support L1/L2 inter-cell mobility is provided as below:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In this contribution, we consider the dynamic switch mechanism using L1/L2 signalling and potential applicable scenario based on L1/L2 signaling.

# Discussion

Regarding the applicable scenario based on L1/L2 signaling, RAN2 made following agreements during the RAN2#119 meeting.

Confirm to Support L1/L2-based inter-cell mobility for inter-DU scenario (as well as intra-DU scenarios).

The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different.

R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery.

## L1/L2 signalling for inter-cell mobility

As a dynamic switch mechanism among candidate serving cells, L1/L2 signaling for inter-cell mobility is required. The basic assumption is that similar signaling mechanism of L1/L2 inter-cell beam management (i.e. unified TCI state activation/deactivation MAC CE and DCI indication) can be used.

- MAC CE is used to indicate the L2 mobility (i.e. indicate a TCI state for L2 mobility); or

- MAC CE is used to indicate the candidate beams (activation/deactivation) and DCI indicates the L1 mobility among the candidate beams

At least, the additional information is required to differentiate between the L1/L2 inter-cell mobility and the inter-cell beam management because we should cover all inter-cell change cases. For example following information is required in the (new or legacy) MAC CE.

- Indicates the inter-cell mobility (i.e. HO) by new LCID or field.

- Indicates whether RACH procedure is required or not.

- Optionally, indicates the whether L2 reset is required or not.

- Confirmation signaling (e.g. MAC CE) regarding the MAC CE signaling for L1/L2 inter-cell mobility.

Proposal 1: New MAC CE is introduced similar as Rel-17 L1/L2 inter-cell beam management signaling (i.e. unified TCI state activation/deactivation MAC CE and DCI indication).

Proposal 2: Additional information is needed to differentiate L1/L2 inter-cell mobility from the inter-cell beam management.

- Indicates the inter-cell mobility (i.e. HO) by new LCID or field.

- Indicates whether RACH procedure is required or not.

- Optionally, indicates the whether L2 reset is required or not.

- Confirmation signaling (e.g. MAC CE) regarding the MAC CE signaling for L1/L2 inter-cell mobility.

# Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: New MAC CE is introduced similar as Rel-17 L1/L2 inter-cell beam management signaling (i.e. unified TCI state activation/deactivation MAC CE and DCI indication).

Proposal 2: Additional information is needed to differentiate L1/L2 inter-cell mobility from the inter-cell beam management.

- Indicates the inter-cell mobility (i.e. HO) by new LCID or field.

- Indicates whether RACH procedure is required or not.

- Optionally, indicates the whether L2 reset is required or not.

- Confirmation signaling (e.g. MAC CE) regarding the MAC CE signaling for L1/L2 inter-cell mobility.

# References

RP-213565	New WID on Further NR mobility enhancements	MediaTek.