3GPP TSG RAN WG2 Meeting #119bis-e                                                        	R2-2209905

E-Meeting, 10th – 19th October, 2022

Agenda item:	8.16.2

Source:	Samsung

Title:	AI/ML Model Management

Document for:	Discussion and Decision

# 1. Introduction

In RAN#96 meeting, RAN agreed the Rel-18 revised study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1]. The following are selected objectives of this SID:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels […] For the use cases under consideration: […] - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - .. - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case

In this contribution, we discuss aspects of RAN1 study and agreements on AI/ML model management (e.g. training and inference location) for the initial set of three use cases.

# 2. Discussion

In RAN1#109-e and RAN1#110 meetings, RAN1 discussed the general aspects of AI/ML framework, terminologies, gNB-UE collaboration levels, and evaluation and other aspects of AI/ML for CSI feedback enhancement, beam management, and positioning accuracy enhancement use cases.

More specifically, RAN1 discussed AI/ML model training related terminologies, i.e. whether training is performed online/offline, model update, and model transfer (full model or partial model) with respect to the following network-UE collaboration levels [2]:

Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signalling-based collaboration without model transfer - Level z: Signalling-based collaboration with model transfer

Additionally, RAN1 discussed for the three use cases, the different alternatives for AI/ML model training and/or inference location. That is, whether the model is located at UE-side, network-side, or two-sided [3]:

### CSI feedback enhancement Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, repectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. ### Beam management Agreement At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training: - Alt.1: AI/ML model training at NW side; - Alt.2: AI/ML model training at UE side. Note: Whether it is online or offline training is a separate discussion. ### Positioning accuracy enhancement Agreement Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least - UE-side or Network-side training - UE-side or Network-side inference - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches

Observation 1: RAN1 will further study alternatives of AI/ML model training and inference for the three use cases, considering the network-UE collaboration levels.

Proposal 1: RAN2 to study specifications impact due to AI/ML model training and/or inference at the UE-side, network-side, or two-sided model (at network- and UE-side) for the three uses cases.

Moreover, RAN1 agreed to discuss necessity/feasibility/potential specification impact for data collection for AI/ML model training/inference/update/monitoring, for the different uses cases:

### CSI feedback enhancement Agreement In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact, for data collection for AI/ML model training/inference/update/monitoring: · Assistance signaling for UE’s data collection · Assistance signaling for gNB’s data collection · Delivery of the datasets ### Beam management Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: · Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals · Content/type of the collected data · Other aspect(s) is not precluded ### Positioning accuracy enhancement Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report

Observation 2: RAN1 to further discuss necessity/feasibility/potential specification impact for data collection for AI/ML model training/inference/update/monitoring for the three uses cases.

Proposal 2: RAN2 to discuss specification impact due to data collection for AI/ML model training/inference/update/monitoring, for the three uses cases.

# 3. Conclusion

In this contribution we discussed aspects of RAN1 study and agreements on AI/ML model management (e.g. training and inference location) for the initial three use cases. The following are the observations and proposals in this document:

Observation 1: RAN1 will further study alternatives of AI/ML model training and inference for the three use cases, considering the network-UE collaboration levels.

Observation 2: RAN1 to further discuss necessity/feasibility/potential specification impact for data collection for AI/ML model training/inference/update/monitoring for the three uses cases.

Proposal 1: RAN2 to study specifications impact due to AI/ML model training and/or inference at the UE-side, network-side, or two-sided model (at network- and UE-side) for the three uses cases.

Proposal 2: RAN2 to discuss specification impact due to data collection for AI/ML model training/inference/update/monitoring, for the three uses cases.

# 4. Reference

RP-221348 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

RAN1#109-e Chairman report

RAN1#110 Chairman report.

R1-2208178 Summary#1 of General Aspects of AI/ML Framework