3GPP TSG-RAN WG2 Meeting # 119-bis-e                                                          R2-2209926

# Online, 10th - 19th October 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3518 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Small Data Transmission under TN & NTN Mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FGI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Rel-17, UE may support RA-SDT/SRB-SDT in TN access but not support RA-SDT/SRB-SDT in NTN access. Such condition can be found while the UE reports NTN-parameters to the serving RAN. - However, while TN/NTN mobility event happens, the UE may consider the conditions for initiating SDT are fulfilled even the UE does not support RA-SDT/SRB-SDT in NTN access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add one additional condition: 1> if the UE receives the sdt-Config from a TN cell and the UE receives the sdt-Configcommon from an NTN cell; 2> if the UE supports RA-SDT in NTN access; and to further indicate that UE should check UE capability for RA-SDT after TN/NTN mobility event. - Replace the original condition in the conditions for initiating SDT: 1> all the pending data in UL is mapped to the radio bearers configured for SDT; by the new condition: 1> all the pending data in UL is mapped to the radio bearers configured and supported by the UE for SDT; to further indicate that the radio bearers (i.e., SRB2) shoud be confiugred and supported by the UE for SDT(e.g., in NTN access). Impact analysis Impacted functionality: Conditions for initiating SDT in UE Inter-operability: No network impact in this CR. Implementation of this CR by a UE will not cause compatibility issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Un-predictable UE behavior |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.3.13.1b	Conditions for initiating SDT

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	the upper layers request resumption of RRC connection; and

1>	SIB1 includes sdt-ConfigCommon; and

1>	sdt-Config is configured; and

1> if the UE receives the sdt-Config from a TN cell and the UE receives the sdt-Configcommon from an NTN cell;

2> if the UE supports RA-SDT in NTN access; and

1>	all the pending data in UL is mapped to the radio bearers configured and supported by the UE for SDT; and

1>	lower layers indicate that conditions for initiating SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

End of  change