3GPP TSG-RAN WG2 Meeting #119bis-e		R2-2209931

Online, October 10-19, 2022

Agenda Item:	8.4.2.3

Source: 	MediaTek Inc.

Title:  	Cell Switch for L1/L2-based Inter-cell Mobility

Document for:	Discussion and decision

# Introduction

In last RAN2 meeting, we discussed target performance improvements and candidate solutions for L1/L2-based inter-cell mobility and following agreements/working assumptions were made:

- Assumption: HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. FFS if TRS tracking after HO and CSI RS measurement should also be included, i.e. the time to use a high-performance beam (can be clarified further). - Assumption: To reduce HO interruption time, investigate e.g. solutions to reduce the time for UE reconfiguration (already in the WID), downlink and uplink synchronization after handover decision (other parts of dynamic switch not precluded). - Confirm to Support L1/L2-based inter-cell mobility for inter-DU scenario (as well as intra-DU scenarios). - The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different. - R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery. - ICBM is one scenario considered for L1L2 mobility, but is not the only one, and is not a prerequisite for using L1L2 mobility. - RAN2 to consider preparation of target cell configurations capable of dynamic switching without need for full configuration. - Measurement delay can/may be considered in this work - Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS) - R2 will initially focus on PCell mobility. - R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases - a) the target PCell/target SCell(s) is not a current serving cell (CA CA scenario with PCell change) - b) FFS the target PCell is a current SCell - c) FFS the target SCell is the current PCell. - DC scenarios are FFS (e.g. PSCell mobility may be a low hanging fruit FFS). - Current options on the table: to configure a L1/L2 inter-cell mobility candidate cell: - a. One RRCReconfiguration message for candidate target cell - b. One CellGroupConfig IE for each candidate target cell - c. One SpCellConfig IE for each candidate target cell - Will send an LS to RAN1 and RAN3 on the progress of this meeting.

A baseline time chart is provided as the output of [Post119-e][036], captured in the endorse running stage-2 CR [1].

Figure 1.	Mobility latency time chart

In this contribution we discuss key aspects of dynamic cell switch in L1/L2-based mobility from RAN2 perspective. Then we provide the overall procedure of L1/L2-based inter-cell mobility.

# Discussion

## Cell switch

### Trigger of L1/L2 mobility and L1 enhancements

In last meeting, we agreed the working assumption that we rely on L1 measurements to trigger L1/2 mobility. Although many details need to be further discussed, the assumption implies that Rel-17 unified TCI framework for inter-cell beam management is a reasonable starting point. That is, UE is configured to perform L1 measurement and reporting for inter-cell reference signals corresponding to beams from another cell, and network sends inter-cell beam indication if a good inter-cell beam is reported. However, unified TCI framework introduced in Rel-17 does not support mobility (i.e., no cell change). In Rel-18, additional mechanism (L1/L2 signalling) to trigger cell change needs to be defined.

Observation 1: Rel-17 unified TCI framework does not support mobility; if used for Rel-18 inter-cell mobility, new signalling to trigger cell switch needs to be introduced.

The L1/L2 signalling used to trigger L1/L2 mobility can be MAC CE (L2) or DCI (L1). Considering MAC CE allows more flexible format and requires less standardization effort than DCI, we suggest that a new MAC CE be introduced. The MAC CE fields include at least the index of target cell/cell group and TCI state(s) to be used in target cell, details FFS.

Proposal 1: Cell switch in L1/L2-based inter-cell mobility is triggered by MAC CE.

Proposal 2: The MAC CE with cell switch command includes target cell index and TCI state(s). Other information FFS.

Although L1 enhancements for inter-cell beam management (including L1 measurement and reporting, and beam indication) are mainly of RAN1 scope, it is stated in the WID that early RAN2 involvement is necessary. The RAN2 assumption also indicates directions for the following objective of L1 enhancements for inter-cell beam management:

- L1 enhancements are needed to allow L1 measurement and reporting of inter-cell beams. Notice that unlike Rel-17 ICBM, where the configurations for neighbour cell RSs and TCI states are included in serving cell configurations (e.g., CSI-SSB-ResourceSet with servingAdditionalPCIList), in Rel-18, the inter-cell RSs and TCI states may be configured

- Regarding beam indication, if MAC CE is used to trigger cell switch, the signalling for inter-cell beam indication (with cell switch) will be discussed in RAN2, but DCI-based beam indication remains for the case without serving cell change.

We suggest that RAN2 inform RAN1 of our progress on RRC configurations and cell switch mechanism, to help RAN1 identify potential L1 enhancements for inter-cell beam management.

Proposal 3: Inform RAN1 of RAN2 decisions about dynamic switch mechanism.

### Co-existence with inter-cell beam management

In last meeting, RAN2 discussed the relationship between Rel-17 inter-cell beam management (ICBM) and Rel-18 L1/L2-based inter-cell mobility and agreed that ICBM is one scenario to consider for L1/L2 mobility, although ICBM is not the only one scenario and not prerequisite for using L1/L2 mobility.

We understand that both mechanisms may be configured together for a UE (if UE supports both). In this case, a neighbour cell is likely be considered as an “additional cell” for ICBM as well as a “candidate cell” for L1/L2-based inter-cell mobility. UE may start using neighbour cell beams before cell switch and continue using the same beam after cell change.

An example considering two cells, each with 3 TCI states (beams), is illustrated below. In Rel-17 ICBM (Figure 2(a)), all 6 TCI states are included in UE’s TCI state list, and TCI states A4 to A6 are associated with additionalPCI (i.e., Cell B), since they actually come from another cell. Assume UE is using the beam corresponding to TCI-State A5 when served by Cell A, and it wants to continue using this beam after switching to Cell B. In this way, UE only needs to perform necessary reconfiguration upon cell change, and we achieve seamless mobility with minimized interruption. However, in Rel-18 L1/L2 mobility (Figure 4(b)), The TCI states (beams) from Cell B may have different IDs (B1 to B3). Then there needs to be some mechanism for UE to recognize that TCI state B2 is equivalent to TCI state A5.

We propose to study beam continuation from Rel-17 ICBM to Rel-18 L1/L2 mobility when both methods are configured for a UE.

(a) Rel-17 ICBM

(b) Rel-18 L1/L2 mobility

Figure 2.	Co-existence of Rel-17 ICBM and Rel-18 L1/L2 mobility

Proposal 4: Beam continuation from Rel-17 ICBM to Rel-18 L1/L2 mobility is supported when both methods are configured for a UE.

## DC and CA scenarios

DC scenarios were left FFS in last meeting. We believe that L1/L2-based procedure should also support PSCell mobility, since the procedure resembles that of PCell mobility.

Proposal 5: L1/L2-based inter-cell mobility procedure is applicable to both PCell and PSCell mobility.

Regarding CA scenarios, in last meeting we confirmed that L1/L2-based mobility procedures support CA to CA scenario with PCell change (target cell is not a current serving cell). The scenarios of swapping PCell and SCell (i.e., (b) and (c)) were left FFS. This is related to the support of “inter-frequency” scenario. Even if the measurement on PCell and SCell(s) are on-going, the comparation of measurement results of different frequencies is required. From the RAN2 aspects of measurement and cell switch, PCell and SCell swap is considered as inter-frequency operations.

Observation 2: From RAN2 perspective, when target PCell [SCell] is current SCell [PCell], both measurement and cell switch are considered as inter-frequency operations.

In legacy procedure, such PCell/SCell swap is done by RRC procedures, relying on L3 measurement. According to the assumption that we rely on L1 measurements to trigger L1L2 mobility, we believe the same principle is also applied to the scenario of PCell/SCell swap. Considering L1 measurement and report is not performed for the deactivated SCell, PCell/SCell swap is only possible when the SCell (target PCell) is activated.  If the SCell is deactivated, replacing RRC message with L1/L2 cell switch command doesn’t really reduce the mobility latency. Only when the SCell (target PCell) is activated, UE can skip DL synchronization and UL synchronization when performing PCell/SCell swap. If L1/L2-based mobility is intended to be supported for PCell and SCell swap, we need to limit the “inter-frequency” support of L1/L2-based mobility to the case that the SCell (target PCell) is an activated cell. The goal of faster cell switch may not be achieved in other cases where measurement gaps are required.

Moreover, the signalling formats of L1/L2 indications may be different for the scenarios where target cell is and is not a current serving cell.

Proposal 6: L1/L2-based mobility procedure for PCell/SCell swap is applicable to the scenarios where target PCell [SCell] is activated.

Proposal 7: The “inter-frequency” scenario supported by L1/L2-based mobility only consider PCell/SCell swap with the target PCell(SCell) being activated.

The signalling formats of L1/L2 indications may be different for the scenarios where target cell is and is not a current serving cell.

## Inter-DU operation

In last RAN2 meeting, we confirmed the support of L1/L2-based inter-cell mobility for inter-DU scenario. Furthermore, the design for intra-DU and inter-DU L1/L2 mobility should share as much commonality as reasonable. But it’s for sure that there are inter-DU specific issues, which need to be addressed.

The main aspect needs to be considered for inter-DU mobility is UP (RLC/MAC) handling and the coordination among CU, source DU and target DU over F1 interface.  However, the impact of additional latency due to UP reset and cell switch preparation delay over F1 interface to the mobility performance is marginal. The mobility performance of L1/L2-based inter-cell mobility for inter-DU is comparable to intra-DU.

In legacy handover mechanism, the handover decision is made in a relatively stable manner based on the RRM measurements/reports.  The network tries to balance the mobility performance of ping-pong effect and HOF rate by adjusting TTT value and thresholds of the triggering events. However, the L1/L2 inter-cell mobility mechanism is targeting to fast cell switch based on L1 measurements/report to reduce the mobility latency and improve the mobility reliability. The ping-pong rate is much higher than legacy handover mechanism.

If inter-DU mobility also wants to obtain the similar benefits as intra-DU mobility, it also needs to rely on L1 measurements/reports to make mobility decision and take advantages of ping-pong effect instead of minimizing it. But the drawback is increased interaction/signalling overhead over F1 interface due to the high ping-pong rate.

RAN2 should discuss how to handle the ping-pong effects for inter-DU mobility. The opposite option is to minimize the ping-pong rates as legacy handover procedure by setting longer TTT or introducing additional L1-RSRP threshold for cell switch, similar as legacy handover procedure. However, the benefits obtained by L1/L2 inter-cell mobility mechanism will shrink dramatically. The mobility reliability will degrade, and mobility latency will increase.

Proposal 8: RAN2 discuss how to handle the ping-pong effects for inter-DU mobility:

- Option 1: Minimize ping-pong rates/longer TOS to reduce the interactions between CU and DUs.

- Option 2: Take advantage of ping-pong effect/shorter TOS to improve the mobility reliability.

Compared with intra-DU mobility, the major extra procedures for inter-DU mobility are RLC reestablishment and MAC reset.  The frequent RLC re-establishment will result in more packet loss and longer packet delivery latency just as illustrated in Figure 3.

- Packet loss for RLC-UM bearer: PDCP PDUs which are not successfully transmitted/received to/from source cell are discarded.

- Longer packet delivery latency for RLC-AM bearers: PDCP PDUs which are not successfully transmitted/received to/from the source cell are retransmitted/re-received to/from the target cell.

This is illustrated below.

Figure 3.	Pack loss in RLC-UM and longer latency in RLC-AM

Considering Rel-18 L1/L2-based mobility targets more frequent cell switching (e.g., back-and-forth among cells in a region), UE experience degradation due to more packet loss or longer packet delivery latency is more severe. Therefore, RAN2 should discuss potential enhancements to mitigate packet loss and latency problems due to RLC reestablishment for L1/L2-based inter-DU mobility.

One possible way to minimize packet data loss for RLC-UM bearer is to support PDCP packet retransmission when cell switch occurs, similar as RLC-AM bearer.  Considering PDCP status report has already been supported in Rel-16 DAPS handover upon uplink data switching, it can also be extended to support inter-DU mobility in Rel-18.

Proposal 9: Support PDCP status report and PDCP packet retransmission for UM DRBs to reduce the packet loss for L1/L2-based inter-DU mobility.

To support fast cell switch, there are two possible methods to support L1/L2-based inter-DU mobility from UP protocol point of view, i.e., single-protocol or dual-protocols, as illustrated in Figure 4.

RLC

MAC

CU/PDCP

PDCP

Source DU

UE

F1

MAC

RLC

CU/PDCP

PDCP

Target DU

UE

RLC

MAC

F1

MAC

RLC

RLC

MAC

CU/PDCP

PDCP

Source DU

UE

F1

RLC

MAC

F1

MAC

RLC

MAC

RLC

RLC

MAC

CU/PDCP

PDCP

UE

F1

RLC

MAC

F1

MAC

RLC

MAC

RLC

Target DU

Source DU

Target DU

a) Single-protocol                                                                       b) Dual-protocol

Figure 4.	Single-protocol vs. Dual-Protocol for inter-DU mobility

The two methods are further explained below.

- Single Protocol: From UE side, single protocol of one MAC entity and one set of RLC entities is associated with the serving cell. Before cell switch, the single protocol is associated with the source cell/DU and is associated with the target cell/DU after cell switching.

- Dual Protocols: Frome UE side, dual protocols, each protocol comprising one MAC entity and one set of RLC entities, are associated with the source cell/DU and target cell/DU respectively. Only one single protocol is actively in use. Before cell switching, the protocol associated to the source cell/DU is in use; after cell switching, the protocol associated to the target cell/DU is in use.

The single protocol method is considered as the basic solution for inter-DU mobility. MAC reset and RLC re-establishment are always required for cell switching. The issue of frequent packet loss can’t be avoided. Further optimization should be considered, just as mentioned Proposal 6. The dual protocol method is considered as a further optimization, which can further reduce the mobility latency. In dual protocol, the MAC entity and the RLC entities can be prepared upon reception of the reconfiguration message. UE can use the protocol associated to the target cell immediately upon reception of the cell switch command. Furthermore, the architecture of dual protocol is the same as split bearer. It allows that possibility to avoid RLC re-establishment and MAC reset when UE is switched back and forth between the source cell and the target cell. Considering the UP process for dual protocol is similar as split-bearer, there is not additional requirement on UE capability. UE which is capable of supporting split-bearer is able to support dual protocol directly.

Proposal 10: RAN2 consider both the solutions of single protocol and dual protocols for L1/L2-based inter-DU mobility.

- Single protocol: single protocol of one MAC entity and one set of RLC entities is associated to the serving cell from UE aspect.

- Dual Protocols: dual protocols with each protocol comprising one MAC entity and one set of RLC entities are associated to the source cell/DU and target cell/DU respectively from UE aspect.

## Overall procedure

Inter-cell mobility is not as fast as intra-cell beam switch because UE needs to perform reconfiguration and DL/UL synchronization towards target cell.

To allow faster cell switching via L1/L2 signalling, candidate cells can be pre-configured for UE. Then, UE can perform synchronization to some candidates (potential target cell) earlier. In this way, UE can quickly switch to target cell once it receives cell switch command.

Taking inter-DU operation as an example, the overall procedure of L1/L2-basedinter-cell mobility is illustrated in the figured below.

Figure 5.	Overall procedure of L1/L2-based inter-cell mobility

L1/L2-based inter-cell mobility in inter-DU operation consists of the following major parts (some steps among CU source DU and target DU can be skipped in case of intra-DU. It is left to RAN3 to consider the detailed procedures and signalling among CU, source DU and target DU to support inter-DU inter-cell mobility):

Pre-configuration

- (Step 1 to 2) UE sends L3 measurement report to gNB-DU based on measurement configurations. The measurement report is forwarded to gNB-CU.

- (Step 3 to 8) The gNB-CU decides the candidate set for UE, sends a preparation request to target DU and receives acknowledgement from target DU (This may be implemented by existing signalling, e.g., UE context setup request/ response). Then gNB-CU sends the RRC reconfiguration to UE and receives complete message, via source gNB-DU.

Early synchronization

- (Step 9 to 11) UE performs L1 measurements and reports for reference signals (SSB or CSI-RS) corresponding to inter-cell beams, following configurations from network. Based on L1 measurement reports, network may activate some TCI states QCL-ed with cells whose PCI is different from serving cell. UE performs DL synchronization (DL and optionally UL) for these cells.

Cell switch

- (Step 12 to 13) Based on further L1 report, gNB-DU may indicate target cell and beam (TCI state). UE applies target cell configurations.

- (Step 14) If TA is not available, UE performs RACH towards the indicated target cell.

- (Step 15 to 16) UE receives PDCCH from target cell using new TCI state.

The detailed procedures may be further elaborated during the WI. The intention to provide the full picture of the overall procedure and identify the key enablers to support L1/L2-based inter-cell mobility, i.e., pre-configuration of candidate cell(s), early synchronization, cell switch by L1/L2 signalling. The major steps shown here can be considered as a baseline, starting from which, we can study the design of each step to achieve the goal of fast cell switching.

Proposal 11: The basic L1/L2-based inter-cell mobility procedure is comprised of pre-configuration of candidate cell(s), early synchronization towards the candidate cell(s), and fast cell switch by L1/L2 signalling.

# Conclusion

We have the following observations:

Observation 1:	Rel-17 unified TCI framework does not support mobility; if used for Rel-18 inter-cell mobility, new signalling to trigger cell switch needs to be introduced.

Observation 2: From RAN2 perspective, when target PCell [SCell] is current SCell [PCell], the measurement is considered intra-frequency, but cell switch can be considered inter-frequency.

It is proposed to discuss and decide on the following proposals:

Proposal 1: Cell switch in L1/L2-based inter-cell mobility is triggered by MAC CE.

Proposal 2: The MAC CE with cell switch command includes target cell index and TCI state(s). Other information FFS.

Proposal 3: Inform RAN1 of RAN2 decisions about dynamic switch mechanism.

Proposal 4: Beam continuation from Rel-17 ICBM to Rel-18 L1/L2 mobility is supported when both methods are configured for a UE.

Proposal 5: L1/L2-based inter-cell mobility procedure is applicable to both PCell an PSCell mobility.

Proposal 6: L1/L2-based mobility procedure for PCell/SCell swap is applicable to the scenarios where target PCell [SCell] is activated.

Proposal 7: The “inter-frequency” scenario supported by L1/L2-based mobility only consider PCell/SCell swap with the target PCell(SCell) being activated.

Proposal 8: RAN2 discuss how to handle the ping-pong effects for inter-DU mobility:

- Option 1: Minimize ping-pong rates/longer TOS to reduce the interactions between CU and DUs.

- Option 2: Take advantage of ping-pong effect/shorter TOS to improve the mobility reliability.

Proposal 9: Support PDCP status report and PDCP packet retransmission for UM DRBs to reduce the packet loss for L1/L2-based inter-DU mobility.

Proposal 10: RAN2 consider both the solutions of single protocol and dual protocols for L1/L2-based inter-DU mobility.

- Single protocol: single protocol of one MAC entity and one set of RLC entities is associated to the serving cell from UE aspect.

- Dual Protocols: dual protocols with each protocol comprising one MAC entity and one set of RLC entities are associated to the source cell/DU and target cell/DU respectively from UE aspect.

Proposal 11: The basic L1/L2-based inter-cell mobility procedure is comprised of pre-configuration of candidate cell(s), early synchronization towards the candidate cell(s), and fast cell switch by L1/L2 signalling.

# Reference

- R2-2209255, 38.300 running CR for introduction of NR further mobility enhancements, MediaTek Inc, Aug. 2022

- RP-221799, Revised WID on Further NR mobility enhancements