3GPP TSG-RAN WG2 Meeting #119bis electronic          	          R2-2210006

Online, October 10 – 19, 2022

Agenda Item:	6.21.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on per-FR-gaps

Document for:	Discussion and decision

- Introduction

Per-FR-gaps has been discussed in RAN#97e with the following conclusions [1].

Conclusion F1: The following Root Cause / Justification is applicable: a) the possibility to do gapless measurements is valuable, which is possible by using per-FR-gaps, b) the assumption behind per-FR-gaps that FR1 and FR2 has separate resources in the UE is no longer true, and is the main reason why per-FR gaps need more fine-granular capability, or why other solution like needforgap should be considered. Conclusion F2: RAN2 to be tasked to progress this issue (also taking into account comments collected at TSG RAN), including solutions Alt 1.1 (More fine grained capability for Per-FR-Gaps, 1 bit per BC), Alt 1.3 (more fine grained capability for Per-FR-Gaps - limited by number of carriers), and Alt 2 (Use similar framework/procedure as for ”NeedForGap”). Conclusion F3: RAN2 to consider Alt. 1.1, 1.3 and 2, and discuss the signaling overhead and network processing requirements/complexity. RAN2 is tasked to provide results after one Quarter, and leave final decision(s) to TSG RAN. Comment by RAN2 chair: RAN2 would attempt to: select solution, make assumption for the Release and provide technically endorsed CRs to next TSG RAN, and leave remaining decision(s) if any, and final decision(s) to TSG RAN.

In this contribution, we further analyse the potential solutions and provide our views on them.

- Discussion

To support finer granularity indication for per-FR-gaps, the motivation is that with more complex NR CA combinations (UE operating in a high order CA combination will not have additional hardware available to perform per FR gap), the per-UE level per-FR-gap capability becomes less effective since if there is one BC that cannot support per-FR-gap, UE can only report per-UE level per-FR-gap capability as “not supported”. In this case, the NW will not configure per-FR-gap to some BCs even if per-FR-gap can be supported. This downgrades the system performance.

Observation: The per-UE level per-FR-gap capability becomes less effective if there exist complex BC(s) that cannot support per-FR-gap.

Three potential solutions are provided to support finer granularity indication for per-FR-gaps as below.

Alt 1.1. Introducing per-BC level per-FR-gap capability. This solution is simple and backward compatible, although the signalling overhead increases.

Alt 1.3. Introducing a threshold N indicating the maximum number of CCs (per UE/per FR). This solution is non-backward compatible since the new field needs to be combined with existing capability field for interpretation. If the new UE indicates support of per-FR-gap by existing IE independentGapConfig and additionally indicate a threshold N, the old gNB will misunderstand that all BCs support per-FR-gap, but only the BCs whose number of CCs is smaller than N support per-FR-gap. Besides, for different BCs, the threshold N may be different, so a per UE/per FR threshold N may not be effective.

Alt 2. Using similar framework as for “NeedForGap”. How this solution works does not fully discussed in RAN#97e. In our view, enhancement for existing “NeedForGap” is needed, for example, 1-bit per-FR-gap field needs to be added for each target band to indicate whether per-FR-gap is supported or not. Then, the relation between existing per-UE level per-FR-gap capability independentGapConfig and new per-FR-gap field needs to be explicitly stated. For example, if the existing per-UE level per-FR-gap capability is set to “supported”, the UE does not need to report new per-FR-gap field and the network comprehends that all BCs support per-FR-gap; If the existing per-UE level per-FR-gap capability is indicated as “not supported” (i.e. the IE is absent), the UE can further report new per-FR-gap field and the network will check the new per-FR-gap field. This solution is also backward compatible.

Proposal 1: RAN2 to downselect the solution from:

- Alt 1.1. Introducing per-BC level per-FR-gap capability

- Alt 2.    Using similar framework as for “NeedForGap”

Regarding which release to support finer granularity indication for per-FR-gaps, if RAN2 can determine the solution, e.g. from Alt 1.1 or Alt 2 without NBC issue, during this Quarter, it can be introduced from Release 17. However, if the solution is complicated and it is difficult to reach consensus, Release 18 is suggested.

Proposal 2: It is suggested to support finer granularity per-FR-gaps capability from Release 17 if solution can be determined during this Quarter. Otherwise, Release 18 is suggested.

- Conclusion

In this contribution, we further analyse the potential solutions for per-FR-gaps and provide the following observation and proposals:

Observation: The per-UE level per-FR-gap capability becomes less effective if there exist complex BC(s) that cannot support per-FR-gap.

Proposal 1: RAN2 to downselect the solution from:

- Alt 1.1. Introducing per-BC level per-FR-gap capability

- Alt 2.    Using similar framework as for “NeedForGap”

Proposal 2: It is suggested to support finer granularity per-FR-gaps capability from Release 17 if solution can be determined during this Quarter. Otherwise, Release 18 is suggested.

- References

- RP-222590, 97e-30-UE-Capabilities - Version 0.0.4 RAN, Moderator (Mediatek Inc).