3GPP TSG-RAN WG2 Meeting #119bis electronic	R2-2210007

Online, October 10 – 19, 2022

Agenda Item:	8.17.2.2

Source:	Huawei, HiSilicon

Title:	Discussion on UE capability update for MUSIM

Document for:   Discussion and decision

Introduction

R18 MUSIM WID [1] have two objectives as below:

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE The work item shall identify whether the WI will have RAN3 or RAN4 impacts by RAN#99 [RAN2]. 2. Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2] - Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2] - The following MUSIM gap requirements are considered - Measurements in Network A - Measurements in Network B in RRC idle/inactive - Note: it is up to RAN4 decision whether to define requirements for Network B. - Identify and specify, if needed, solutions for MUSIM gap collision handling for the following cases [RAN4, RAN2] - Case 1: Collisions between MUSIM gap and legacy measurement gap (i.e., Rel-15 to Rel-17 measurement gaps) - Case 2: Collisions between MUSIM gap and SMTC - Case 3: Collisions between different MUSIM gaps - Note: RAN2 work can be triggered by RAN4 LS only, if needed - Identify impacts on L1 measurements, RLM/BFD and L3 measurements and specify corresponding UE requirements, if necessary, when MUSIM gap(s) are configured, for the following scenarios [RAN4] - Only MUSIM gap(s) are configured - MUSIM gap(s) and legacy measurement gap are configured - Note: requirements are applicable to MUSIM gaps defined in Rel-17 MUSIM WI (LTE_NR_MUSIM)

In this contribution, we discuss the objective 1 and mainly focus on the UE capability update.

Discussion

Considering the scenarios discussed in [2], if the SIM B is in RRC_IDLE/RRC_INACTIVE, SIM A can use full capability, e.g. all RF chains will be occupied by the SIM A. But if the SIM B enters RRC_CONNECTED, some of the RF chains will be occupied by the SIM B, so the RF chains that can be used by SIM A will be restricted. Thus, it is clear that the RF chains, which is indicated by capability field maximum UL/DL MIMO layer, needs to be updated for MUSIM purpose. For example, assuming the UE with capability of 2 UL MIMO layers and 4 DL MIMO layers, if the SIM B is in RRC_IDLE/RRC_INACTIVE, the UE can use 2 UL MIMO layers for transmission and 4 DL MIMO layers for reception in NW A. After the SIM B switches to RRC_CONNECTED, the UE may split the capability between NW A and NW B, e.g. the UE may only use 1 UL MIMO layer for transmission and 2 DL MIMO layers for reception in NW A. After the SIM B switched from RRC_CONNECTED state to RRC_IDLE/INACTIVE state again, the UE will resume its capability to support 2 UL MIMO layer and 4 DL MIMO layers in NW A. In this case, the UE should be able to inform the NW A of its capability update in terms of the maximum UL/DL MIMO layer. Otherwise, the NW A may schedule the UE with the number of MIMO layer that UE cannot support.  For other capabilities, we not see the real motivation to support the update for MUSIM operation in RRC_CONNECTED state simultaneously in two networks.

Proposal 1: For the scenarios where the MUSIM UE operates in RRC_CONNECTED state simultaneously in two networks, a procedure by which the UE can inform the NW A of the update on the capability of maximum UL/DL MIMO layer, will be specified. It is FFS whether there is the need for the UE to indicate the other capability update via this procedure.

For the above procedure, we think the current UE assistance information framework can be used, as illustrated in Figure 1.

- The MUSIM UE needs to be configured by the gNB whether the UE capability update for MUSIM operation is allowed or not via RRC Reconfiguration message.

- If the UE capability update for MUSIM operation is enabled by the gNB, the UE can initiate the UAI procedure for temporary capability update. In the UAI message, the UE can indicate the maximum UL/DL MIMO layer it temporary supports due to the MUSIM operation. The detailed signalling structure design can be further studied.

- After receiving the UAI for UE capability update for MUSIM operation, the gNB can adjust the configuration to meet the UE temporary capability, e.g. by sending RRC Reconfiguration message to update the configuration.

UE

gNB

2) UE Assistance Information

(indicate the temporary capability)

3) RRC Reconfiguration

UE initiate UE capability update

1) RRC Reconfiguration

(enable the UE capability update)

Figure 1: Signalling framework for UE capability update

Proposal 2: UE assistance information framework shall be used for UE temporary capability update procedure for MUSIM operation.

After the UE transmits the UAI message for temporary capability restriction, if the UE receives RRC Reconfiguration message corresponding to temporary capability, the UE starts to use new configuration (and the temporary capability). However, if the UE does not receive RRC Reconfiguration message, what’s the UE behavior? For MUSIM case, if the SIM B decides to enter RRC_CONNECTED from RRC_IDLE/RRC_INACTIVE, partial capability has to be occupied by the SIM B, so SIM A can only use restricted capability. Thus, similar to Rel-17 UE preference of leaving RRC_CONNECTED state for MUSIM purpose, if the UE does not receive RRC Reconfiguration message for a period of time, the UE is allowed to use the temporary updated capability autonomously.

Proposal 3: After the UE transmits the UAI message for temporary capability restriction due to MUSIM operation, if the UE does not receive RRC Reconfiguration message for a period of time, the UE is allowed to use the temporary updated capability autonomously.

Conclusion

In this contribution, we discuss the UE capability update and provide the following proposals.

Proposal 1: For the scenarios where the MUSIM UE operates in RRC_CONNECTED state simultaneously in two networks, a procedure by which the UE can inform the NW A of the update on the capability of maximum UL/DL MIMO layer, will be specified. It is FFS whether there is the need for the UE to indicate the other capability update via this procedure.

Proposal 2: UE assistance information framework shall be used for UE temporary capability update procedure for MUSIM operation.

Proposal 3: After the UE transmits the UAI message for temporary capability restriction due to MUSIM operation, if the UE does not receive RRC Reconfiguration message for a period of time, the UE is allowed to use the temporary updated capability autonomously.

References

- 3GPP RP-220955, “Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR,” Vivo, RAN#95e, Electronic Meeting, March 17 – 23, 2022.

- R2-2210017, Applicable scenarios for R18 MUSIM WI, Huawei, HiSilicon.