3GPP TSG-RAN WG2 Meeting #119bis Electronic	R2-2210070

Elbonia, 10 – 19 October 2022

Agenda item:	8.17.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	UE Architecture, Assumptions, and Primary scenarios for Dual TX/RX MUSIM operation

WID:	MUSIM-enh - Release 18

Document for:	Discussion and Decision

# Introduction

Rel-18 WID for MUSIM enhancements has the following objectives list in WID [1].

Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4].

- Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose

- RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR.

- Applicable UE architecture: Dual-RX/Dual-TX UE

The work item shall identify whether the WI will have RAN3 or RAN4 impacts by RAN#99 [RAN2].

# 2 Discussion

## 2.1 	UE Architecture and basic assumptions

A MUSIM UE capable of dual TX/RX can support two RRC connections simultaneously in NW-A and NW-B. This operation is also referred to as DSDA (Dual SIM Dual Active) mode in UE implementation-based MUSIM operation. In this case, the available RF resources such as a number of TX/RX chains, and antenna ports are shared between the USIMs. As the overall device processing power and capability remain the same irrespective of the number of RRC connections supported at the device, the capability needs to be shared between the RRC connections. The capabilities such as the number of carriers for CA operations, band combination for CA operations, and DC operations available per RRC connection will have to be reduced when the device operates with more than one RRC connection simultaneously.

Observation 1: In DSDA MUSIM operation many of the device capabilities need to be divided across two RRC connections. The capability available for each RRC connection at any time depends on the resources/capability consumed by other RRC connections.

When the resources required for each RRC connection of MUSIM operation is within the maximum limit of the UE capability, it is possible to establish and maintain these RRC connections without any additional changes in the specification. It is also possible that a connection can be started at NW-B without any signaling towards NW-A if the available resource at UE does not restrict this connection setup.

Observation 2: MUSIM operation with simultaneous RRC connection to two NW is possible already without new signaling procedures for many scenarios.

When all the available radio resources of the MUSIM device are assigned across the RRC connections for DSDA operation, if one of the RRC connection requires additional resources, the USIM of other RRC connection should coordinate with its network for release of resources or restriction of capability. For this coordination, new signaling procedure or change of existing MUSIM procedure is needed.

Observation 3: When either of the RRC connections require additional resources beyond the UE capability, the MUSIM signaling procedure is required for switching the UE resources (capability) from NW-A to NW-B.

The work item objective assumes that a new signaling procedure is needed at NW-A where capabilities need to be restricted and relaxed based on the activity at the W-B RRC connection. Here the RRC signaling connection and procedures at NW-B use the existing procedures and changes will be specified for NW-A operation.

Proposal 1:  RAN2 to confirm that the WID scope is to define specification changes for NW-A behaviour only. As per WID description, NW-A is NR-SA or NR-DC.

Even though the WID scope indicates the definition of new mechanism to restrict and remove restrictions at NW-A for start and stop of connection at NW-B, there could be different interpretation possible for start/stop operation at NW-B.  Whether it is start/stop of RRC connection or start/stop of operation at specific secondary cell/cell group is not clear from the description. So we propose to clarify that the NW-B operation referred in the WID objective is RRC connection setup and Release.  Because this is the basic operation needed for the UE to enter into DSDA operation.

Proposal 2:  RAN2 to confirm that NW-B operation indicated in WID scope refers to RRC connection setup and Release at NW-B.  It can also refer to resuming and releasing of RRC connection in an RRC-INACTIVE state.

### 2.2 Scenarios

For MUSIM UE, many of the UE capabilities related to radio resources such as carrier aggregation capability, and MIMO capabilities are shared across the RRC connections of MUSIM operation. Updating of every capability information dynamically to the network will increase the signaling load by a considerable amount. Moreover, many features supported by UE capability are temporarily restricted and removed only for short duration. Notification of such changes to other NW is not necessary for the basic functionality.  The WID scope is to define mechanism to enable RRC connection setup and release at NTWK-B. So, identification of primary scenarios relevant for this objective is important.

Scenario A:  Change capability related to CA;

Use case A1: Release of the secondary cell at NW-A for RRC connection setup at NW-B

MUSIM UE is in RRC-CONNECTED state in NW-A and operating in CA with a set of component carriers with specific band combinations. UE attempts to set up an RRC connection in NW-B and the maximum capacity in terms of the number of carriers is already in use at NW-A.  In this case, for the RRC connection to start in NW-B, at least one secondary cell needs to be released in NW-A. Here, the UE should trigger a new procedure to request NW-A to release a secondary cell to start the connection at NW-B.

Use case A2:  Release of the secondary cell at NW-A to increase the number of secondary cells at NW-B.

In this scenario, the resource requirement for the current user plane operation may need additional resources at NW-B. But it is also important to note that the release of further resources in NW-A will impact the performance at NW-A.  Hence, the MUSIM signaling procedure for this scenario is not essential as it is related to increasing the capability at one network not the basic entry.

Use case A3: Deactivation of secondary cells to allow RRC connection in NW-B

This is similar to scenario A but with the possibility for UE to maintain the secondary-cell configuration in a deactivated state for one network to allow RRC connection in the other network. In this case, even though the actual capability used by UE is within the maximum capability, the stored configuration is beyond the maximum capability. In this type of situation, MUSIM UE should support this additional functionality on preserving the configuration and also fast reactivation. Support for this solution may also impact lower layer signaling at the network side.

Scenario B:  Change capability related to DC;

Use case B1: Release of Cell-Group of DC in NW-A for RRC connection at NW-B

In NW-A, UE is in RRC connected with dual connectivity. In this case, both TX/RX resources are already assigned for MCG and SCG operations at NW-A. When new RRC connection is needed at NW-B one of the TX/RX resource needs to be released. In some scenarios, the RRC connection setup at NW-B will require the release of MCG at NW-A so that the MUSIM operation will operate within the supported band combination of UE. As an example, if the UE operates in DC for one band-combination (Nx-Ny) where Nx is the operating band for MCG and Ny is the operating band for SCG in NW-A and if the UE needs RRC connection at band Nz, where the UE capability only supports Nz-Ny combination and not Nz-Nx combination, the MCG needs to be released to allow the RRC connection in NW-A.

Use case B2: Deactivation of Secondary Cell-Group for RRC connection in NW-B

Similar to B1, the UE should be capable of operating with more than 2 Cell-group configurations with one cell-group configuration maintained in a deactivated state. As the deactivated state operation of cell group still requires some RX resources to maintain the minimum operation, the UE may need additional capability at least in the downlink to share the RX resources between two cell groups.

Scenario C: Indication of preference for NW-A operating carriers/bands for mobility for DSDA operation

UE supporting Dual TX/RX can support simultaneous RRC connection with NW-A and NW-B only for specific band combinations allowed by UE capability. If the NW-A switches the operating carrier to a different frequency, it may affect the overall band combination and UE needs to release the RRC connection at NW-B. For this scenario, an indication of list of carriers for switching during mobility will be beneficial.

Among the above scenarios, A1 and B1 can be considered without additional UE capability changes. A3 and B2 are beneficial for faster resuming of resources at NW-A after NW-B operation but require additional capability at UE for storing configurations beyond actual UE capability. Scenario 3 is related to UE indicating its preference of operating band /carrier to assist the connected mode mobility decisions to maintain both RRC connections.

Proposal 3: RAN2 to prioritize the release of a secondary cell or secondary-cell group for RRC connection setup in NW-B as the main scenario for defining the new signaling procedure.

Proposal 4: The addition of secondary cells to the existing RRC connection at NW-B is not considered as a  scenario for defining the basic signaling procedure.

Proposal 5: RAN2 to discuss additional capability and support needed for deactivation of secondary-cells /secondary-cell group at NW-A before considering these scenarios for signaling procedure.

Proposal 6: RAN2 to consider inter-frequency mobility in NTWK-A when NTWK-B RRC connection is active as a scenario for Rel-18 MUSIM signaling procedure.

# Summary

In this document we analyse the Dual TX/RX UE architecture and the basic assumptions for DSDA MUSIM operation with capability sharing. We propose some clarification on the WID scope to better define the scenarios and

We also analyse the main scenarios to be considered for developing the signalling procedure. Based on the analysis we make the following observations and proposals.

Assumptions and Scope Clarification

Observation 1: In DSDA MUSIM operation many of the device capability needs to be divided across two RRC connections. The capability available for single RRC connection at any time depends on the resources/capability consumed by other RRC connection.

Observation 2: MUSIM operation with simultaneous RRC connection to two NW is possible already without new signaling procedures for many scenarios.

Observation 3: When either of the RRC connections requires additional resources beyond the UE capability, the MUSIM signaling procedure is required for switching the UE resources (capability) from NW-A to NW-B.

Proposal 1:  RAN2 to confirm that the WID scope is to define specification changes for NW-A behavior only. As per WID description, NW-A is NR-SA or NR-DC.

Proposal 2:  RAN2 to confirm that NW-B operation indicated in WID scope refers to RRC connection setup and Release at NW-B.  It can also refer to resuming and releasing of RRC connection in an RRC-INACTIVE state.

Scenarios

Proposal 3: RAN2 to prioritize the release of a secondary cell or secondary-cell group for RRC connection setup in NW-B as the main scenario for defining the new signaling procedure.

Proposal 4: The addition of secondary cells to the existing RRC connection at NW-B is not considered as a scenario for MUSIM operation in Rel-18.

Proposal 5: RAN2 to discuss additional capability and support needed for deactivation of secondary-cells /secondary-cell group at NW-A before considering these scenarios for signaling procedure.

Proposal 6: RAN2 to consider inter-frequency mobility in NTWK-A when NTWK-B RRC connection is active as a scenario for Rel-18 MUSIM signaling procedure.

# References

- RP-220955 Revised WID:  Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, RAN meeting #95e, Release 18.