3GPP TSG-RAN WG2 Meeting #119bis	R2-2210113

# Electronic meeting, 10 – 19, October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1424 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on resource re-selection in IUC scheme 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, TS 38.321 5.22.1.2b is specifying UE-B's resource re-selection behaviours when UE-B received resource conflict indication and the conflicted resource is indicated by the physical layer as specified in clause 8.1.4B of TS 38.214. However, clause 8.1.4B of TS 38.214 is talking about UE-A's behaviours on how to determine a resource conflict for UE-B and will not report anything to higher layers. Actually, specifying UE-B's physical layer behaviours on determining a conflicted resource and reporting it to higher layers is captured in TS 38.213 clause 16.3.1 as follow: TS 38.213 16.3.1 A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by inter-UECoordinationScheme2, attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers - if slotLevelResourceExclusionScheme2 is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A - if slotLevelResourceExclusionScheme2 is provided, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change "8.1.4B of TS 38.214 [7]" to "16.3.1 of TS 38.213 [6]" Impact Analysis Impacted functionality: Sidelink resource allocation mode 2. Inter-operability: There is no inter-operability issue between UE and network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong referring to PHY layer spec, grant re-selection for using a received resource conflict indication can not be correctly performed by UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 5.22.1.2b	Re-selection for using a received resource conflict indication

If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on full sensing, or partial sensing or random selection or any combination(s), the MAC entity shall for each Sidelink process:

1>	if sl-interUECoordinationScheme2 enabling reception/transmission of a resource conflict indication is configured by RRC; and

1>	if the next resource of the selected sidelink grant which has been indicated by a prior SCI is overlapped with conflict resource(s) indicated by the physical layer as specified in clause 16.3.1 of TS 38.213 [6]:

2>	remove the resource from the selected sidelink grant associated to the Sidelink process;

2>	randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] excluding the conflict resource(s) for the removed resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9];

NOTE 1:	If retransmission resource cannot be selected by ensuring that the resource can be indicated by the time resource assignment of a prior SCI, how to select the time and frequency resource for more transmission opportunities from the available resources is left for UE implementation by ensuring the minimum time gap between any two selected ‎resources in case that PSFCH is configured for this pool of ‎resources.

2>	replace the removed resource by the selected resource for the selected sidelink grant.

NOTE 2:	It is left for UE implementation to reselect any pre-selected but not reserved resource(s) other than the resource overlapping with the conflict resource(s) indicated by the physical layer during reselection triggered by the conflict resource(s) indicated by the physical layer.

NOTE 3:	It is up to UE implementation whether and how to set the resource reservation interval in the re-selected resource to replace the resource overlapping with the conflict resource(s) indicated by the physical layer.

End of change