3GPP TSG RAN WG2#119bis-e	R2-2210159

Online: October 10th – 19th, 2022

Agenda Item:	8.7.4

Source:	CMCC

Title:	Cell reselection enhancements

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions about cell reselection enhancements and some agreements were reached as follows [1]:

Agreements:

- RAN2 to work on a solution so that measurements for TN’s coverage are performed only when relevant (FFS what relevant means).

- RAN2 to work on assistance information that can be provided to NTN UEs for the above.

- Cell reselection enhancements (for both NTN-NTN and NTN-TN mobility) are considered for both Earth-moving and (quasi-)Earth-fixed scenarios, at least via the use of system information for broadcasting necessary parameters (dedicated signalling is not precluded). FFS whether the same or different solutions are used for Earth-moving and (quasi-)Earth-fixed scenarios

However, in R17 late stage, location-based reselection solution does not converge. Therefore, in this contribution, we would like to provide some considerations for R17 left issue and R18 reselection enhancements.

Discussion

## 2.1 R17 left issue about location-based reselection solution down selection

As known, we have discussed location based cell reselection method for intra-NTN in late R17 without convergence. And there are 3 candidate options as follows:

Option 1: Introduce a distance threshold.

- Cell ranked on R-criterion first and then the distance threshold applies to down scope the candidate cells for reselection.

-  Step 1: UE performs cell ranking based on the R-criterion.

-  Step 2: Among the highest ranked N cells:

- For cells provided with reference location: only those whose distance to UE shorter than the distance threshold will be considered by UE as candidate cells.

- For cells not provided with reference location:

Alt.1: Not considered as candidate cell for reselection

Alt.2: Considered as candidate cell for reselection

- Step 3: Among all the candidate cells decided by on the distance threshold in step 2, UE reselect to the highest ranked cell based on R-criterion.

Option 2: Introduce a distance threshold.

- Distance threshold applies to decide the candidate cells and then rank the candidate cells based on R-criterion to decide the target cell for reselection.

- Step 1:

- For cells provided with reference location: UE evaluate the distance to neighbor cell reference location and only consider cells whose distance to UE are shorter than the threshold to be candidate cells for cell ranking

- For cells not provided with reference location:

Alt.1: Not considered as candidate cell for reselection

Alt.2: Considered as candidate cell for reselection

- Step 2: UE perform cell ranking on candidate cells decided in step 1 according to R-criterion.

- Step 3: UE reselect to the highest ranked cell.

Option 3: Cell ranked on R-criterion first and then the distance criteria apply to decide the target cell for reselection.

- Step 1: UE perform cell ranking based on the R-criterion.

- Step 2: Among the highest ranked N cells:

- For cells provided with reference location, UE reselect to the cell with the smallest distance to the cell’s reference location.

- For cells not provided with reference location, UE reselect to the highest ranked cell based on R-criterion.

To start power consumption reduction for R18, maybe we could consider down selection one solution among the above 3 options for intra-NTN firstly. Based on the comments among companies, option 2 could help UE to reduce measurement with the candidate neighboring cells narrowed down, and alt.2 is better to support due to that NW could control the neighbor cell provided to UE, that is to say if a neighbor cell is not to be a candidate cell for UEs in current cell, NW should not broadcast it. However, from our perspective, if alt.2 is supported, distance between a given UE and the reference location of a broadcast candidate cell may be larger than the threshold, if the cell is provided without reference location, the UE is not able to judge the distance criteria, which may lead to invalid measurement. Therefore, option 2 with alt.1 may be more reasonable.

Proposal 1: We sincerely suggest RAN2 to consider down selection one solution among the above 3 options for intra-NTN reselection power consumption reduction.

Proposal 2: It is proposed to support option 2 with alt.1 to enhance the cell reselection procedure for intra-NTN considering power consumption reduction.

## 2.2 cell reselection for earth moving cell

Then regarding the reselection solution for earth moving cell scenario, enhancements for quasi-earth fixed cell in R17 could be discussed firstly. However, the main difference from quasi-earth fixed cell is that the beam moves with the satellite in earth moving cell. Then, for timing based solution, the timing information on when a cell is going to stop serving the area, i.e. t-service, should be per UE, and needs to calculate by each UE in the cell, which may be more complex compared to the quasi-earth fixed cell.

And for location-based solution, evaluate the distance between UE and serving cell reference location whether shorter than a threshold consider to help trigger neighbour cell measurements is also benefit for moving cell. Nevertheless, the reference location in a moving cell would change along with the satellite beam. Therefore, for earth moving cell, considering that the moving trajectory of the satellite is able to predict with the ephemeris information, and the reference location of the cell could be deduced by combining the moving trajectory of the satellite and satellite coverage information together.

Proposal 3: For cell reselection enhancements for earth moving cell, solutions for quasi-earth fixed cell in R17 could be discussed firstly.

Proposal 4: For timing based solution, the timing information on when a cell is going to stop serving the area, i.e. t-service, should be per UE, and needs to calculate by each UE in the earth moving cell, which may be more complex compared to that in the quasi-earth fixed cell.

Proposal 5: For location-based solution, the reference location of the earth moving cell could be deduced by combining the moving trajectory and the coverage information of the satellite together.

## 2.3 power consumption reduction for cell reselection

Furthermore, another R18 WID objective is to specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized).

As we know, a NTN cell with a large coverage, for example decuple TN cell’s coverage, may be adjacent with substantive neighbor TN cells. However, from a given UE perspective, only partial TN neighbor cells are reachable to it, demanding cell search and/or measurement, as depicted in Fig. 1. Another more extreme case, maybe only partial boundary of NTN cell has TN neighbor cells as depicted in Fig. 2(e.g. partial boundary of NTN cell is close to the ocean, desert, etc.). Therefore, to reduce UE power consumption due to  unnecessary TN neighbor cell search and measurements beyond its reachable scope, it is beneficial that the network delivers available TN neighbor cell information to the UE. And according to the last meeting agreements, we need further discuss the FFS what relevant means. For example, we could consider to provide reference location of TN neighbor cell, cell coverage radius of TN neighbor cell, relative position to the current NTN cell, etc. to help UE choose suitable TN neighbor cell. Then for another FFS issue whether the same or different solutions are used for Earth-moving and (quasi-)Earth-fixed scenarios, regarding the   TN neighbor cell measurements reduction, maybe it does not have difference between Earth-moving and (quasi-)Earth-fixed scenarios due to that the main issue is to provide assistance information about TN neighbor cell.

Fig.1 UE has only limited reachable TN neighbor cells    Fig.2 Only partial boundary of NTN cell has TN neighbor cells

Proposal 6: It is proposed to provide reference location of TN neighbor cell, cell coverage radius of TN neighbor cell, relative position to the current NTN cell, etc. to help UE choose suitable TN neighbor cell.

Proposal 7:Regarding the TN neighbor cell measurements reduction, maybe it does not have difference between Earth-moving and (quasi-)Earth-fixed scenarios due to that the main issue is to provide assistance information about TN neighbor cell.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on cell reselection enhancements and have the following proposals:

Proposal 1: We sincerely suggest RAN2 to consider down selection one solution among the above 3 options for intra-NTN reselection power consumption reduction.

Proposal 2: It is proposed to support option 2 with alt.1 to enhance the cell reselection procedure for intra-NTN considering power consumption reduction.

Proposal 3: For cell reselection enhancements for earth moving cell, solutions for quasi-earth fixed cell in R17 could be discussed firstly.

Proposal 4: For timing based solution, the timing information on when a cell is going to stop serving the area, i.e. t-service, should be per UE, and needs to calculate by each UE in the earth moving cell, which may be more complex compared to that in the quasi-earth fixed cell.

Proposal 5: For location-based solution, the reference location of the earth moving cell could be deduced by combining the moving trajectory and the coverage information of the satellite together.

Proposal 6: It is proposed to provide reference location of TN neighbor cell, cell coverage radius of TN neighbor cell, relative position to the current NTN cell, etc. to help UE choose suitable TN neighbor cell.

Proposal 7:Regarding the TN neighbor cell measurements reduction, maybe it does not have difference between Earth-moving and (quasi-)Earth-fixed scenarios due to that the main issue is to provide assistance information about TN neighbor cell.

Reference

[1] RAN2-119-e - NR-NTN-IoT-NTN-REDCAP-CE_EOM