3GPP TSG-RAN WG2 Meeting #119bis-e	R2-2210178

Online, Oct 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3528 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on conditionalReconfiguration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current NR RRC spec, it’s specified that there are two conditionalReconfiguration and VarConditionalReconfig in NR-DC, one associated with the MCG and one associated with the SCG, to avoid the conditional reconfiguration ID collision between the MN generated configuration (for inter-SN CPC and CHO) and the SN generated configuration (for intra-SN CPC). And the procedural text related to the handling of conditional reconfiguration in section 5.3.5.3, 5.3.5.4, 5.3.7.3, 5.3.10.5, 5.3.8.3 and 5.3.11, is indicated which variable(s) is concerned, e.g. for the MCG or/and the SCG VarConditionalReconfig. However, the procedural text on the handling of conditional reconfiguration in the current spec is not only applicable to NR-DC case, but also for NR SA (e.g. for CHO) and EN-DC (e.g. for inter-SN and intra-SN CPC) cases. In NR SA and EN-DC, it’s not defined what does the MCG/SCG VarConditionalReconfig refer to, which may cause some ambiguity on the implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the definition of the MCG and the SCG conditionalReconfiguration and VarConditionalReconfig in section 5.3.5.13.1, to clairify that such definition is applicable to all NR involved cases where MCG/SCG VarConditionalReconfig is received, i.e. not only in NR-DC. Impact analysis Impacted 5G architecture options: NR SA, EN-DC, NR-DC Impacted functionality: CHO, CPAC Inter-operability: If the network implements the CR and the UE does not, there may be different interpretations on what does the MCG/SCG VarConditionalReconfig refer to. If the UE implements the CR and the network does not, there may be different interpretations on what does the MCG/SCG VarConditionalReconfig refer to. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on the MCG/SCG VarConditionalReconfig remains, which may cause different interpretations on the handling of MCG/SCG VarConditionalReconfig. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# START OF CHANGE

#### 5.3.5.13	Conditional Reconfiguration

##### 5.3.5.13.1	General

The network configures the UE with one or more candidate target SpCells in the conditional reconfiguration. The UE evaluates the condition of each configured candidate target SpCell. The UE applies the conditional reconfiguration associated with one of the target SpCells which fulfils associated execution condition. The network provides the configuration parameters for the target SpCell in the ConditionalReconfiguration IE.

The UE may receive one or two independent conditionalReconfiguration:

-	a conditionalReconfiguration, associated with MCG, that is included in the RRCReconfiguration message received via SRB1; and

-	a conditionalReconfiguration, associated with SCG, that is included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message or embedded in an E-UTRA RRCConnectionReconfiguration message received via SRB1.

In case that the UE receives two independent conditionalReconfiguration (UE in NR-DC):

-	the UE maintains two independent VarConditionalReconfig, one associated with each conditionalReconfiguration;

-	the UE independently performs all the procedures in clause 5.3.5.13 for each conditionalReconfiguration and the associated VarConditionalReconfig, unless explicitly stated otherwise;

-	the UE performs the procedures in clause 5.5 for the VarConditionalReconfig associated with the same cell group like the measConfig.

The UE performs the following actions based on a received ConditionalReconfiguration IE:

1>	if the ConditionalReconfiguration contains the condReconfigToRemoveList:

2>	perform conditional reconfiguration removal procedure as specified in 5.3.5.13.2;

1>	if the ConditionalReconfiguration contains the condReconfigToAddModList:

2>	perform conditional reconfiguration addition/modification as specified in 5.3.5.13.3;

# END OF CHANGE