3GPP TSG-RAN WG2 #119bis-e 	R2-2210191

Electronic meeting, 10th –19th October 2022

Agenda Item:	8.5.4.1 Feedback enhancements

Source:	NEC

Title:	Feedback Enhancements for Capacity Improvement

Document for:	Discussion, Decision

# Introduction

A new Rel-18 study on XR Enhancements for NR was agreed at RAN#94e meeting [1]. One of the objectives is XR-specific capacity improvement:

Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

And at RAN2 #119e meeting, it is agreed:

1: As starting point, RAN2 can further discuss the solutions in TR 38.838 that can impact on L2 operation (e.g., BSR, LCP, assistance information for scheduling, packet discarding, prioritization) for XR-specific capacity improvement. RAN2-specific solutions are not precluded (even if RAN1 hasn’t discussed them before). 2: Enhancement to SPS/CG should be justified for XR scheduling and should be evaluated against dynamic grant (DG) scheduling which should be considered as baseline. Should justify why enhancements are needed. 3: RAN2 considers SPS enhancements may not be needed in Rel-18 XR since PDCCH capacity is not assumed to be a problem for XR. FFS if SPS has some power consumption benefits.

This contribution provides our initial consideration on which additional feedback could be useful for XR-specific capacity improvements.

# Discussion

## UL XR traffic characteristics feedback

For signal stream UL traffic model, according to [2], key parameters are packet size, packet arrival, packet delay budget, packet success rate requirement. Different from single stream DL traffic, no jitter for the UL traffic and periodicity seems to be an integer ([2] gives an example of 4ms), which can be properly supported by ConfiguredGrantConfig periodicity and periodicityExt fields (i.e., integral multiple of slot duration with the maximum value of 640 ms). Therefore, no impacts on the resource allocation and scheduling are foreseen at this moment.

However, to enable gNB to configure type 1 or type 2 Configured Grant in UL, Input of the XR service characteristics would be useful for gNB e.g., periodicity and start of periodicity, potential packet size, potential jitter range and so on. UE is aware of this information better then network side entities. we can consider that UE report the XR traffic characteristic to gNB

Two options to report UL XR traffic characteristics feedback:

- explicit way: report XR traffic characteristics directly, may including periodicity, start offset, packet size and so on. This information can be sent as UL Assistance Information, gNB may use this to configure proper CG

- Implicit way: UE could ask for a certain CG, including the periodicity, offset and TB size of the CG, this could be sent with a MAC CE

Both options may have pros and cons, RAN2 can further discuss which direction to go

RAN2 agree to introduce UL XR traffic relevant feedback.

If P1 is agreeable, one of following ways could be considered to report XR traffic relevant feedback:

- a) explicitly report XR traffic characteristics

- b) implicitly report by asking for desired CG for UL XR transmission

## BSR enhancements

BSR is a very important input for gNB scheduler to make dynamic schedule decision. However, currently, BSR mechanism may not be sufficient for XR type of traffic:

- Regular BSR will not be triggered when more data arrive in buffer for the same LCH/LCG.

- Padding BSR may be transmitted only if there are enough padding bits available, because padding BSR has lower priority then data. Periodical BSR may not be applicable for burst data arrival unless very short periodicity is configured which does not sound reasonable

- The BSR report become less precise with higher BSR index/buffer size value. e.g., the mean packet size of video is 62500 bytes, for a packet with mean packet size, according to following table copied from TS38.321, with 5 bits buffer size field, UE will report BS index 28 (≤ 77284 but > 55474), with 8 bits buffer size field, UE reports BS index is 139 (<=62599 but > 58784 in bytes) (refer to 38.321 for the 8bits table);

Table: Buffer size levels (in bytes) for 5-bit Buffer Size field from 38.321

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 8 | ≤ 102 | 16 | ≤ 1446 | 24 | ≤ 20516 |
| 1 | ≤ 10 | 9 | ≤ 142 | 17 | ≤ 2014 | 25 | ≤ 28581 |
| 2 | ≤ 14 | 10 | ≤ 198 | 18 | ≤ 2806 | 26 | ≤ 39818 |
| 3 | ≤ 20 | 11 | ≤ 276 | 19 | ≤ 3909 | 27 | ≤ 55474 |
| 4 | ≤ 28 | 12 | ≤ 384 | 20 | ≤ 5446 | 28 | ≤ 77284 |
| 5 | ≤ 38 | 13 | ≤ 535 | 21 | ≤ 7587 | 29 | ≤ 107669 |
| 6 | ≤ 53 | 14 | ≤ 745 | 22 | ≤ 10570 | 30 | ≤ 150000 |
| 7 | ≤ 74 | 15 | ≤ 1038 | 23 | ≤ 14726 | 31 | > 150000 |

To improve capacity, it is very important to make gNB scheduler have a precise BSR estimation especially when it is about to finish the scheduling of all data. Only when the UE buffer size estimation is close to the actual UE buffer, gNB could schedule a right grant size which efficiently fit for data volume. Otherwise, it will end up with more padding bits if the grant is too big, or further latency when the grant is smaller than the data size and an additional scheduling has to be followed.

There are several directions to enhance BSR to improve the BSR estimation at gNB side:

- Solution direction1: increase the BSR report granularity, e.g.:

  - define a new buffer size level mapping table which can provides a finer BSR granularity in general

  - define a new buffer size level mapping table which is tailored particularly for XR traffic. As an example, a finer granularity could be provided around the typical/comment packets size (size range)

- Solution direction 2: introduce new BSR report triggers, which will provide more up to date BSR information to gNB . as we mentioned at beginning, since BSR will not always triggered and update when the buffer size changes e.g., due to new fresh data arrival or due to part of the data has been transmitted, the estimation of the buffer size at gNB become less precise especially if the previous BSR indicates a big BS value range. So the solution is to report more BSR. The trigger should be designed carefully for efficiency. Some examples are:

  - BSR is requested by gNB when needed

  - BSR is triggered when large grant have been given but more data is still in buffer.

In our understanding, both solution directions could be considered together, especially at study phase.

RAN2 agree to enhance BSR to improve the accuracy of Buffer size information

If P3 is agreeable, BSR report can be enhanced in following two directions:

- Increase the BSR report granularity

- Introduce new BSR report triggers

# Summary

This contribution provided our initial proposals on feedback enhancements as follows:

- RAN2 agree to introduce UL XR traffic relevant feedback.

If P1 is agreeable, one of following ways could be considered to report XR traffic relevant feedback:

- a) explicitly report XR traffic characteristics

- b) implicitly report by asking for desired CG for UL XR transmission

RAN2 agree to enhance BSR to improve the accuracy of Buffer size information

If P3 is agreeable, BSR report can be enhanced in following two directions:

- Increase the BSR report granularity

- Introduce new BSR report triggers

# References

[1] RP-213587	New SID “Study on XR Enhancements for NR”

[2] TR38.838 V17.0.0