3GPP TSG-RAN WG2 #119-bis-e	R2-2210318

e-Meeting, October 10th – 19th, 2022

Agenda Item:	8.2.4

Source:	Ericsson

Title:	LPHAP and Text Proposal for TR

Document for:	Discussion, Decision

# 1	Introduction

Inactive mode positioning was specified in Rel-17. Rel-18 WID provides the below objective

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1]

    - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study.

    - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

# 2	Discussion

In RAN2#119e, the following was agreed:

Agreements:

Proposal 1: RAN2 shall restrict the use case for any LPHAP discussions in RAN2 to Tracking of workpiece (in- and outdoor) in assembly area and warehouse (Use case # 6 in Table A.7.2-1 in TS 22.104).

RAN2 to consider at least the ‘Low Power Periodic and Triggered 5GC-MT-LR Procedures’ in TS 23.273.  Other procedures are not excluded from discussion.

RAN2 shall wait for RAN1 conclusions from evaluations on UE power consumption with respect to baseline functionality and whether enhancements are needed.  RAN2 will study potential areas for higher layer enhancements that may result in reduction of UE power consumption.

## 2.1	Multi-Cell UL-SRS Interference, Capacity Loss & TA

If UE roams in multiple cells transmitting UL-SRS, it is bound to cause interference in other cells. If in serving cell; resources were configured in PRB1 it would imply in other cells the UE UL-SRS transmission will interfere to other UE which also uses PRB1. To avoid this, it has been mentioned to reserve resource or to have pool of UL-SRS resources. However, this would lead to capacity loss. Implicitly for above example, PRB1 in other cells would not be used or would be blocked.

UL-SRS Resource pool would lead to capacity loss.

Another source of interference would be the sequence ID. UL-SRS signals are generated using sequence ID and cell-ID. If UE is allowed to transmit UL-SRS while roaming in multiple cells; this would imply the UL-SRS resource pool should also reserve the sequence ID.

UL-SRS Resource pool would not only require reserving time/frequency resource but also other resource such as sequence ID.

It has been suggested that UE will receive TA update based upon RACH procedure. If UE RSRP fluctuates by certain margin or TA timer expires; UE will have to perform RACH procedure and obtain TA. However, TA obtained from RACH procedure (Preamble) is not accurate as compared to UL-SRS based TA. A UE which is unsynchronized and transmit with an offset would cause interference to other UEs and at the same time the RTOA measurements performed by base station would have timing errors as UE’s UL-SRS Tx timing may not be aligned. The objective of the study item however is high accuracy.

Preamble based TA update would lead to large positioning error. The objective however is High Accuracy.

Hence a solution where NW can push the TA would be needed. This can be supported by means of MT-SDT procedure or configuring occasional PDCCH monitoring after UL-SRS transmission.

Figure X: Configuring PDCCH Monitoring after UL-SRS Transmission

Send an LS to RAN1 to capture observations 1 to 3.

- UL-SRS Resource pool would lead to capacity loss.

- UL-SRS Resource pool would not only require reserving time/frequency resource but also other resource such as sequence ID.

- Preamble based TA update would lead to large positioning error. The objective however is High Accuracy

Occasional PDCCH monitoring is configured to provide accurate TA to improve positioning accuracy.

Further, there will be problem in terms of configuring the TRPs for the multi-SRS validity area; several RAN nodes will be listening to UE UL SRS when UE moves in multiple cells with UL-SRS Tx. This calls for overprovisioning of TRPs and consumption of network resources. Since NW does not know where UE will pop up, this will lead to extra processing and signalling cost that will impact several RAN3 interfaces: XnAP, NRPPA and F1AP

As UE may pop up anywhere in the configured multiple cell area with UL-SRS Tx, it is not possible to know in which gNBs/TRPs the UE would finally resume in within the configured multiple UL SRS Tx area. Hence, over-provisioning of TRPs may have to be done where some of the TRPs must keep on listening even when nothing is heard. This leads to extra processing and signalling costs.

Send an LS to RAN3 asking on NW overhead and signalling impacts issue and if it is feasible to transmit UL-SRS in muti-cell during Inactive.

  - For UL-SRS multi-cell (in a validity area), can we assume the listening TRPs would always be same? Does NW need to over-provision (configure several TRPs where some of them simply must listen even when nothing is heard”?

Capture the Observation 1 to 4 in the TR as shown in the Annex (Text Proposal for TR).

## 2.2	Draft LS

3GPP TSG RAN WG2 #116bis-e	R2-22xxxx

Online, 17 – 25 January 2022

Title:	[Draft] LS on UL based Low Power High Accuracy Positioning

Release:	Rel-18

Work Item:	FS_NR_pos_enh2

Source:	Ericsson [to be RAN WG2]

To:	RAN1, RAN3

Cc:	-

Contact person:	Ritesh Shreevastav

Ritesh.shreevastav@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None)

# 1	Overall description

To RAN1:

RAN2 would like to know if it is possible to configure UL SRS resource which can be applicable for multiple cells in RRC Inactive state. Will there be problem because of interference or any other reason when UE uses configuration defined for one cell in another cell.

To RAN3:

Does RAN3 foresee additional processing and signalling overhead impacts due to the overprovision of several RPs, when some of the RPs may not be listening to UL-SRS when UE goes in RRC_INACTIVE to transmit multiple UL-SRS?

RAN2 respectfully asks RAN1 to provide the answer for above.

# 2	Actions

To RAN11

ACTION: 	RAN2 respectfully requests RAN1 to provide the answer for above.

# 3	Dates of next TSG RAN WG2 meetings

RAN2 Meeting #119bis-e	    October 10 – 18, 2022		Electronic Meeting

RAN2 Meeting #120               	    November 14-18,2022 		Canada

## 2.3	Enhanced Power Saving mechanisms

MICO state or no paging monitoring etc. requires SA2 involvement. There is not much from RAN2 perspective one may do. This should be discussed in SA2. We should note that we are positioning group and not UE power saving group. If any specific power saving such as RRM relaxation etc. needs to be discussed, then it should be separated from Positioning WI.

Positioning WI may not have specific competence in saving UE power. This should be discussed in relevant power saving WI and in SA groups; for example, for granting UE with eDRX, No paging monitoring or MICO state

Non-Positioning procedure related (e.g., communication related; MICO states, RRM relaxation, eDRX cycle) power saving enhancement is not discussed in Positioning WI.

## 2.4	Pre-configuring UL SRS based

UL SRS configuration for inactive depends upon several factors such as TA validity timer, RSRP thresholds, TA value and UL power to use for UL SRS Tx. It is beneficial if these configurations are provided as close as possible when UE is released to Inactive from connected mode. If configuration is provided well in advance, it may not be accurate as UE spatial relation or RSRP etc. may have changed.

UL SRS configuration is valid for a serving cell. In connected mode the NW is aware of UE’s situation: there are dedicated mode procedures and there is closed loop power control to minimize interference. Hence, even when UE drifts to neighbor cell the positioning may continue as NW may reconfigure the necessary parameters including TA, spatial relations, Tx Power etc. In RRC Inactive mode, it is unclear as how this would be maintained.

It is very risk prone, and it is likely UE will cause interference if UE performs cell reselection but happens to still reuse the resource from the previous cell.

SRS configuration for Inactive mode should be provided when UE is going to transit to Inactive mode; rather than preconfiguring it and should be applicable only when UE is in that cell. After cell reselection Rel-17 behaviour should be applicable.

SRS configuration in RRC Inactive mode is not valid to multiple cells.

## 2.5	RRC Idle mode positioning

In LTE, for NB-IoT RRC Idle mod positioning is already supported. The UE may receive AD in connected mode and can go to RRC Idle mode and perform the positioning measurements. The UE would transit to connected mode to provide the positioning measurements.

The same principle can be also applied to NR. The power saving is achieved by means of not being in RRC Connected mode for positioning. Further, the assistance data can also be obtained using posSIB.

In RRC Idle mode UE can obtain AD using posSIB and can perform measurement in RRC idle mode to save UE power.

Send an LS to RAN1 to define RSTD measurements for RRC idle mode.

When it comes to measurement reporting; our view is that even if UE was in RRC Inactive the Inactive mode data transfer may not work because of data volume threshold as positioning measurements are quite big. Hence, in terms of latency and power saving it may in fact be beneficial to transit to connected mode.

Positioning Measurement reporting in RRC Inactive would be inefficient as it may need subsequent transmission implying longer duration and prolonged battery consumption. RRC Connected mode can be efficient.

# Conclusion

In the previous sections we made the following observations:

Observation 1	UL-SRS Resource pool would lead to capacity loss.

Observation 2	UL-SRS Resource pool would not only require reserving time/frequency resource but also other resource such as sequence ID.

Observation 3	Preamble based TA update would lead to large positioning error. The objective however is High Accuracy.

Observation 4	As UE may pop up anywhere in the configured multiple cell area with UL-SRS Tx, it is not possible to know in which gNBs/TRPs the UE would finally resume in within the configured multiple UL SRS Tx area. Hence, over-provisioning of TRPs may have to be done where some of the TRPs must keep on listening even when nothing is heard. This leads to extra processing and signalling costs.

Observation 5	Positioning WI may not have specific competence in saving UE power. This should be discussed in relevant power saving WI and in SA groups; for example, for granting UE with eDRX, No paging monitoring or MICO state

Observation 6	SRS configuration for Inactive mode should be provided when UE is going to transit to Inactive mode; rather than preconfiguring it and should be applicable only when UE is in that cell. After cell reselection Rel-17 behaviour should be applicable.

Observation 7	In RRC Idle mode UE can obtain AD using posSIB and can perform measurement in RRC idle mode to save UE power.

Observation 8	Positioning Measurement reporting in RRC Inactive would be inefficient as it may need subsequent transmission implying longer duration and prolonged battery consumption. RRC Connected mode can be efficient.

	UL-SRS Resource pool would not only require reserving time/frequency resource but also other resource such as sequence ID.

	Preamble based TA update would lead to large positioning error. The objective however is High Accuracy

	To enable multi-cell SRS; NW may have to over-provision (configure several) TRPs where some of them must listen even when nothing is heard.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Send an LS to RAN1 to capture observations 1 to 3.

	UL-SRS Resource pool would lead to capacity loss.

	UL-SRS Resource pool would not only require reserving time/frequency resource but also other resource such as sequence ID.

	Preamble based TA update would lead to large positioning error. The objective however is High Accuracy

Proposal 2	Occasional PDCCH monitoring is configured to provide accurate TA to improve positioning accuracy.

Proposal 3	Send an LS to RAN3 asking on NW overhead and signalling impacts issue and if it is feasible to transmit UL-SRS in muti-cell during Inactive.

	For UL-SRS multi-cell (in a validity area), can we assume the listening TRPs would always be same? Does NW need to over-provision (configure several TRPs where some of them simply must listen even when nothing is heard”?

Proposal 4	Capture the Observation 1 to 4 in the TR as shown in the Annex (Text Proposal for TR).

Proposal 5	Non-Positioning procedure related (e.g., communication related; MICO states, RRM relaxation, eDRX cycle) power saving enhancement is not discussed in Positioning WI.

Proposal 6	SRS configuration in RRC Inactive mode is not valid to multiple cells.

Proposal 7	Send an LS to RAN1 to define RSTD measurements for RRC idle mode.

# References

RP-210903 , “Revised WID on NR Positioning Enhancements“, March 16 - 26, 2021

# Text Proposals for TR

Some observations for Multi-cell UL-SRS configuration are as below:

- UL-SRS Resource pool would lead to capacity loss.

- UL-SRS Resource pool would not only require reserving time/frequency resource but also other resource such as sequence ID.

- Preamble based TA update would lead to large positioning error. The objective however is High Accuracy

- To enable multi-cell SRS; NW may have to over-provision (configure several) TRPs where some of them must listen even when nothing is heard.

Some observations for Power Saving are as below:

- Non-Positioning procedure related (e.g., communication related; MICO states, RRM relaxation, eDRX cycle) power saving enhancement is not discussed in Positioning WI as Positioning WI as such may not have specific competence in saving UE power.