# 3GPP TSG RAN WG2 Meeting #119bis-e                     	    R2-2210363 E-meeting, 10 – 19 October 2022

Agenda item:	8.2.2

Source:	Qualcomm Incorporated

Title:	Study of Sidelink Positioning Architecture, Signaling and Procedures

WID/SID:

Document for:	Discussion and Decision

# Introduction

This contribution discusses architecture, functionality, and procedures for support of UE-based and network-based sidelink positioning and ranging to address the requirements in RP-222616 Revised SID on Study on expanded and improved NR positioning [1]. This contribution emphasizes protocol aspects of sidelink positioning, recognizing lower-layer aspects, including positioning reference signals, will be addressed by RAN1 working group.

RP-222616 Revised SID on Study on expanded and improved NR positioning 4.1 Objective of SI or Core part WI or Testing part WI [text omitted] - Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#98 to see if sufficient information is available for this review.

# Discussion

Sidelink Positioning for relative positioning, ranging and absolute positioning may include UE discovery, capability exchange, measurement configuration, and exchange of location information resulting from performed measurements. Depending on the deployment scenario and operational use case, sidelink positioning and ranging may occur between UEs, or between UEs and an LMF, potentially over multiple cast types and access technologies. To realize this capability in a manner satisfying the desired use cases (V2X, public safety, commercial and IIOT) and within the time frame and work scope of the release, RAN2 should study approaches minimizing and/or not requiring changes to the underlying sidelink transport.

Fundamental to sidelink positioning and ranging is UE peer-to-peer operation. As such, the current 5GS architecture, using network-based location servers (user plane location servers or control plane location servers), is not sufficient for sidelink positioning. To address this limitation UEs should be capable of calculating position and/or range based on sidelink positioning and ranging measurements, and support a UE-to-UE Sidelink Positioning Protocol (SLPP).

RAN2 #119-e reached the following agreements on sidelink positioning architecture, supported scenarios and protocol [2]:

Agreements on supported scenarios, architecture: - Confirm that for sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios shall be supported. FFS if partial coverage case assumes anything about which UEs are in coverage. - Study the architecture and signaling procedures to enable at least the following two operation scenarios: - Operation Scenario 1: PC5-only-based positioning. - Operation Scenario 2: Combination of Uu- and PC5-based positioning. - RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server. FFS from RAN2 perspective if there are cases without a UE in the location server role.

Agreements on Protocol: - Introduce a new protocol for sidelink positioning procedures between UEs (name FFS, e.g., RSPP, SLPP). FFS where it is specified. The new protocol is a separate ASN.1 module from LPP (this does not necessarily imply whether it is included in 37.355) - Study the potential impact to LPP for support of sidelink positioning procedures between UE and LMF. FFS how much impact (if any), e.g., only to carry the new protocol, and if the PC5-only and hybrid PC5+Uu cases are the same or different. - RAN2 wait for SA2 on the triggering of the positioning procedures from upper layers.

A possible architecture for sidelink positioning and ranging is shown in Figure 1 [3]. The illustrated architecture is for a non-roaming, same PLMN operation, but is readily extensible to inter-PLMN and roaming scenarios. This architecture supports sidelink positioning and ranging for in-coverage, out-of-coverage, or partial-coverage operation and is suitable for both PC5-only positioning and for joint PC5-Uu positioning. A Sidelink Positioning/Ranging Function (SPRF) provides SLPP support between UEs to configure measurements and exchange measurement results. The SPRF may also provide location and range computation on behalf of other UEs. For scenarios involving an LMF, through the SPRF, the LMF may request UE capability information, configure measurements, receive measurement results, and perform location and range computation for, and/or on behalf of, participating UEs. Two or more UEs may participate in a one-to-many or many-to-one sidelink positioning and ranging session using discovery and positioning protocols over the SR5 reference point. SR5 is carried over the PC5 reference point and provides the interface for discovery and SLPP positioning protocol between UEs.

Observation 1: A UE-based Sidelink Positioning and Ranging Function (SPRF) communicating over the SR5 reference point can be used to provide location and range computation and sidelink positioning and ranging protocol (SLPP) functionality.

Proposal 1: A UE supporting sidelink positioning and ranging should be capable of being enabled with the ability to calculate position and or range based on sidelink positioning and ranging measurements.

Figure 1: Reference architecture for Sidelink Positioning and Ranging-based services for non-roaming and same PLMN operation

## Transport and Protocol Layering

Transport mechanisms over the SR5 interface used for SLPP for sidelink positioning and ranging should provide sufficient flexibility to address all use cases and cast types. Over PC5, possible SLPP transport mechanisms RAN2 may consider for SLPP include the following:

- PC5-Signaling (PC5-S):

PC5-S enables sidelink positioning and ranging over a unicast link between two UEs. Each PC5-S unicast link establishment requires the exchange of 4 PC5-S messages followed by the exchange of 4 PC5-RRC messages, as shown in Figure 2 for the case of a UE (UE-1) establishing two separate unicast links, one unicast link with UE-2 and a second unicast link with UE-3. SLPP transport over PC5-S limits sidelink positioning and ranging to unicast, and excludes sidelink positioning support over broadcast/groupcast, resulting in a constrained solution excluding applications such as V2X from efficiently using sidelink positioning and ranging.

Figure 2: PC5-S Unicast link establishment

- PC5-Signaling enhanced to support non-unicast cast types:

RAN2 could enhance PC5-S to support SLPP over groupcast and broadcast. Such an enhancement would introduce significant flexibility to PC5-S, but would require substantial standards modification, a challenge that may be precluded considering the large scope of the current release.  This approach would also not enable sidelink positioning and ranging support for applications operating over currently deployed LTE PC5, including V2X.

- PC5 User Plane (PC5-U):

PC5-U transport enables UE-to-UE communication with no network involvement. Unlike PC5-S, PC5-U enables UE-to-UE communication between two or more UEs, supporting not only one-to-one communication for two UEs (unicast) but also one-to-many (broadcast/groupcast). Also, unlike PC5-S, PC5-U link establishment is more efficient allowing data transmission immediately without requiring multiple request-response exchanges between participating UEs, as is required for PC5-S.  This more efficient link PC5-U link establishment is illustrated in Figure 3 and Figure 4 for broadcast and groupcast, respectively.

PC5-U has a well-defined QoS management procedure [23.287], allowing the upper layer to specify the exact QoS requirements to the AS layer.  This is in contrast to PC5-S, which can only utilize default QoS settings because no ProSe or V2X QoS control is defined for PC5-S. PC5-U supports Quality-of-Service (QoS) through the mapping of PC5 QoS Flow Identifier (PFI) from the application layer to the AS layer, including Resource Type, Priority Level, Packet Delay Budget, Packet Error Rate [4], thus providing flexibility in specification of QoS for different use cases.

Figure 3: PC5-S Broadcast link establishment

Figure 4: PC5-S Groupcast link establishment

The comparative evaluation of possible PC5 transport mechanisms for SPRF SLPP over the SR5 reference point identifies the following key points:

- Both PC5-S and PC5-U support inter-UE SLPP communication without network involvement.

- PC5-S SLPP transport supports one-to-one unicast communication with multiple UE-to-UE request-response transactions required to establish a link.

- PC5-U SLPP transport supports one-to-one (unicast) and one-to-many (broadcast/groupcast).

- PC5-U does not incur the link establishment latency penalty associated with PC5-S multiple request-response unicast link establishment (no inter-UE messaging is required for groupcast/broadcast link establishment).

- PC5-U supports QoS via PFI as part of the Layer-2 link establishment.

- PC5-S SRBs are constrained to a default QoS

From the above analysis, it is evident transport over the PC5 user plane provides the greatest flexibility for sidelink positioning and ranging, with no modifications to the PC5 reference point needed.

Observation 2: SLPP transport over PC5-U supports in-coverage, partial-coverage and out-of-coverage sidelink positioning avoiding unnecessary link establishment latency, providing QoS and enabling flexibility in cast type.

A protocol layering for SLPP over PC5-U is shown in Figure 5. This protocol layering is suitable for inter-UE sidelink positioning and ranging operation for in-coverage, out-of-coverage and partial coverage. When a network LMF supporting a SPRF is a participant in a sidelink positioning and ranging session, a UE may send measurements to the LMF for the LMF to assist the UE in performing sidelink positioning and ranging, or for the LMF to compute the results of a sidelink positioning and ranging session. SLPP messages sent to, or received from the LMF SPRF can be sent using existing NAS without requiring any modification to the PC5 reference point or the Uu reference point, easily realized through the non-IP PDCP SDU type

Observation 3: Exchange of Sidelink Positioning and Ranging Function (SPRF) messaging can be accomplished without modification to the PC5 reference point over the PC5 user plane.

Proposal 2: RAN2 should support SLPP transport over PC5 User Plane to provide SLPP QoS and flexibility in cast type while introducing no change to the PC5 reference point.

Figure 5: SLPP protocol layering over PC5 reference point

## Architecture

At RAN2 #119-e, the following agreements related to the sidelink positioning architecture were made:

Study the architecture and signaling procedures to enable at least the following two operation scenarios:

-	Operation Scenario 1: PC5-only-based positioning.

-	Operation Scenario 2: Combination of Uu- and PC5-based positioning.

RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server.  FFS from RAN2 perspective if there are cases without a UE in the location server role.

RAN2#119-e also reached the following agreements on “UE roles”:

Align with SA2/RAN1 on the terms for sidelink positioning, and introduce the following terms of UE role as the baseline for further discussion:

-	Target UE: UE to be positioned

-	Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface.  FFS: clarification of the knowledge of the anchor UE.

Additional roles can be considered.

In addition to the above UE roles of Target and Anchor, the UE roles under discussion in RAN1 and SA2 [5] may be useful for the discussion:

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

-	Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface).

-	Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface.

The terms Target UE and Anchor UE describe specific UE functions or capabilities a UE may exercise when participating in sidelink positioning (see also 2.5). These roles are not entities of an architecture, but simply UE functions. The architectural entity/node is still a “UE”, and each UE in the architecture may act as a “target”, “anchor”, “server” etc., depending on its respective capabilities). In particular, a UE may support several roles simultaneously, e.g., if the UE is both a target of positioning and assisting positioning of other UEs. Therefore, like the SA2 architecture summarized in section 2.2 above, the NG-RAN positioning architecture should be independent of “UE role” (function). As such, there is no need to delineate separate architecture diagrams for in-coverage, partial-coverage, or out-of-coverage UEs.

The reference architecture described in Figure 1 can be directly mapped to the RAN2 UE overall positioning architecture [7] as shown in Figure 6. In-coverage UEs (UE A, UE B) and out-of-coverage UEs (UE C, UE D) conduct sidelink positioning and ranging over the PC5 reference point using SLPP. Network components, such as an LMF supporting SLPP, can provide UE assistance by requesting UE capabilities, configuring measurements, receiving measurement results, and computing resultant positions or relative locations, as further discussed in 2.5.

Proposal 3:	The UE Positioning Architecture applicable to NG-RAN should not introduce additional entities/nodes (e.g., “anchor UE”, “server UE”, “target UE”, etc.) and should be applicable to all coverage scenarios (e.g., no separate architecture for in-coverage or out-of-coverage scenarios is needed).

Proposal 4:	Extend the UE Positioning Architecture applicable to NG-RAN as shown in Figure 6.

Figure 6: UE Positioning Overall Architecture applicable to NG-RAN

## Functionality

A SPRF can provide the following functionality in support of a sidelink positioning and ranging service:

- SLPP: SLPP messages over the SR5 reference point enable configuring and executing a sidelink positioning and ranging session. This may include capability transfer, measurement configuration and location information exchange. SLPP can be a separate protocol from LPP, such that SLPP can be supported without having to necessarily support LPP.

- Location and range calculation: UEs capable of performing location or range calculations may do so on behalf of other UEs participating in a sidelink positioning and ranging session based on location information exchanged via SLPP. When an LMF is a participant in the sidelink positioning and ranging session the LMF may perform location and range calculation for, or on behalf of, participating UEs through its SPRF.

An example message flow for a sidelink positioning and ranging exchange is shown in Figure 7. Device discovery is conducted over the SR5 reference point. After device discovery, UEs may communicate via SLPP over the SR5 reference point to perform capability transfer and measurement configuration. Once measurement configuration is established, UEs transmit and measure respective Positioning Reference Signals (PRS), as defined by RAN1. SLPP is used to exchange measurement results (location information), and range and position calculation is performed by participating UEs, or may be performed by one UE on behalf of another UE.

Observation 4: SLPP between UEs over the SR5 reference point can use the same basic transaction types as LPP, capability transfer, assistance data transfer and location information transfer.

Figure 7: Sidelink positioning Device Discovery, SLPP

## Reliable transport

Sidelink communication for both PC-U and PC-S may be susceptible to packet loss due to changes in UE separation (i.e., in the case of moving UEs), or due to occlusion between UEs. Such behavior is a characteristic of sidelink communication regardless of cast or transport type. This packet loss may lead to the loss of SLPP messages.

In the case of LPP for Uu operation, loss of LPP messages is also possible when an LPP message is not successfully forwarded by an MME or AMF. Consequently, LPP already contains an acknowledgment and retransmission capability at the LPP level. Such an acknowledgment and retransmission capability could also be supported by SLPP. In addition, or as an alternative, SLPP procedures can be designed with redundancy and tolerance to transport failure. With redundancy, a UE can send an SLPP request message to a group of UEs with each UE in the group then responding to acknowledge receipt of the message or acknowledge having performed or being able to later perform the requested operation (e.g. such transmitting and/or measuring sidelink reference signals). If each response also carries the original information from the SLPP request message, any UE that did not receive the SLPP request message but receives at least one response message will be able to infer and reconstruct the original SLPP request and thereby overcome the original SLPP message loss. With tolerance to transport failure, the sender of an SLPP request message to a group of UEs can expect that not all of the UEs may receive the request message (or successfully return a response) and thus when an SLPP response message is not received from all the UEs, the sender can take appropriate actions, such as resending the request message at a later time or not expecting the UEs from which a response was not received to take any requested actions.

Support of acknowledgment and retransmission, redundancy and/or tolerance to transport failure may add some signalling and procedural overhead, but the greater robustness, efficiency and lower latency may more than compensate for this.

Observation 5: Communication over sidelink may incur packet loss due to changes in participating UE relative position and/or occlusion, potentially leading to SLPP message loss.  RAN2 should study mechanisms to introduce SLPP robustness such as redundancy, acknowledgement and retransmission, and/or tolerance to transport failure.

## SLPP Session and Session-less Modes

With LPP [6], a UE with NR or LTE access to 5GC and an LMF engage in an LPP session to locate the UE using position methods defined for LPP. Similarly, for SLPP, a pair of UEs or a group of UEs, and possibly one or more LMFs, could be used in a session mode in which they engage in an SLPP session to obtain absolute locations, relative locations or ranges for the participating UEs based on position methods supported in SLPP. Participating UEs should be cognizant of the other participating UEs in order to effectively apply measurements received from other UEs. This awareness of SLPP session participants implies a benefit in having a mechanism to establish an SLPP session in some formal manner and identify which UEs are participants in that SLPP session. For example, consider a scenario where a UE A has discovered UE B and UE C; UE B has discovered UE A but not UE C; and UE C has discovered UE A but not UE B. Which of the UEs A, B and C should participate in an SLPP session, and how each UE is notified of the participants is beyond the capability of Discovery alone and instead suggests additional capability is required. Such session management capability could be part of SLPP. Once an SLPP session has been established, positioning can proceed.

Observation 6: UEs participating in sidelink positioning and ranging should be aware of the other participating UEs in order to effectively apply measurements received from the other UEs.

Figure 8 illustrates a possible overall procedure in which an SLPP session is created to support positioning among a group of UEs 1 to n.

Figure 8: SLPP Positioning based on an SLPP Session

- UEs 1 to n discover one another.

- An SLPP session is established between UEs 1 to n based on the discovery information from step 1.

- Positioning is performed among UEs 1 to n using SLPP (as described in section 2.6).

- As an option, the SLPP session may be modified by adding and/or removing UEs (e.g. if some of UEs 1 to n are no longer in PC5 range of other UEs and/or if other UEs move within PC5 range of UEs 1 to n). For example, in a dynamic V2X environment, it is quite likely that any initial set of UEs 1 to n will not stay within PC5 range of one another for any long period of time.

- Also as an option, the SLPP session can be explicitly terminated.

Figure 9 provides one example of how the session establishment at step 2 of Figure 8 might operate if UE1 acting as a coordinating UE (e.g. UE1 is an RSU for V2X) were to separately discover each of the other UEs 2 to n and then start an SLPP session for all of the UEs 1 to n.

Figure 9: Starting an SLPP Session among a group of UEs 1 to n

- UE1 discovers each of the other UEs 2 to n, including their capability to support SLPP.

- UE1 invites UE 2 to become part of an SLPP session.

- UE1 invites UE 3 to become part of an SLPP session.

- UE1 invites UE n to become part of an SLPP session.

Note: Possibly not all of UEs 2 to n may accept joining an SLPP session, which is one reason for performing steps 2 to 4.

- UE1 transmits a message to all UEs 2 to n to initiate an SLPP session.  UE1 may include the addresses and possibly UE IDs used in SLPP for each of the UEs 1 to n. The group address used might be provided to each UE at one of steps 2 to 4.

- The participating UEs individually acknowledge the session start, subsequent to which the positioning within the session can start. If the coordinating UE1 needs to modify the SLPP session after the session has started (e.g. if some of UEs 2 to n move out of range of UE1 and/or if UE1 discovers other UEs nearby which might join the session), UE1 can repeat steps similar to steps 2-4 for any new UEs and repeat steps 5 and 6 to restart the session and include addresses and IDs only for UEs which will be part of the restarted session for the repetition of step 5.

Observation 7: SLPP sessions may be used between 2 or more UEs to support sidelink positioning in a structured and coordinated manner.

The SLPP session mode described requires an explicit SLPP session between participating UEs which may result in additional signaling overhead (e.g. all the steps shown in Figure 9). As an alternative approach, a group of UEs might perform positioning using SLPP in a session-less mode in which there is no discovery, no UE associations and no SLPP sessions. The motivation would be to minimize signaling overhead and enable instant adaptation to changing UE conditions. For example, on a busy highway, there might be many UEs near enough to one another to exchange signaling and measure PRS, but where the set of nearby UEs changes every few seconds such that SLPP sessions, if used, might have to also change, introducing additional signaling overhead. This session-less approach is illustrated in Figure 10. Although Figure 10 illustrates a session-less approach for 3 UEs, the principles are extensible to any number of UEs.

Figure 10: SLPP Session-less mode between UEs 1, 2 and 3

- Each UE broadcasts an SLPP message indicating when the UE intends to broadcast a reference signal (e.g. sidelink PRS) and a configuration for the reference signal (e.g. frequency, bandwidth, coding, duration, timing, etc.). As potential for collisions may be present, UEs could first listen for SLPP messages from other UEs and avoid reusing the same radio resources. There is also an assumption that UEs share some common time reference, which could be based on GNSS or on transmission timing from a network or nearby RSUs (in the case of V2X).

- UEs broadcast their reference signals as indicated at step 1 and measure reference signals broadcast by other UEs, also as indicated at step 1.

- UEs exchange measurements (also via broadcast).

- UEs determine ranges (and possibly bearings) based on performed measurements and measurements received from other UEs at step 3.  Steps 1 to 4 can then be repeated.

Observation 8: SLPP could be used in a session-less mode between 2 or more UEs to support sidelink positioning with reduced signaling overhead and greater adaptivity to changing UE locations and other changing conditions.

The examples in Figure 8 to Figure 10 provide possible directions for study in RAN2 to determine their suitability. RAN2’s study could include the following aspects:

- SLPP sessions

Session establishment, modification and termination

Signaling Overhead

- Session-less SLPP

Signaling overhead versus session oriented SLPP

Means of identifying UEs

Supporting a common time reference (e.g. UTC or GNSS)

Proposal 5: SLPP should support session-based operation and session-less operation.  SLPP session-based operation includes session establishment among a group of UEs, session modification to add or remove UEs and session termination.  Session-less operation enables sidelink positioning with no discovery, no UE associations and no SLPP session.

## SLPP Positioning Procedures

According to the RAN2#119-e agreement, at least the following two operation scenarios should be studied:

Study the architecture and signaling procedures to enable at least the following two operation scenarios:

-	Operation Scenario 1: PC5-only-based positioning.

-	Operation Scenario 2: Combination of Uu- and PC5-based positioning.

### PC5-only based positioning

#### SLPP Operation without LMF Support, PC5-only-based positioning

Figure 11 and Figure 12 illustrate two possible example scenarios for SLPP-managed sidelink positioning and ranging for the Operation Scenario 1 (PC5-only-based positioning). Figure 11 illustrates a centralized mode of operation where one UE (UE B) performs the computations to determine the range and/or position of the four participating UEs, and Figure 12 illustrates a distributed (or decentralized) mode of operation, where each of the participating UEs perform range/position computations. As clarified in the stepwise descriptions below Figure 11 and Figure 12, in both centralized and distributed operation, following discovery and potential SLPP session establishment, the initiating UE (UE B ) may transmit a request for the position and range capabilities of the participating UEs (SLPP Request Capabilities message). The addressed UEs can subsequently respond with their respective capabilities (SLPP Provide Capabilities message). Individual UE capabilities may include details of the supported SL-PRS configurations and supported SL-PRS measurements. Based on the received information from the participating UEs, the initiating UE (UE B) may transmit positioning assistance data to the participating UEs (SLPP Provide Assistance Data message). Examples of positioning assistance data could include a SL-PRS configuration for each of the participating UEs. After receipt of assistance data, participating UEs may transmit their respective SL-PRS (according to received SL-PRS configurations from the initiating UE) and perform the requested measurements of SL-PRS received from other UEs. Upon completion of measurements, UEs may transmit measurement results (SLPP Provide Location Information message). In the centralized mode of operation (Figure 11), the participating UEs (apart from the initiating UE B) would transmit measurement results, which the initiating UE B would use to determine ranges and/or positions for all participating UEs, and potentially subsequently distribute those results to the participating UEs. In the distributed mode of operation (Figure 12), all UEs may transmit measurement results, and each participating UE can perform the range and/or position computations. Such a sequence of capability transfer, assistance data transfer and measurement/location information transfer enables UEs to conduct sidelink positioning transactions in the absence of network coverage.

Figure 11: Sidelink-only Positioning – Initiator (centralized) position calculation

1.	UEs A-D discover one another as specified in [8]. The discovery protocol messages include information about SLPP support of the UE (e.g., analogous to the LPP support indication in the 5GMM capability IE [9]).

2.	An SLPP Session is established between UEs A-D based on the discovery information from Step 1 as described in section 2.5.

3.	UE B transmits a SLPP Request Capabilities message to the group of UEs established at Step 2 requesting the SL positioning capabilities from the UEs participating in this session. The SLPP Request Capabilities message may be sent by UE B to UE A, UE C, UE D via broadcast, groupcast or unicast, depending on the specific scenario.

4.	The UEs in Step 3 addressed by UE B transmit a SLPP Provide Capabilities message. Similar to LPP, the UE capabilities may refer to particular sidelink position methods (e.g., SL-RTT, SL-AoA, SL-TDOA, etc.) or may be common to multiple position methods.
The capabilities included in the SLPP Provide Capabilities message may also include the SL-PRS capabilities, an indication whether the UE can function as an anchor UE and/or location server UE, position/range calculation capabilities, etc.. If the UE can function as an anchor UE, the SLPP Provide Capabilities message may also include the location coordinates. The SLPP Provide Capabilities message may be sent by UE B to UE A, UE C, UE D via broadcast, groupcast or unicast, depending on the specific scenario.

5.	The SLPP Session may be modified as described in section 2.5 based on the capability information from Step 4, e.g., by removing UEs from the SLPP Session which do not support the required capability (for example, do not support the desired SL positioning method, or can not function as anchor UE, etc.). 
Based on new discovery information and capability exchange (e.g., by repeating Steps 1, 3 and 4), additional UEs with the desired capability may also be added to the session.

6.	UE B transmits a SLPP Provide Assistance Data message to the group of UEs established at Step 1 and 5. The message includes the SL-PRS configuration for each of the UEs in the group. The SLPP Provide Assistance Data message may be sent via broadcast, groupcast or unicast, depending on the specific scenario. 
After receipt of the assistance data (or at an indicated starting time included in the message), each UE in the group transmit their respective SL-PRS. The duration of SL-PRS transmission may be controlled by a timer provided in the message or may continue until the SLPP session is terminated at Step 12.

7.	UE B transmits a SLPP Request Location Information message to the group of UEs. The message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation will be performed by the initiating UE (UE B). The SLPP Request Location Information message may be sent via broadcast, groupcast or unicast, depending on the specific scenario.

8.	Each participating UE performs the requested measurements of SL-PRS.

9.	Upon completion of the measurements or after the response time received at Step 7 expired, the UEs transmit a SLPP Provide Location Information message with the location results requested at Step 7. 
The message may also include an indication whether the initiating UE (UE B) is requested to provide the location result (e.g., calculated range and/or position) back to this UE.

10.	The initiating UE B performs the range/position computations using the location results received at Step 9 and the location results UE B has obtained at Step 8.

11.	If requested at Step 9, UE B may distribute the location results to the other UEs in this session.

12.	The SLPP session may be terminated as described in section 2.5.

Figure 12: Sidelink-only Positioning – Distributed position calculation

1.-6. As in Figure 11.

7.	UE B transmits a SLPP Request Location Information message to the group of UEs. The message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation should be performed by each of the participating UEs. The SLPP Request Capabilities message may be sent by UE B to UE A, UE C, UE D via broadcast, groupcast or unicast, depending on the specific scenario.

8.	Each participating UE performs the requested measurements of SL-PRS.

9.	Upon completion of measurements or after the response time received at Step 7 expired, the UEs transmit a SLPP Provide Location Information message with the location results requested at Step 7. 
The message may also include an indication whether each UE is requested to provide the location result (e.g., calculated range and/or position) to the other UEs in the session. The SLPP Provide Location Information message may be sent via broadcast, groupcast or unicast, depending on the specific scenario.

10.	Each participating UE performs the range/position computations using the location results received at Step 9 and the own location results obtained at Step 8.

11.	If requested at Step 9, participating UEs distribute the location results to the other UEs in this session.

12.	The SLPP session may be terminated as described in section 2.5.

Proposal 6: Sidelink positioning protocol (SLPP) should support centralized operation, where one UE performs position/range computation based on shared measurement/location information, and distributed operation, where all participating UEs perform position/range computation.

#### SLPP Operation with LMF Support, PC5-only-based positioning

The basic SLPP transaction types (capability transfer, assistance data transfer, location information transfer) suitable for supporting out-of-coverage sidelink positioning and ranging scenarios may also be applied to in-coverage and partial coverage scenarios. These scenarios include LMF-assisted (MO-LR) and LMF-initiated (MT-LR) sidelink positioning and ranging. When at least one participating UE is in network coverage and enabled via a subscription to access a PLMN, the UE may request LMF assistance for a sidelink positioning and ranging session, or may be requested by an LMF to participate in a sidelink positioning and ranging session. In this mode of operation, not all UEs may have PLMN access and be supported by, or provide support to, an LMF (LMF support may be restricted to just some UEs). Possible examples of LMF-assisted (MO-LR) and LMF-initiated (MT-LR) are illustrated in Figure 13 and Figure 14, respectively.

LMF-assisted Sidelink positioning (MO-LR)

Figure 13 shows an example of LMF-assisted sidelink positioning and ranging, where the LMF acts as an adjunct to UE B which is in network coverage and initiating a sidelink positioning and ranging session.

Figure 13: LMF-assisted Sidelink positioning (MO-LR)

1.-5. As in Figure 11.

6.	The initiating UE B requests positioning assistance data from the LMF (assistance data could include SL-PRS configurations) via an SLPP Request Assistance Data message. To enable the LMF to determine suitable SL-PRS configurations for all UEs, UE B may also provide the obtained SL-PRS capabilities of all UEs in the session to the LMF in a SLPP Provide Capabilities message.
The SLPP transactions towards the LMF might be part of a location session between UE B and the LMF, e.g. initiated by an MO-LR or a new supplementary services operation. Thus, one possibility would be for UE B to send a supplementary services (e.g. MO-LR) message to the LMF and include the SLPP Provide Capabilities message for step 6a and possibly the SLPP Request Assistance Data message for step 6b to increase signalling efficiency.

7.	The LMF determines the requested assistance data and provides them to UE B in a SLPP Provide Assistance Data message.

8.-11. Same as Steps 6.-9. In Figure 11.

12.	The initiating UE B may perform the range/position computations using the location results received at Step 11 and the location results UE B has obtained at Step 10.

13.	If Step 12 did not occur, UE B may provide the obtained location measurements from all participating UEs in the session to the LMF in an SLPP Provide Location Information message.
The SLPP transactions towards the LMF might be part of a location session between UE B and the LMF, e.g. initiated by an MO-LR or a new supplementary services operation.

NOTE:	The SLPP transactions towards the LMF at Steps 6 and 13 may be part of the same MO-LR operation or may be separate MO-LR operations.

14.	The LMF performs the range/position computations.

15.	The LMF provides the computed ranges/positions to the UE B.

16.	The SLPP session may be terminated as described in section 2.5.

Proposal 7:	Support a MO-LR or a new supplementary services operation for UE initiated SLPP transactions towards an LMF.

Figure 14 shows an example of LMF-initiated sidelink positioning and ranging operation with a target UE (UE B) in network coverage to obtain MT-LR location results. The MT-LR could have been instigated by an external client or AF (not shown in Figure 14) which would have provided all necessary information for the MT-LR to the LMF (e.g. via a GMLC and AMF).

Figure 14: LMF-initiated Sidelink positioning for an MT-LR

1.	The LMF requests the sidelink positioning capabilities from the target UE via SLPP Request Capabilities message.

2.	The target UE provides the sidelink positioning capabilities in a SLPP Provide Capabilities message to the LMF.

NOTE:	Steps 1 and 2 may be embedded in Step 3.

- The LMF requests location results from the target UE using a supplementary services operation request, which indicates the type of location results requested (e.g. location of the target UE and/or of other UEs), identities and/or addresses of specific UEs to be involved (e.g. UEs A, C, D), or whether any UEs can be used, and whether a single set of location results is requested (immediate location), or whether deferred (e.g. periodic or triggered) location results are requested. The supplementary services request might also include an embedded SLPP Request Location Information message indicating specific SLPP location results or measurements to be provided by the target UE and/or an embedded SLPP Provide Assistance Data message to provide assistance data for the SLPP positioning session to the target UE (e.g. SL-PRS configurations).

NOTE:	The reason for using a supplementary services request would be to allow inclusion of information such as UE addresses and identities, and use of immediate versus deferred location which might not be suitable for inclusion in an SLPP message. However, it is possible that an SLPP message could be used instead (e.g. an SLPP Request Location Information).

4.	The target UE performs the discovery procedures as specified in [8].

5.	The target UE confirms or acknowledges the supplementary services request from Step 3 with a supplementary services response, which may indicate whether any requested UEs are available (e.g. whether UEs A, C, D have been discovered by the target UE B).

NOTE:	This Step 5 may also be performed after Step 8. Once the target UE has obtained the sidelink positioning capabilities from UEs A, C, and D, the supplementary services response could indicate to the LMF the available UEs with required capability.

6.-14. Same as Steps 2.-10. In Figure 11.

15.	The target UE B provides the obtained location results and/or measurements from all participating UEs in the session to the LMF in an SLPP Provide Location Information message.

NOTE: 	This Step 15 may be a Supplementary Services Operation with embedded SLPP message. This Service Operation may be part of Step 3, or may be a separate Service Operation (e.g., an enhancement of the existing Event Report).

16.	The LMF performs the range/position computations for the target UE and possibly for the other UEs and return location(s) to the external client or AF. For deferred (periodic or triggered) location, the SLPP positioning by the target UE and return of location results to the LMF might be repeated.

17.	The SLPP session may be terminated as described in section 2.5.

Proposal 8:	Support a MT-LR or a new supplementary services operation for LMF-initiated SLPP transactions towards a UE.

Figure 15 shows the same scenario as Figure 14, but where the LMF actively assists with the SLPP positioning of the four UEs as in Figure 13. For example, the LMF might indicate in the supplementary services request whether no LMF assistance is preferred as in Figure 14 or whether assistance is preferred (or required) as in Figure 15.

Figure 15: LMF-initiated Sidelink positioning for an MT-LR with additional LMF Assistance

1.-2. Same as in Figure 14.

3.	Same as in Figure 14.
The supplementary services request also indicates that LMF assistance is allowed or preferred.

4.-9. Same as in Figure 14.

10.	If indicted at Step 3, the initiating UE B may request positioning assistance data from the LMF (assistance data could include SL-PRS configurations) via an SLPP Request Assistance Data message. To enable the LMF to determine suitable SL-PRS configurations for all UEs, UE B may also provide the obtained SL-PRS capabilities of all UEs in the session to the LMF in a SLPP Provide Capabilities message.
The SLPP transactions towards the LMF might be part of a location session between UE B and the LMF, e.g. initiated by an MO-LR, or part of the MT-LR from Step 3, or a new supplementary services operation.

11.	The LMF determines the requested assistance data and provides them to UE in a SLPP Provide Assistance Data message.

12.-18. Same as Steps 10.-16. In Figure 14.

19. The SLPP session may be terminated as described in section 2.5.

Observation 9:	An LMF initiated sidelink/SLPP MT-LR can be supported on the target UE side using the same SLPP procedures as applicable to UE initiated sidelink/SLPP positioning and with either no LMF support during the positioning or active LMF support during the positioning.

### Combination of Uu- and PC5-only based positioning

The basic SLPP transaction types (capability transfer, assistance data transfer, location information transfer) suitable for supporting in-coverage, partial coverage and out-of-coverage sidelink positioning and ranging scenarios may also be applied to support joint sidelink-Uu positioning. This could simply be achieved by jointly performing the SLPP, LPP and NRPPa procedures for the desired positioning methods as shown in Figure 16. For Uu-only positioning, LPP and NRPPa are jointly used for UE positioning operation (e.g. for Multi-RTT). For joint Uu and SL positioning, any combination of LPP, SLPP, and NRPPa may be used, dependent on the desired positioning method(s) (UL-, DL-, SL-positioning).

Figure 16: Joint Uu-Sidelink Positioning

1.	The LMF may obtain the sidelink positioning capabilities from the target UE and may request sidelink location results from the target UE.

2.	The LMF may obtain the LPP positioning methods capabilities from the target UE and may request location results for one or more LPP positioning method.

3.	The LMF may request an UL-SRS configuration from the serving NG-RAN node of the target device and may configure TRPs for UL-SRS measurements.

NOTE: 	The procedures and transactions for Steps 1-3 may be serialized in any preferred order. For example, the LMF may first jointly obtain the SLPP and LPP capabilities, then jointly request SLPP and LPP location results, etc.

- The target device performs the sidelink positioning procedures as described in Figure 11, or Steps 6.-14. In Figure 14, or Steps 6.-16. In Figure 15.

- The target device provides the SLPP location results to the LMF.

- The target device provides the LPP location results to the LMF.

- The TRPs provide the NRPPa location results to the LMF.

The procedure described above in Figure 16 will support hybrid/joint Uu (including RAT-Independent methods) and sidelink positioning in the same way as it is supported today with LPP (and possibly NRPPa). For example, several LPP positioning methods can be used for the same positioning session. The LMF knows that the different positioning methods apply to the same positioning session. The UE does not know that unless the same LPP messages and transactions are used for the different positioning methods. When different LPP transactions and messages are used, the UE may not know if the different positioning methods refer to the same LPP session or to different LPP sessions. When both Uu and PC5 based procedures and signalling are used, then from the perspective of the coordinating entity (LMF in this case), the Uu and PC5 based positioning procedures are part of the same positioning session. This methodology allows SLPP and LPP (and NRPPa) to be defined and used independently even though a coordinating entity, such as an LMF, can be aware of a common positioning session. In particular, this means that SLPP signalling does not need to convey LPP session-related information and similarly, LPP signaling does not need to convey SLPP session-related information. This can significantly reduce SLPP and LPP impacts, RAN2 specification work, and requirements levied on the UE and the network.

Proposal 9:	Support hybrid Uu, SL, and RAT-independent positioning by jointly using the SLPP, LPP, and NRPPa procedures.

Using SLPP transactions/procedures independently of LPP also means there are no additional impacts to those UEs (and LMFs) that do not need to support sidelink positioning and vice versa. For example, if an LMF or UE were to receive an LPP message with sidelink positioning content, then even if this content is not supported, the decoding of the LPP message might need to be “sidelink capable”. Similarly, if a sidelink positioning or ranging device (or LMF) does not need to support LPP positioning, there would be no need to support LPP just for the sidelink positioning or ranging content. This allows a smaller protocol to be defined to reduce ASN.1 footprint size and processing support. That could simplify UE implementation when LPP does not otherwise need to be supported.

From the discussions in section 2.6, we can make the following additional observations and proposals:

Observation 10: The three fundamental transaction types from LPP (capability transfer, assistance data transfer, and location information transfer) are also suitable for the SLPP positioning operation over sidelink for both operation scenarios agreed to by RAN2, including operation Scenario 1 (PC5-only-based positioning) with or without LMF support, and operation Scenario 2 (Combination of Uu- and PC5-based positioning).

Observation 11: Using the LMF as an SLPP endpoint would not only allow embedding SLPP messages into Supplementary Services Operations (e.g., MT-LR, MO-LR, etc.), but would also enable joint sidelink and Uu positioning with minimal changes to existing protocols.

Observation 12: Using the LMF as an SLPP endpoint would allow sidelink-only positioning (with LMF support) without a requirement for LPP support, which would reduce complexity for sidelink-only capable positioning and ranging devices and LMFs.

Observation 13: Specifying the sidelink positioning functionality isolated within SLPP would require no modifications to LPP, and therefore, would not increase the LPP ASN.1 footprint for non-sidelink capable UEs and LMFs.

Proposal 10: SLPP should support at least the three fundamental transaction types from LPP: capability transfer, assistance data transfer, and location information transfer.

Proposal 11: The LMF should be a protocol endpoint for SLPP.

# Conclusion

In this paper, we have addressed key issues for the development of SLPP, including architecture, transport, cast type, procedures and joint Uu-sidelink positioning, summarized as follows.

Architecture: The SA2 reference architecture can be directly mapped to the RAN2 UE overall positioning architecture. This architecture is applicable for all operational scenarios, such that a separate architecture is not required for in-coverage, partial coverage or out-of-coverage scenarios.

Procedures: The three fundamental LPP transaction types (capability transfer, assistance data transfer and location information transfer) are appropriate for SLPP and should be used for sidelink positioning.

Transport: For SLPP transport, PC5 User Plane (PC5-U) enables UE-to-UE communication with no network involvement (just as PC5-S transport), but PC5-U enables lower latency, higher efficiency, greater flexibility in cast type and QoS than PC5-S, including:

- PC5-U link establishment allows immediate data transmission without incurring the latency associated with multiple request-response message exchanges required for PC5-S link establishment.

- PC5-U supports one-to-one communication between two UEs (unicast) and one-to-many (broadcast/groupcast). All three cast types are applicable to SLPP for the three fundamental transactions (Capability, Assistance, Location Information) as a function of operational scenario supported. Unicast may be used for sidelink positioning sessions involving only two UEs, while broadcast/groupcast may be used for sidelink positioning sessions involving larger groups of UEs.

- PC5-U supports QoS through the PC5 QoS Flow Identifier (PFI) from the application layer to the AS layer, including Resource Type, Priority Level, Packet Delay Budget, Packet Error Rate.

- All transport types (PC5-U, PC5-S) will be susceptible to SLPP message loss due to changes in participating UE relative position and/or occlusion. However, there are potential remedies for this which can be studied and supported if found to be suitable.

In view of the very low latency requirements of sidelink applications such as V2X, and the limited ITS spectrum, using multiple unicast connections between pairs of UEs (e.g. vehicles on a crowded roadway) may not only lead to unacceptably high delay but also bandwidth congestion, potentially negatively impacting the performance of sidelink positioning.

Joint Uu-sidelink positioning:

- Using the LMF as an SLPP endpoint allows embedding SLPP messages into Supplementary Services Operations (e.g., MT-LR, MO-LR, etc.), and enables joint sidelink and Uu positioning with minimal changes to existing protocols. Using the LMF as an SLPP endpoint also allows sidelink-only positioning (with LMF support) without a requirement for LPP support, which reduces complexity for sidelink-only capable positioning and ranging devices and LMFs.

- When both Uu and PC5 based procedures and signalling are used, from the perspective of the coordinating entity (i.e., the LMF), the Uu and PC5 based positioning procedures are part of the same positioning session. This methodology allows SLPP and LPP (and NRPPa) to be defined and used independently even though a coordinating entity, such as an LMF, can be aware of a common positioning session. As such, SLPP signalling does not need to convey LPP session-related information and similarly, LPP signaling does not need to convey SLPP session-related information, thereby significantly reducing SLPP and LPP impacts, RAN2 specification work, and requirements levied on the UE and the network. Using SLPP transactions/procedures independently of LPP also means there are no additional impacts to those UEs (and LMFs) that do not need to support sidelink positioning, and vice versa. For example, if an LMF or UE were to receive an LPP message with sidelink positioning content, even if this content is not supported, decoding that LPP message might required the LMF/UE to be sidelink capable. Similarly, if a sidelink positioning or ranging device (or LMF) does not need to support LPP positioning, there would be no need to support LPP just for the sidelink positioning or ranging content. This allows a smaller protocol to be defined, thus reducing the ASN.1 footprint size and processing support. This simplifies UE implementation when LPP does not need to be supported.

# Summary

Observation 1: A UE-based Sidelink Positioning and Ranging Function (SPRF) communicating over the SR5 reference point can be used to provide location and range computation and sidelink positioning and ranging protocol (SLPP) functionality.

Observation 2: SLPP transport over PC5-U supports in-coverage, partial-coverage and out-of-coverage sidelink positioning avoiding unnecessary link establishment latency, providing QoS and enabling flexibility in cast type.

Observation 3: Exchange of Sidelink Positioning and Ranging Function (SPRF) messaging can be accomplished without modification to the PC5 reference point over the PC5 user plane.

Observation 4: SLPP between UEs over the SR5 reference point can use the same basic transaction types as LPP, capability transfer, assistance data transfer and location information transfer.

Observation 5: Communication over sidelink may incur packet loss due to changes in participating UE relative position and/or occlusion, potentially leading to SLPP message loss.  RAN2 should study mechanisms to introduce SLPP robustness such as redundancy, acknowledgement and retransmission, and/or tolerance to transport failure.

Observation 6: UEs participating in sidelink positioning and ranging should be aware of the other participating UEs in order to effectively apply measurements received from the other UEs.

Observation 7: SLPP sessions may be used between 2 or more UEs to support sidelink positioning in a structured and coordinated manner.

Observation 8: SLPP could be used in a session-less mode between 2 or more UEs to support sidelink positioning with reduced signaling overhead and greater adaptivity to changing UE locations and other changing conditions.

Observation 9: An LMF initiated sidelink/SLPP MT-LR can be supported on the target UE side using the same SLPP procedures as applicable to UE initiated sidelink/SLPP positioning and with either no LMF support during the positioning or active LMF support during the positioning.

Observation 10: The three fundamental transaction types from LPP (capability transfer, assistance data transfer, and location information transfer) are also suitable for the SLPP positioning operation over sidelink for both operation scenarios agreed to by RAN2, including operation Scenario 1 (PC5-only-based positioning) with or without LMF support, and operation Scenario 2 (Combination of Uu- and PC5-based positioning).

Observation 11: Using the LMF as an SLPP endpoint would not only allow embedding SLPP messages into Supplementary Services Operations (e.g., MT-LR, MO-LR, etc.), but would also enable joint sidelink and Uu positioning with minimal changes to existing protocols.

Observation 12: Using the LMF as an SLPP endpoint would allow sidelink-only positioning (with LMF support) without a requirement for LPP support, which would reduce complexity for sidelink-only capable positioning and ranging devices and LMF's.

Observation 13: Specifying the sidelink positioning functionality isolated within SLPP would require no modifications to LPP, and therefore, would not increase the LPP ASN.1 footprint for non-sidelink capable UEs and LMFs.

Proposal 1 A UE supporting sidelink positioning and ranging should be capable of being enabled with the ability to calculate position and or range based on sidelink positioning and ranging measurements.

Proposal 2: RAN2 should support SLPP transport over PC5 User Plane to provide SLPP QoS and flexibility in cast type while introducing no change to the PC5 reference point.

Proposal 3: The UE Positioning Architecture applicable to NG-RAN should not introduce additional entities/nodes (e.g., "anchor UE", "server UE", "target UE", etc.) and should be applicable to all coverage scenarios (e.g., no separate architecture for in-coverage or out-of-coverage scenarios is needed).

Proposal 4: Extend the UE Positioning Architecture applicable to NG-RAN as shown in Figure 6.

Proposal 5: SLPP should support session-based operation and session-less operation. SLPP session-based operation includes session establishment among a group of UEs, session modification to add or remove UEs and session termination.  Session-less operation enables sidelink positioning with no discovery, no UE associations an no SLPP session.

Proposal 6: Sidelink positioning protocol (SLPP) should support centralized operation, where one UE performs position/range computation based on shared measurement/location information, and distributed operation, where all participating UEs perform position/range computation.

Proposal 7:	Support a MO-LR or a new supplementary services operation for UE initiated SLPP transactions towards an LMF.

Proposal 8:	Support a MT-LR or a new supplementary services operation for LMF-initiated SLPP transactions towards a UE.

Proposal 9:	Support hybrid Uu, SL, and RAT-independent positioning by jointly using the SLPP, LPP, and NRPPa procedures.

Proposal 10: SLPP should support at least the three fundamental transaction types from LPP: capability transfer, assistance data transfer, and location information transfer.

Proposal 11: The LMF should be a protocol endpoint for SLPP.

# References

- RP-222616

- R2-2208704 Report from session on positioning and sidelink relay

- TR 23.700-86

- TS 23.287

- RP-222258

- 37.355

- TS 38.305

- TS 23.304

- TS 24.501