3GPP TSG-RAN WG2 Meeting #119bis Electronic	R2-2210421

Online, October 10 – 19, 2022

Agenda item:	8.17.2.1

Source:	Sharp

Title:	eMUSIM Scenarios

WID/SID:	NR_DualTxRx_MUSIM-Core

Document for:	Discussion and Decision

1	Introduction

In the Rel-18 eMUSIM WID [1], one of the objectives is:

Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4].

- Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose

- RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR.

Applicable UE architecture: Dual-RX/Dual-TX UE

In this discussion paper, we focus on the scenarios.

2 Discussion

According to the WID, what needs to be defined is what should UE/network do to enable UE working in MUSIM (i.e., operate in RRC_CONNECTED state simultaneously in both of the network) and what should UE/network do when UE exits MUSIM.

Before the UE operates in RRC_CONNECTED state in both of the networks simultaneously, the possible states of UE in Network A and Network B are:

Scenario 1: UE is in RRC Connected state in Network A and configured with activated SCells or SCG, UE is in RRC_IDLE or RRC_INACTIVE in network B and needs to setup/resume RRC Connection with Network B

Scenario 2: UE is in RRC Connected state in Network A and configured with deactivated SCells or SCG (or SCells or SCG is not configured), UE is in RRC_IDLE or RRC_INACTIVE in network B and needs to setup/resume RRC Connection with Network B

Scenario 3: UE is in RRC Connected state in Network B and in RRC_IDLE or RRC_INACTIVE in network A and needs to setup/resume RRC Connection with Network A

There is also a scenario that UE is in RRC_IDLE or RRC_INACTIVE in both of the networks and needs to setup/resume the RRC Connection with both of the networks. For this scenario, UE may setup/resume the RRC_CONNECTION with one of the networks firstly, then this scenario becomes scenario 1/2/3. So we do not need to handle this scenario independently.

When UE is in RRC_CONNECTED state in both network A and network B, and one of the RRC connection is no more needed and will be released, the possible states of UE in Network A and Network B are:

Scenario 4: UE is in RRC Connected state in both Network A and Network B initially, the RRC Connection with Network B is released

Scenario 5: UE is in RRC Connected state in both Network A and Network B initially, the RRC Connection with Network A is released

There is also a scenario that UE is in RRC Connected state in both Network A (without DC or CA or with deactivated SCells or SCG) and Network B, Network A reconfigures UE with activated SCells or SCG or Network A activates SCells or SCG:

Scenario 6: UE is in RRC Connected state in both Network A (without DC or CA or with deactivated SCells or SCG) and Network B, Network A reconfigures UE with activated SCells or SCG or Network A activates SCells or SCG.

Considering that Rel-17 MUSIM only considered enhancement to NR, so for scenario 3/5/6, we can only consider the case that Network B is NR. If Network B is LTE, indicates/removes the restriction can be fulfilled by release and re-setup the RRC connection with Network B. If network B is NR, the scenario 3/5 is same as scenario 1/2/4. So it does not need to be further considered.

Then the scenarios mentioned above need to be further studied includes.

Proposal 1: The scenarios need to be further studied are:

Scenario 1: UE is in RRC Connected state in Network A and configured with activated SCells or SCG, UE is in RRC_IDLE or RRC_INACTIVE in network B and needs to setup/resume RRC Connection with Network B

Scenario 2: UE is in RRC Connected state in Network A and configured with deactivated SCells or SCG (or SCells or SCG is not configured), UE is in RRC_IDLE or RRC_INACTIVE in network B and needs to setup/resume RRC Connection with Network B

Scenario 4: UE is in RRC Connected state in both Network A and Network B initially, the RRC Connection with Network B is released

Scenario 6: UE is in RRC Connected state in both Network A (without DC or CA or with deactivated SCells or SCG) and Network B, Network A reconfigures UE with activated SCells or SCG or Network A activates SCells or SCG.

3 Summary

In this section, we summarized our proposal as showed below:

Proposal 1: The scenarios need to be further studied are:

Scenario 1: UE is in RRC Connected state in Network A and configured with activated SCells or SCG, UE is in RRC_IDLE or RRC_INACTIVE in network B and needs to setup/resume RRC Connection with Network B

Scenario 2: UE is in RRC Connected state in Network A and configured with deactivated SCells or SCG (or SCells or SCG is not configured), UE is in RRC_IDLE or RRC_INACTIVE in network B and needs to setup/resume RRC Connection with Network B

Scenario 4: UE is in RRC Connected state in both Network A and Network B initially, the RRC Connection with Network B is released

Scenario 6: UE is in RRC Connected state in both Network A (without DC or CA or with deactivated SCells or SCG) and Network B, Network A reconfigures UE with activated SCells or SCG or Network A activates SCells or SCG.

4 References

[1] RP- 220955 New WID on Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR.