3GPP TSG-RAN WG2 #119bis-e	R2-2210443

Electronic meeting, 10th – 19th October 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Network Verified UE Location

Document for:	Discussion and decision

Agenda Item:	8.7.3

- Introduction

In the last RAN2 meeting, following agreements were made in discussion on network verified UE location.

Agreements:

- Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability

- For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN

- From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT).

In this contribution, we provide our view from RAN2 perspective on selection of positioning method, reports from UE, and scenarios.

- Discussion

  - Selection of positioning method

In the August meeting, RAN1 agreed that Multi-RTT and DL/UL-TDOA are considered as a starting point as following[1]:

Agreement The following 3GPP defined RAT dependent positioning methods shall be considered as starting point for the study on Network verified UE location in case of NGSO based NTN deployment: - Multi-RTT - DL/UL-TDOA Note-1: Other methods (e.g. AoA based) are not precluded Note-2: RAT independent positioning methods are not under the scope of the study

According to Objective[2], RAN2/1/3 are expected to “study and evaluate solutions for network to verify UE reported location information”. Is should be also noted that that study is planned to be concluded by RAN#98.

4.1.3 Network verified UE location Pending on the conclusion of the RAN SI FS_NR_NTN_netw_verif_UE_loc study item, study and evaluate, if needed, solutions for network to verify UE reported location information [RAN2,RAN1,RAN3]. RAN is expected to determine by RAN#98 whether the study has identified any need for Network verified UE location specification support in Rel-18.

In our understanding, above milestone is achieved by evaluation and discussion in October RAN1 meeting if some of candidate solutions is proved to have enough characteristics to address the verification issue. In other words, we understand down-selection of candidate solutions is not certain to be done in RAN1.

By the way, in the post e-mail discussion of RAN2#119-e, criteria for down-selection of solutions were discussed: where accuracy, latency, and spec impact were mentioned to. Generally, as we mentioned in the offline discussion, accuracy is just a requirement, i.e., solutions shall fulfil the accuracy requirement (<5-10 km) but we do not have to choose the most accurate solution. Latency and spec impact are worth considering. However, it is not appropriate that RAN2 make agreement on issues in RAN1 aspect, i.e., accuracy and latency. RAN2 should wait for discussion in RAN1 with regard to accuracy and latency, then RAN2 can take the leadership on determining the best solution.

Proposal 1. RAN2 should wait for RAN1 evaluation then select appropriate solutions in terms of latency and spec impact from candidate solutions which has enough accuracy.

  - Credibility of reports from UE

In the post e-mail discussion of RAN2#119-e, it was discussed whether UE reported information exclusively provided by 3GPP defined functions can be trusted. In principle, it is better to separately ask SA3 whether each report is trustworthy, because that type of discussion is out of RAN2 scope.

By the way, it was also on table of post e-mail discussion whether RAN2 should ask SA3 to confirm the credibility of TA report. More than half of companies proposed to leave it to RAN1, and we are also ok to wait. Then we would like to discuss a bit more generally: Can RAN2 ask SA3 for confirmation even while the solution is just a candidate, i.e., even before RAN2 down-select one solution? Our preference is yes. This is because, e.g., if we conclude multi-TAR solution is the way to go then ask SA3 whether TAR is trustworthy, then we are wondering what we do if SA3 concludes TAR cannot be trusted. Maybe we have to re-discuss the solution (we want to avoid that).

Proposal 2. It should be confirmed by SA3 case by case whether a certain report from UE can be trusted.

Proposal 3. RAN2 can ask SA3 about UE reports used in candidate solutions even before down-selection.

  - Different solutions for different scenarios

In the post e-mail discussion of RAN2#119-e, it was discussed whether following parameters could be taken into consideration: type of orbit (GSO, NGSO, HAPS) and beam characteristic (Earth moving/fixed).

In the recommendation in TR38.882[3], it is clear that RAN2 study can consider different solutions for different orbit types.

The study in [RAN2,RAN1,RAN3], which will study and evaluate solutions for the network to verify UE reported location information, shall consider the following aspects: - The scenario of single satellite (or HAPS) in view by the UE at a time is considered with higher priority. - Multiple satellite (or HAPS) in view by the UE may be considered if time allows - Assume that the UE is attached to a network (so that its context has been set up in the network) for the purpose of positioning - Different solutions or positioning methods for NGSO, GSO or HAPS are not precluded - When considering solutions based on positioning methods, existing 3GPP defined RAT dependent positioning methods shall be considered as baseline. Other methods are not precluded. - Solutions using existing NG-RAN architecture and procedures shall be considered

On the other hand, we do not see necessity to distinguish beam characteristic (Earth moving/fixed). We propose to assume common solutions for earth moving or earth fixed cell.

Proposal 4. Separate solution for each orbit type (NGSO, GSO, HAPS) can be considered, but common solutions should be assumed whichever the NTN-cell is earth-moving or earth-fixed.

- Summary and proposal

Proposal 1. RAN2 should wait for RAN1 evaluation then select appropriate solutions in terms of latency and spec impact from candidate solutions which has enough accuracy.

Proposal 2. It should be confirmed by SA3 case by case whether a certain report from UE can be trusted.

Proposal 3. RAN2 can ask SA3 about UE reports used in candidate solutions even before down-selection.

Proposal 4. Separate solution for each orbit type (NGSO, GSO, HAPS) can be considered, but common solutions should be assumed whichever the NTN-cell is earth-moving or earth-fixed.

- References

[1] “Draft Report of 3GPP TSG RAN WG1 #110 v0.2.0,” RAN1#110.

[2] RP-222654, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements,” Thales, RAN#97-e.

[3] TR38.882, “Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR,” V1.0.0.