3GPP TSG RAN WG2 Meeting #119b-e	R2-2210546

Electronic, 10th Oct. – 19th Oct., 2022

Agenda Item:	8.2.2 (Sidelink positioning)

Source: 	Samsung

Title: 	Discussion on out-of-coverage sidelink positioning

Document for:	Discussion & decision

Introduction

In this contribution, we discuss the possible issues in out-of-coverage sidelink positioning.

- Involvement of LMF

- Protocol architecture

- Necessary functions and signalling

Discussion

Involvement of LMF

One of three main scenarios, out-of-coverage would have the most different aspects in sidelink positioning. In in-coverage and partial coverage scenarios, at least minimum number of UEs can be connected with LMF in the core network. Even there is no discussion and agreement about LMF’s role and functionality, it is straightforward that LMF can handle most of positioning procedures for sidelink positioning. The reason is that already LMF does this in normal Uu link case, and there is no reason to introduce additional entity to handle sidelink positioning only. Even this needs to be agreed in SA2 It is good to reuse LMF entity.

Observation 1. To reuse LMF as a main control entity for sidelink positioning is highly probable in hybrid of PC5 and Uu case and Uu only case, i.e., in-coverage and partial coverage scenario.

There is no guarantee that LMF in core network can be connected with the SLP playing parties such as anchor UEs and target UEs in out-of-coverage. In our understanding, without LMF, there would be alternative server or similar entity located in one of UEs involved in sidelink positioning.

Observation 2. There is no LMF connected to any UEs involved in out-of-coverage scenario for sidelink positioning.

However, there should be anyway the responsible entity or agent within group of involved UEs to handle sidelink positioning operation in out-of-coverage. The most probable way is to have location server in one of UEs in out-of-coverage for sidelink positioning.

Proposal 1. RAN2 agree that there is no connection with LMF in any of involved UEs in sidelink positioning in OOC.

Proposal 2. RAN2 agree that there is a location server or equivalent entity in any of involved UEs in sidelink positioning out-of-coverage scenario.

Regarding this, RAN2#119 made the following agreement:

Agreement:

RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server.  FFS from RAN2 perspective if there are cases without a UE in the location server role.

And SA2 TR 23.700-86 [1] enclosed the solution #26 for the functional split and positioning Protocol for SL Positioning. The location server UE therein has the role of LMF without a connection to the network. RAN2 follow the terminology and the architecture therein.

Proposal 3. RAN2 adopt the terminology and architecture regarding UE with location server role in the out of coverage scenario from SA2.

Protocol architecture

If referring to ranging/sidelink positioning protocol (RSPP) defined in SA2 or SLPP in RAN2 appetite, one basic issue is whether to reuse the LPP for carrying RSPP in PC5-only scenario. Since there is no LMF involved in this case, it is not strict to reuse LPP protocol to carry / handle this new protocol. Therefore, it is simplest way to introduce a separate protocol architecture for only RSPP in PC5-only scenario. For using NAS container in legacy LPP case, it is not needed to use that container since there is no chance that transferred packets over PC5 is again to be carried to AMF. So the same reasoning is applied here too.

Observation 3. There is no chance that packets carried over PC5 for sidelink positioning out-of-coverage scenario are transferred to LMF and AMF in the core network.

There could be other view that in-coverage and partial coverage scenario also handle sidelink positioning, and those scenario is LMF involved, thus LMF/NAS protocol architecture can be reused. However, it is still unclear all the scenario including in-coverage and partial coverage are specified in this release discussion, and partial coverage once could be postponed after establishing in-coverage case. Moreover, having different or independent protocol stack can make clear specified behavior of UE, and can have no confusion which might happened by reusing ASN.1 code for different protocol. That’s why, in sidelink related WI, PC5-RRC was made for sidelink control plane separately with RRC.

Proposal 4. RAN2 discuss and agree that for out-of-coverage scenario, protocol for sidelink positioning would have its own protocol architecture which is different with LPP’s protocol architecture, i.e., not reuse LPP for carrying SLPP/RSPP message.

RAN2#119 already has shown the possible alternatives for new protocol stack for RSPP/SLPP as below:

-	Alt 1: CP-based solution, with new protocol over V2X/ProSe layer, e.g., PC5-D, PC5-S;

-	Alt 2: UP-based solution, with new protocol over PC5-U.

-	Alt 3: CP-based solution, with new protocol over PC5-RRC.

-	Alt 4: CP-based solution, with new protocol over PDCP.

At first glance, the reusing CP protocol stack for carrying new protocol data has the benefit of having some level of security protection. In case of using PC5-S or PC5-RRC, the security protection is done by underlying PDCP based on upper layer’s configuration. However, using UP stack needs additional effort to guarantee the security level. In the contents perspective, alts of using PC5-S/PC5-D needs the SA2’s involvement to be confirmed since those layer are in SA2 remit. Meanwhile PC5-RRC is used mainly for radio resource control between two UEs via unicast. Therefore, if the new protocol message is agreed also to be used for groupcast, then there should be discussion on how the groupcast can be realized with PC5-RRC. Moreover if using UP stack, then SDPA can be considered. If SDAP entity is involved, there should be a QoS flow mapping between this new protocol data and DRB. The thing is there is no information of QoS flow handling on this new protocol data since that information is from CN, and out-of-coverage scenario has no connection with core network.

Proposal 5. RAN2 discuss and determine the protocol architecture to be used among UEs in out-of-coverage scenario w.r.t.

- Security level required

- Message characteristics, i.e., user plane data or control plane data

- Allowed cast type for sidelink positioning

- Possibility of QoS flow mapping

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1. To reuse LMF as a main control entity for sidelink positioning is highly probable in hybrid of PC5 and Uu case and Uu only case, i.e., in-coverage and partial coverage scenario.

Observation 2. There is no LMF connected to any UEs involved in out-of-coverage scenario for sidelink positioning.

Proposal 1. RAN2 agree that there is no connection with LMF in any of involved UEs in sidelink positioning in OOC.

Proposal 2. RAN2 agree that there is a location server or equivalent entity in any of involved UEs in sidelink positioning out-of-coverage scenario.

Proposal 3. RAN2 adopt the terminology and architecture regarding UE with location server role in the out of coverage scenario from SA2.

Observation 3. There is no chance that packets carried over PC5 for sidelink positioning out-of-coverage scenario are transferred to LMF and AMF in the core network.

Proposal 4. RAN2 discuss and agree that for out-of-coverage scenario, protocol for sidelink positioning would have its own protocol architecture which is different with LPP’s protocol architecture, i.e., not reuse LPP for carrying SLPP/RSPP message.

Proposal 5. RAN2 discuss and determine the protocol architecture to be used among UEs in out-of-coverage scenario w.r.t.

- Security level required

- Message characteristics, i.e., user plane data or control plane data

- Allowed cast type for sidelink positioning

- Possibility of QoS flow mapping

References

[1] 3GPP TR 23.700-86: "Study on Architecture Enhancement to support Ranging based services and sidelink positioning".