3GPP TSG-RAN WG2 Meeting #119bis electronic                     R2- 2210600

Online, 10th – 19th October, 2022

Agenda Item	: 8.5.4 (FS_NR_XR_enh)

Source	: LG Electronics Inc.

Title	: Discussion on Scheduling enahancement for XR

Document for	: Discussion and Decision

Introduction

According to the SID agreed in RANP#95e, the objectives for the XR capacity enhancement is as follows [1]:

Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In RAN2#119 meeting [2], it is agreed that solutions in TR 38.838 and other RAN2 specific solutions can be considered for XR-specific capacity improvement.

1: As starting point, RAN2 can further discuss the solutions in TR 38.838 that can impact on L2 operation (e.g., BSR, LCP, assistance information for scheduling, packet discarding, prioritization) for XR-specific capacity improvement. RAN2-specific solutions are not precluded (even if RAN1 hasn’t discussed them before).

In this contribution, several aspects on scheduling enhancement for XR are discussed.

Discussion

DG enhancement for multiple streams

In TR38.838 [3], XR traffic may include different type of streams (e.g., pose/control, scene, video, audio, and other data), which have different QoS requirements. For example, while the video/audio traffic may be generated in 60 fps (1 packet with 16.66ms periodicity), the pose/control data can be generated for each 4 ms. In addition, since each of type of steam may have different PDB, each stream should be handled differently.

## 5.5 AR traffic model ### 5.5.1 AR DL stream(s) The AR DL stream(s) has/have the same models as VR DL stream model given in clause 5.3.1. ### 5.5.2 AR UL stream(s) In this clause, we provide four different options for AR UL traffic model. Given that AR has multiple streams in UL, one can choose a model from various options depending on what/how to model the streams. Four options are as follows. - Model 1: one stream model - Model 2: Two streams model - Model 3A: Three streams model A - Model 3B: Three streams model B The detail of each model is given in following clauses. - 5.5.2.3 Model 3A (three streams model A) In Model 3A, three steams are considered. - Stream 1: pose/control - Traffic model/requirement for stream 1 follows clause 5.2. - Stream 2: A stream aggregating streams of scene and video - Follows the statistical parameters shown in Table 5.5-1. - Stream 3: A stream aggregating streams of audio and data Table 5.5.2.3-1: Statistical parameters for stream 3 of AR UL Model 3A (three streams model) Parameters; unit; value. Data rate: R; Mbps; 0.756, 1.12. Periodicity: P; ms; 10. Packet size; byte; mean packet size = R×1e6 × P/1000 / 8. PDB; ms; 30.

In order to support the multiple traffic streams with different QoS requirements, each stream could be allocated to different LCG with different priorities. In this case, when the BSR is transmitted to the network, the network may have information on the amount of data for each stream, since the BSR reports the volume of available data for each LCG.

On the other hand, however, current LCP procedure for the received a UL grant is processed with all logical channels in the UE. Thus, it cannot be guaranteed that the UE uses the received UL grant for a specific stream. In other words, even though the network allocate the uplink grant to meet the QoS requirement for a specific stream, the UE may use the allocate grant to transmit the data of other traffic streams, e.g., due to the current LCP procedure and the PBR operation.

Given that satisfying QoS requirements of different streams is one of the most important objectives in XR, it would be beneficial to allocate the uplink grant with a fine control of traffic streams. Therefore, for better scheduling of each stream, it should be possible to allocate UL grant only for a specific LCG.

Proposal 1. RAN2 should study an enhancement UL grant in order to allow the transmission only for a specific LCG.

DG enhancement for paired UL and DL traffics

In a XR service scenario, it is possible or maybe usual that the UL XR traffic could be followed by the DL XR traffic. For example, when the pose/control UL data is expected after the specific scene of DL data, the transmission of UL data is roughly predicable. However, since the network does not know the exact time point when the UE produces the pose/control data, current CG mechanism may not be enough to serve the XR traffic in this case, e.g., for paired UL and DL traffics.

We also think that XR services can use a TCP protocol and TCP ACK packet can be another example of expecting UL data transmission is followed by the DL transmission. After the transmission of the DL packets, it is likely to expect to receive the corresponding UL data, e.g., TCP ACK/NACK, from the UE. Although the exact timing when the UE will transmit the TCP ACK is not known by the network, it can be roughly predictable when the UE will transmit the UL data corresponding to the DL data.

In the current dynamic grant operation, when there is no UL data available at the time point of the beginning of the grant, the UL transmission would be skipped. Given that the exact time point when the UE generates the UL data cannot be expected, in order to support the roughly expected UL data without BSR operation, the network should allocate dynamic grants multiple times, i.e., using multiple PDCCH indications. It naturally increases signalling overhead and causes additional latencies.

If the network may allocate the dynamic UL grant which is valid for a specific time duration, and the UE may transmit the UL data using the allocated grant within the specific time duration, the transmission of roughly predictable UL data could be ensured with low latency without increasing the signalling overhead. Therefore, in order to support for paired UL and DL traffics in which the exact timing of the UL transmission is not predictable after the DL transmission, it would be beneficial to configure an uplink grant with the valid time information.

Proposal 2. RAN2 should introduce an enhancement of dynamic grant which is valid for a specific time duration to support roughly expected UL data followed by DL data with low latency

Aspects on Intra-UE prioritization for Latency enhancement

Referring from the justification in XR SID: Many of the XR and CG use cases are characterised by quasi-periodic traffic (with possible jitter) with high data rate in DL (i.e., video steam) combined with the frequent UL (i.e., pose/control update) and/or UL video stream. Both DL and UL traffic are also characterized by relatively strict packet delay budget (PDB). Hence, there is a need to study and potentially specify possible solutions to better support such challenging services.

As shown in the above box, given that typical UL traffic is pose/control update and video stream, one important characteristic of XR data should be in time delivery since even if the data is delivered to the destination, if UL data for pose/control/video stream is not arrived in time, XR service or display would be frozen and user experience would be terribly deteriorated. Thus, satisfying strict PDB requirement for XR service should be considered, i.e., XR data should be delivered before exceeding PDB requirement.

Observation 1. If UL data for pose/control/video stream is not arrived in time, XR service or display would be frozen and user experience would be terribly deteriorated.

In NR Rel-16, the intra-UE prioritization mechanism is introduced to support tighter latency and reliability. In the Rel-16 intra-UE prioritization, when the two UL grants are overlapped, the priority of each UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, i.e., logical channel priority is only considered to determine which UL grant is prioritized.

However, the Rel-16 intra-UE prioritization does not consider the remaining time of data, and may have trouble to satisfy strict PDB requirement for XR service because one UL grant which is used for data from the highest priority of logical channel may be always prioritized over any other UL grant. For example, when two UL grants are given for XR service and one of UL grants has the highest priority logical channel with the data having long remaining time till the PDB boundary but another UL grant has the relatively lower priority with the data having short remaining time till the PDB boundary, the Rel-16 intra-UE prioritization prioritizes the UL grant having data from the highest priority logical channel. However, from XR service perspective, another UL grant for data having short remaining time should be prioritized in this condition because the data in the highest priority logical channel has sufficient remaining time till the PDB boundary and it should be no problem to be even transmitted later due to sufficient remaining time of data. Thus, to support tight XR latency requirement, it may need to consider remaining time of data for intra-UE prioritization.

Therefore, remaining time of data can be considered to enhance intra-UE prioritization in order to ensure tight latency requirement of XR service.

Observation 2. The current intra-UE prioritization only considers logical channel priority to determine priority of UL grant and may not be sufficient to support strict latency requirement of XR service.

Observation 3. The network may need to know exact amount of data which has short remaining time to provide smooth XR service.

Proposal 3. RAN2 should consider enhancement on intra-UE prioritization based on remaining time of data for XR service.

Aspects on RLC retransmission

As mentioned in the above section, in XR service, it is important to meet strict PDB requirement, since a packet is considered as lost when it has exceeded the PDB. In addition, in order to ensure the user experience of XR service and accurate tracking of the XR viewer’s pose, the high reliability transmission of XR traffic is required. According to TR 38.838, packet success rate requirement for XR traffic is 99% (or 99.99% in some cases).

On the other hand, in the current RLC specification, a transmitting AM RLC entity retransmits the RLC SDU or an RLC SDU segment only when the negative acknowledgement for the RLC SDU or an RLC SDU segment is received from the peer AM RLC entity in RLC Status Report. However, considering the strict PDB of XR traffic, long delay of ARQ retransmission in the poor radio condition would result in the exceeding PDB requirement and the degradation of XR performance.

In addition, once the RLC PDU of the RLC SDU or RLC SDU segment is submitted to the MAC layer, the transmitting AM RLC entity does not discard the RLC SDU, even though the discard indication is received from the upper layer. In other words, even though the PDB is exceeded for an RLC SDU, the RLC SDU would be retransmitted until the maximum number of retransmissions, which causes the resource waste.

In order to ensure the transmission within the required PDB for XR traffic, it is necessary to perform fast retransmission to reduce the overall transmission delay. For example, in a special condition (e.g., when the remaining time of RLC SDU is lower than a threshold), the transmitting AM RLC entity may retransmit a RLC PDU even if the transmitting AM RLC entity does not receive negative RLC feedback.

Proposal 4. RAN2 should study on the autonomous retransmission method of RLC PDUs in order to ensure the reliability in the required PDB.

Conclusion

In this contribution, we show our views on scheduling enhancement for XR traffic. The discussion includes following observations:

Observation 1. If UL data for pose/control/video stream is not arrived in time, XR service or display would be frozen and user experience would be terribly deteriorated.

Observation 2. The current intra-UE prioritization only considers logical channel priority to determine priority of UL grant and may not be sufficient to support strict latency requirement of XR service.

Observation 3. The network may need to know exact amount of data which has short remaining time to provide smooth XR service.

Based on the observation, followings are proposed:

Proposal 1. RAN2 should study an enhancement UL grant in order to allow the transmission only for a specific LCG.

Proposal 2. RAN2 should introduce an enhancement of dynamic grant which is valid for a specific time duration to support roughly expected UL data followed by DL data with low latency

Proposal 3. RAN2 should consider enhancement on intra-UE prioritization based on remaining time of data for XR service.

Proposal 4. RAN2 should study on the autonomous retransmission method of RLC PDUs in order to ensure the reliability in the required PDB.

Reference

[1] RP-220285, Revised SID : Study on XR Enhancements for NR

[2] R2-2208702, Report from session on LTE legacy, 71 GHz, DCCA, Multi-SIM, RAN slicing, QoE and XR

[3] TR 38.838, Study on XR (Extended Reality) Evaluations for NR