3GPP TSG-RAN WG2 Meeting #119-bis-e	R2-2210622

E-meeting, 10 October – 18 October 2022

Title:	[Draft] Reply LS Out on RAN dependency of FS_eNS_Ph3

Response to:	R2-2209355/S2-2207435

Release:	Rel-18

Work Item:	FS_eNS_Ph3

Source:	Ericsson [RAN2]

To:	SA2, RAN3

Cc:	-

Contact Person:

Name:	Håkan Palm

E-mail Address:	hakan.l.palm@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

RAN2 thanks SA2 for the LS on RAN dependency of FS_eNS_Ph3. In that LS, SA3 asked RAN2 for feedback on three questions.

Below is feedback and answers accordingly.

Q1.	Whether NG-RAN can broadcast one or more Secondary TAIs (up to a number RAN2 agrees, we note that for NTN is already possible to broadcast TWO TACs) via an updated SIB or new SIB, and report them to the CN and between gNBs as per existing Tracking Area related information exchange procedures but with indication they are secondary. The additional TAIs are associated with specific S-NSSAI(s) like the existing TAs and will be treated by UEs supporting secondary TAs as a normal Tracking area from RM standpoint (as described in solution#9)

A1.	RAN2 concluded that the solution in Q1 is not feasible to solve KI#3 because:

-	It has a high impact on the whole 5G system

-	It does not work for legacy UEs and it prevents legacy UEs from using the slices available in Secondary TAIs

-	It causes an increase of over the air and network interface signalling, while impacting some well established functions such as mobility and paging

-	It does not fulfil the objective of KI#3 due to lack of flexibility and scalability

-	It does create ambiguity in nodes behaviours, for example in case of connected mode mobility

Q2.	Whether the NG-RAN can be configured with a slice availability on a per-cell basis and

  - inform AMF and other gNBs in NGAP messages (as described in solution#11 and others)

  - Whether in Constrained Service Area the network slice is still supported but since no dedicated resources are allocated for the network slice the SLA of the network slice is not guaranteed.(as described in solution#45).

A2.	It is already possible to configure the NG-RAN with slice availability on a per-cell basis by means of configuration of RRM policies, i.e. resource pools per slice, however

-	There is no need to signal a list of cells configured with slice availability over RAN interfaces or over the RAN to CN interface

-	It is feasible to leave up to operators´ configuration and based on existing RRM policies configuration tools how a slice is served outside its slice availability area

Q3.	The NG-RAN receives in solution 29 (but conceivably this would be needed for similar solutions) the partially allowed S-NSSAIs in addition to the Allowed NSSAI. Can the NG-RAN in principle trigger handover procedure to a supporting TAI of the partially allowed S-NSSAIs when it is possible to do so? this can happen while in connected mode or when the UE is engaged in transition from Idle to connected mode. The reason is to enable the support of the maximum number of S-NSSAIs in the Allowed and partly allowed S-NSSAIs lists.

A3.	Legacy mobility functions already allow the RAN to carry out handovers of UE’s to other frequency bands, if RAN is aware of the need. RAN could use the partially allowed NSSAI as an indication that a UE may want access to a slice at another frequency band. However, there are other alternatives that also works for legacy UE’s, such as Target NSSAI, or solution 45.

We also note that there are more solutions captured in TR 23.700-41 with RAN impact, so we expect that SA2 will come back with another LS asking for more feedback.

2. Actions:

To RAN3 and SA2 group.

ACTION: 	RAN2 kindly asks SA2 and RAN3 to take the above into account.

3. Date of Next SA2 Meetings:

Please check the RAN2 calendar here