3GPP TSG-RAN WG2 Meeting #119bis-e  	               			   R2-2210648

Online, October 10 – October 19, 2022                          Revision of R2-2208335

Agenda item:	8.8.2

Source:	LG Electronics Inc.

Title:	Measurement report enhancement

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we discuss on the enhancement in NR, related to UE-triggered measurement report based on configured height threshold and measurement reporting based on a configured number of cells.

# 2. Discussion

## 2.1 Height-based measurement report enhancement

According to [2][4][6], it was observed that the HO failure (HOF) rate and RLF rate increase with increasing speed of aerial UEs. However, speed was not considered as a report condition in LTE.

Observation 1. As the speed increases, the HOF rate and RLF rate increase.

At low speed, there is no significant difference in the operation of NR and LTE. However, in a beam-manner in NR, many beams may be detected in the line-of-sight (LOS) environment. Beam can be changed frequently at high speed, and it can lead to many beam failures. Therefore, in NR, speed should be considered as a report condition. That is, if the speed of UE is more than a threshold, UE should send measurement reports. Then, network can make appropriate decisions about mobility and RLM based on the speed of the aerial UE.

Proposal 1. Speed should be considered as a report condition, i.e., if the speed of UE is more than a threshold, UE sends measurement reports.

Also, as both speed and height increase, HOF rate and RLF rate tend to increase further. Therefore, joint condition is useful to set the optimized measurement configuration to reduce unnecessary configuration or report. That is, joint condition of height and speed should be considered as a report condition.

Proposal 2. Joint condition of height and speed should be considered as a report condition.

## 2. 2 Interference measurement enhancement

In the legacy operation of the numberOfTriggeringCells, if a smaller number of neighbour cells than the threshold satisfies report conditions, UE cannot report DL interference to network. However, if the DL interference by a few neighbor cell is too strong, it can affect the operation of UE even though the DL interference cannot be reported. Then, UE may face problems before sending measurement reports such as decoding failure due to interference. Therefore, in the case of strong DL interference from neighbor cells, RAN2 should consider enhancement to report a strong DL interference even when the number of neighbour cells does not satisfy the numberOfTriggeringCells.

Observation 2. UE cannot report DL interference to network if a small number of neighbour cells satisfy report conditions.

Proposal 3. UE should report a strong DL interference even when the number of neighbour cells does not satisfy the numberOfTriggeringCells.

Additionally, according to the LTE specification, additional measurement reports are not sent after a measurement report triggered by the numberOfTriggeringCells even if a new cell satisfies the entering condition. Then, network cannot recognize the DL interference from the new cell after the first measurement report. Therefore, for the case of strong DL interference from neighbor cells, RAN2 should consider enhancement to continuously report DL interference after the measurement report triggered by numberOfTriggeringCells.

Observation 3. UE cannot report additional DL interference to network after a measurement report triggered by numberOfTriggeringCells even if a new cell satisfies the entering condition.

Proposal 4. For the case of strong DL interference from neighbor cells, UE should continuously report DL interference after the measurement report triggered by numberOfTriggeringCells.

To alleviate the problems above, the “leaving condition” can be used. However, there is no limit on measurement reporting by “leaving condition”. Then measurement report can be triggered a lot by “leaving condition”, which is detrimental to the intention of introduction of numberOfTriggeringCells. In order to reduce excessive measurement reporting caused by the leaving condition, we propose to apply the number of cells to the leaving condition.

Observation 4. There is no limit on measurement reporting by “leaving condition”. Then measurement report can be triggered a lot by “leaving condition”, which is detrimental to the intention of introduction of numberOfTriggeringCells.

Proposal 5. The number of triggering cells applies to the measurement report for a leaving condition.

Since aerial UEs can see multiple cells with similar signal strength at the same time due to the LOS environment, it is difficult to select a suitable cell for handover based on cell quality. In particular, in the case of  CHO, the UE performs the UE-based mobility. Then, the UE may execute the CHO to an unsuitable cell before receiving a new configuration according to the UE’s height. As a result, CHO failure may occur.

Observation 5. In case of CHO, if aerial UE detects more neighbour cells as height increases, UE may execute conditional mobility to unsuitable cells.

Considering that the height of aerial UE may change according to the flight path, the network should configure appropriate candidates based on the height in advance. The UE can select proper candidates based on UE’s height and can execute conditional mobility to the suitable cell.

Proposal 6. Network pre-configures CHO candidates based on height

# 3. Conclusion

Follows are our proposal:

Observation 1. As the speed increases, the HOF rate and RLF rate increase.

Proposal 1. Speed should be considered as a report condition, i.e., if the speed of UE is more than a threshold, UE sends measurement reports.

Proposal 2. Joint condition of height and speed should be considered as a report condition.

Observation 2. UE cannot report DL interference to network if a small number of neighbour cells satisfy report conditions.

Proposal 3. UE should report a strong DL interference even when the number of neighbour cells does not satisfy the numberOfTriggeringCells.

Observation 3. UE cannot report additional DL interference to network after a measurement report triggered by numberOfTriggeringCells even if a new cell satisfies the entering condition.

Proposal 4. For the case of strong DL interference from neighbor cells, UE should continuously report DL interference after the measurement report triggered by numberOfTriggeringCells.

Observation 4. There is no limit on measurement reporting by “leaving condition”. Then measurement report can be triggered a lot by “leaving condition”, which is detrimental to the intention of introduction of numberOfTriggeringCells.

Proposal 5. The number of triggering cells applies to the measurement report for a leaving condition.

Observation 5. In case of conditional mobility such as CHO and CPAC, if aerial UEs detect more neighbour cells as height increases, UE may execute conditional mobility to unsuitable cells.

Proposal 6. Network pre-configures CHO candidates based on height

# 4. References

[1] 36331-g60.docx

[2] 36777-f20.doc

[3] RP-213600 WID on NR support for UAV-cl.doc

[4] R2-1802907 Intel Measurement reporting information for Aerial UE

[5] R2-1812322 Nokia Subsequent measurement reporting upon N cells triggered - interference detection for UAVs

[6] R2-1808623 Docomo Mobility Enhancement for UAV UE

[7] ACJA-WT3-LTE-Aerial-Profile_v1.00.pdf