3GPP TSG-RAN2 Meeting #119bis electronic	R2-2210662

Online, Oct 10th – 19th, 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the FR2 Fallback Group 5

Agenda item:	6.24.1

Document for: 	Discussion and Decision

- Introduction

In the last meeting, a LS[1] was received from RAN4 about the newly added Fallback Group 5. In which, the Fallback Group 5 was defined as below:

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| R4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| R5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| R6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| R7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| R8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| R9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| R10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| R11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| R12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |

Furthermore, it also notes that in Rel 17, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz. To indicate this bandwidth limitation, a new element was also recommended to indicate a per band per BC aggregated channel bandwidth for both Uplink and downlink.

In this paper, we’d like to share our views on this LS.

- Discussion

In this chapter, we first share our understanding on the newly added Fallback Group 5, then discuss the interoperability and the feasibility of the new element.

2.1 Relationship between FBG 5 and legacy FBGs

According to RAN2’s understanding, the different FBGs are independent and the aggregated channel bandwidth ranges are not overlapped. But the classes in FBG5 are different. Take the R2/R3 of FBG5 and D/E of FBG 2 as an example:

Table 1: Aggregated bandwidth for each bandwidth class of FBG 2 and FBG 5

| FBG | NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC |
| --- | --- | --- | --- |
| FBG 5 | R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 |
|  | R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |
| FBG 2 | D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 |
|  | E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |

Fig 1: Aggregated bandwidth for each bandwidth class

The <Aggregated channel bandwidth, Number of contiguous CC> of FR2 class D are the same as that of FBG 5 class R2, while the <Aggregated channel bandwidth, Number of contiguous CC> of FR2 class E are the same as FBG 5 class R3 case 1.

Observation 1: The <Aggregated channel bandwidth, Number of contiguous CC> of FBG 2 bandwidth classes are covered by that of FBG 5 bandwidth classes.

Similarly, there are also some overlap between the FBG5 bandwidth class and FBG3 bandwidth class.

Table 2: Aggregated bandwidth for each bandwidth class of FBG 3 and FBG 5

| FBG | NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC |
| --- | --- | --- | --- |
| FBG 5 | R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 |
|  | R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |
| FBG 3 | G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 |
|  | H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |

Fig 2: Aggregated bandwidth for each bandwidth class

Observation 2: The <Aggregated channel bandwidth, Number of contiguous CC> of FBG 3 bandwidth classes are partly overlapped with that of FBG 5 bandwidth classes.

Proposal 1: Ask RAN4 to confirm whether the <Aggregated channel bandwidth, Number of contiguous CC> of legacy bandwidth classes are covered by or partly overlapped with that of FBG 5 bandwidth classes.

If the proposal 1 was confirmed, we’d like to further discuss how to report the BC with FBG5 bandwidth class and the BC with legacy bandwidth class. According to the observation 1, for a BC that support bandwidth class in the FBG5, it can also support the corresponding bandwidth class in the FBG 2. Then the problem is whether the BC with FGB2 bandwidth class can be seen as a fallback of a BC with FBG5 bandwidth class.

For example:

Assume: BC 1 <BAND X Class R3 > and BC 2 <BAND X Class E> The network which support FBG5 feature can derive that the UE support BC2 from the BC1 capability reporting. Question: Whether the UE need to report both BC 1 and BC2?

Proposal 2: Ask RAN4 to confirm whether the BC with FGB2 bandwidth class can be seen as a fallback of a BC with FBG5 bandwidth class.

Furthermore, with the introduction of the FBG 5, it’s also need to discuss the inter-operability between the (old/new) UE and (new/old) gNB as in the Table 2.

Table 2 Inter-operability Analysis

|  | Old UE | New UE |
| --- | --- | --- |
| Old gNB | Case 1: As legacy no interoperability issue | Case 2: Old gNB can’t understand FBG 5 |
| New gNB | Case 3: The UE will not report FBG5, the new gNB take the legacy way. So no interoperability issue | Case 4: For the BC with no FBG 5, the new gNB take the legacy way. So no interoperability issue |
| - Old UE/Old gNB: not implement bandwidth classes in the FBG 5 - New UE/New gNB: support bandwidth classes in the FBG 5 |  |  |

From the Table 2, we can see that the old gNB can’t understand FBG 5 class (e.g. when the UE get the UE capability from a R17 gNB then handover to an old gNB, or when the UE move to an old gNB at idle state and then establish connection with old gNB), then the network may ignore this BC.

Observation 3: The old gNB can’t understand FBG 5 bandwidth class (e.g. when the UE get the UE capability from a R17 gNB then handover to an old gNB, or when the UE move to an old gNB at idle state and then establish connection with old gNB), then the network may ignore the BC with FBG 5 bandwidth class.

According to the LS, RAN4 will defer to RAN2’s decision on whether to introduce an IE as proposed above depending on feasibility and benefit, for better understanding, it’s suggested to include the above observation to the Reply LS.

Proposal 3: Include the observation 3 in the Reply LS to RAN4

2.2 Per band per BC aggregated channel bandwidth Indication

According to the Ran4 LS, the newly added Per band per BC aggregated channel bandwidth Indication would be added for both UL and DL

- The new IE applies to intra-band carrier contiguous aggregation as well as an intra-band contiguous carrier aggregation component within an inter-band carrier aggregation. The new IE is separately applicable to each, UL, and DL. - When signalled for an explicitly supported BW class in FBG5: - It is in addition to the existing signaling for that BW class. - The network understands that the UE has independent maximum limits on number of CCs and max. aggregated bandwidth for that band. For example, when the UE indicates explicit support for R12 and a max. aggregated bandwidth of 1600Mhz using the new IE, it not only means the max. aggregated bandwidth 1600MHz applies to 12 CCs, but also applies to lower order classes, i.e., 11CCs, 10CCs, and so on. - The IE conveys the max. aggregated bandwidth value for each FeatureSetListPerUplink(Downlink)CC. for example, in each FeatureSetUplink(Downlink). - A band may have multiple values of max. aggregated bandwidth associated with different FeatureSetListPerUplink(Downlink)CC.

The recommended signaling structure by RAN4 is as below:

Note: The below figure is for FeatureSetDownlink Structure, the UL can take the same way

Fig 3: Newly added maximum bandwidth per band per BC

About this newly added parameter, generally there are 2 issues need to be further discussed in RAN2

- Whether there are any inter-operability issue?

- How about the efficiency of introducing this new elements?

For the issue 1, Ran4’s understanding in the LS about this parameter is as below:

The new IE is optional for a UE to signal. When the IE is not signalled, legacy operation is assumed: - The UE can still communicate to the network the maximum aggregated BW limitation using the existing framework. - The network understands that the UE supports the legacy fallback BW classes.

From RAN2 side, at least the below issues need to be clarified:

- What does absence of this IE mean for the FBG 5 in R17? Does it mean 1600 M or no additional maximum bandwidth limitation? If absence means no limitation, does it mean that for the bandwidth class R9~R12, it shall always report this new IE?

- What about the value range of the new IE, e.g. whether it can be larger than 1600 M?

Proposal 4: Before any discussion on the inter-operability, RAN2 to ask RAN4 to clarify the below issues about the new IE.

- What does absence of this IE mean for the FBG 5 in R17? Does it mean 1600 M or no additional maximum bandwidth limitation? If absence means no limitation, does it mean that for the bandwidth class R9~R12, it shall always report this new IE?

- What about the value range of the new IE, e.g. whether it can be larger than 1600 M?

From the efficiency aspect, with introduction of this newly added parameter, both the size of FeatureSetUplink(Downlink) list would be increased as shown in Fig 4: in which we assume FSD1~FSD x are affected by the newly added IE.

Fig 4: New IE impact on the FSD list

Based on the Fig4, we can see that the size of FeatureSetUplink(Downlink) list would be increased especially when multiple maximum values were introduced.

Proposal 5: About the efficiency, RAN2 to indicate RAN4 that the size of FeatureSetUplink(Downlink) list may be increased significantly especially when multiple maximum values were introduced.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: The <Aggregated channel bandwidth, Number of contiguous CC> of FBG 2 bandwidth classes are covered by that of FBG 5 bandwidth classes.

Observation 2: The <Aggregated channel bandwidth, Number of contiguous CC> of FBG 3 bandwidth classes are partly overlapped with that of FBG 5 bandwidth classes.

Proposal 1: Ask RAN4 to confirm whether the <Aggregated channel bandwidth, Number of contiguous CC> of legacy bandwidth classes are covered by or partly overlapped with that of FBG 5 bandwidth classes.

Proposal 2: Ask RAN4 to confirm whether the BC with FGB2 bandwidth class can be seen as a fallback of a BC with FBG5 bandwidth class.

Observation 3: The old gNB can’t understand FBG 5 bandwidth class (e.g. when the UE get the UE capabiliy from a R17 gNB then handover to an old gNB, or when the UE move to an old gNB at idle state and then establish connection with old gNB), then the network may ignore the BC with FBG 5 bandwidth class.

Proposal 3: Include the observation 3 in the Reply LS to RAN4

Proposal 4: Before any discussion on the inter-operability, RAN2 to ask RAN4 to clarify the below issues about the new IE.

- What does absence of this IE mean for the FBG 5 in R17? Does it mean 1600 M or no additional maximum bandwidth limitation? If absence means no limitation, does it mean that for the bandwidth class R9~R12, it shall always report this new IE?

- What about the value range of the new IE, e.g. whether it can be larger than 1600 M?

Proposal 5: About the efficiency, RAN2 to indicate RAN4 that the size of FeatureSetUplink(Downlink) list may be increased significantly especially when multiple maximum values were introduced.

- Reference

- R4-2215160 LS on new contiguous BW classes for legacy networks  RAN4, To: RAN2