3GPP TSG-RAN WG2 Meeting#119bis-e		       					   R2-2210667

Electronic, 10th – 19th October 2022

Source: 			ZTE corporation, Sanechips

Title: 	Supporting multiple power states

Agenda item:		8.3.2

Document for: 	Discussion and Decision

- Introduction

In [1], it is encouraged to adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations.

In this paper, we will discuss the impacts of sleep state on RAN2.

- Discussion

In RAN1#110 meeting [2], RAN1 had a heated discussion on the power state. In final, five power states are proposed as below:

Agreement For the purpose of evaluation, adopt the following as BS power consumption model. These entries for this table is per reference configuration set. - FFS: One or multiple values for relative power and transition time. Power state; Characteristic; Relative Power; Additional transition energy3; Total transition time. Deep sleep1; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state.; P1=1; E1; T1. Light sleep; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. (P2>P1); P2; E2; T2. Micro sleep; There is neither DL transmission nor UL reception. Immediate transition is assumed for network energy saving study purpose from or to a non-sleep state.; P3; 0; 0. Active DL; There is only DL transmission.; P4; NA; NA. Active UL; There is only UL reception. FFS: Whether multiple P5 values are needed to address low power UL mode; P5; NA; NA. Note 1: Depending on implementations, there could be a state that the power is lower than deep sleep and requires larger total transition time, e.g. hibernating sleep or Quasi-off, which is not explicitly modeled in this study for evaluation purpose. Note 3: Unit in relative power times duration. FFS: Details on how transition energy is defined.. - For simultaneous DL and UL transmission for FDD, the power for UL reception is neglected in this study. - FFS: Optionally, a state machine where BS may transit between sleep modes without entering non-sleep mode can be considered. Companies are to report the involved sleep modes and the assumptions for inter-sleep mode transition time used in their evaluations. - FFS: Details on how to use the above table for low power uplink reception (e.g. for WUS). Working Assumption For reference configuration set 1, the values are provided as below. FFS set2 and set 3. Power state; Relative Power P; Total transition time T. Deep sleep; 1; 1; Cat 1:50ms; Cat 2: 10s. Light sleep; Cat 1: 25; Cat 2: 2.1; Cat 1: 6 ms; Cat 2: 640 ms. Micro sleep; Cat1: 55; Cat 2: 5.5; 0; 0. Active DL; Cat 1: 280; Cat 2: 32; N.A.; N.A.. Active UL; Cat 1: 110; Cat 2: 6.5; N.A.; N.A..

As above, the gain of network energy saving (NES) decreases gradually from Deep sleep, Light sleep, Micro sleep, Active DL to Active UL. And we can observe that in network energy saving state, the longer the sleep time is, the more energy saving gain would be achieved while the worse the UE’s experience would be. To reduce the negative impact on user experience, RAN2 should discuss the impact of the different power states and study some possible optimization.

Besides, compared with Cat 1, Cat 2 means lower transmission power and longer sleep time, and could achieve more energy saving, hence, RAN2 should study the impacts of Cat 2 at priority.

Proposal 1: For power state, RAN2 should study the impacts of Cat 2 at priority.

Under the very low load case with few users, gNB could turn off and enter into NES state, in which the sleep time that a cell has neither DL transmission nor UL reception could be more than 10s, such as deep sleep Cat 2. In this case, UE could be indicated to switch to other non-NES cells, for example, gNB could trigger a handover for the UE in connected, and prevent access from UE in idle/inactive mode.

Under the low or light load case, there would be some infrequent data. In order to obtain the more network energy saving gain, gNB could try to enter into NES with the longer sleep time. gNB could schedule in some intensive subframes and then enter into sleep state. Perhaps, during the sleep time, a cell has neither DL transmission nor UL reception, and the sleep time is tens or hundreds of milliseconds, such as Light sleep Cat 2, Micro sleep. For example, under the low case, gNB could schedule the data sparsely and transmit SSB regularly in non-NES. If gNB determines to enter into NES in which a longer sleep time is used on purpose to obtain more power saving, gNB could schedule intensively in some subframes and transmit SSB, as illustrated in below.

To allow flexible change of the SSB configuration, different SSB configuration associated with different network states can be introduced.

More than one set of SSB configuration can be provided to UE and each set of configuration is associated with network state information, e.g. power state, applicable time or indication on whether it is in network energy saving state or not. To update the SSB configuration, network can simply indicate its state and UE would then select the corresponding SSB configuration and apply.

Proposal 2a: More than one set of SSB configuration can be provided to UE, with each set of configuration associated with network state information, e.g. power state, applicable time or indication on whether it is in network energy saving state or not.

Proposal 2b: To update the SSB configuration, network can simply indicate its state and UE would then select the corresponding SSB configuration and apply.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal 1: For power state, RAN2 should study the impacts of Cat 2 at priority.

Proposal 2a: More than one set of SSB configuration can be provided to UE, with each set of configuration associated with network state information, e.g. power state, applicable time or indication on whether it is in network energy saving state or not.

Proposal 2b: To update the SSB configuration, network can simply indicate its state and UE would then select the corresponding SSB configuration and apply.

- References

- RP-213554 New SI: Study on network energy savings for NR

- RAN1#110 meeting, chairman notes