3GPP TSG-RAN WG2 Meeting#119bis-e		       					   R2-2210669

Electronic, 10th – 19th October 2022

Source: 			ZTE corporation, Sanechips

Title: 	Consideration on RAN dependency of FS_eNS_Ph3

Agenda item:		8.18

Document for: 	Discussion and Decision

- Introduction

An LS [1] been sent from SA2 to RAN2 and RAN3 on the RAN dependency of some solutions to address Key Issue #3: Network Slice Area of Service for services not mapping to existing TAs boundaries. In this paper, we share some consideration from RAN2’s perspective.

- Discussion

In the coming LS, the following questions have been asked:

-------------------------------------------LS Out on RAN dependency of FS_eNS_Ph3-------------------------------------------

For Key Issue #3:  Network Slice Area of Service for services not mapping to existing TAs boundaries, and Temporary network slices, SA2 has following questions:

- Whether NG-RAN can broadcast one or more Secondary TAIs (up to a number RAN2 agrees, we note that for NTN is already possible to broadcast TWO TACs) via an updated SIB or new SIB, and report them to the CN and between gNBs as per existing Tracking Area related information exchange procedures but with indication they are secondary. The additional TAIs are associated with specific S-NSSAI(s) like the existing TAs and will be treated by UEs supporting secondary TAs as a normal Tracking area from RM standpoint (as described in solution#9)

- Whether the NG-RAN can be configured with a slice availability on a per-cell basis and

  - inform AMF and other gNBs in NGAP messages (as described in solution#11 and others)

  - Whether in Constrained Service Area the network slice is still supported but since no dedicated resources are allocated for the network slice the SLA of the network slice is not guaranteed.(as described in solution#45).

- The NG-RAN receives in solution 29 (but conceivably this would be needed for similar solutions) the partially allowed S-NSSAIs in addition to the Allowed NSSAI. Can the NG-RAN in principle trigger handover procedure to a supporting TAI of the partially allowed S-NSSAIs when it is possible to do so? this can happen while in connected mode or when the UE is engaged in transition from Idle to connected mode. The reason is to enable the support of the maximum number of S-NSSAIs in the Allowed and partly allowed S-NSSAIs lists.

-------------------------------------------LS Out on RAN dependency of FS_eNS_Ph3-------------------------------------------

  - Secondary TAIs

For Q1, the question can be split into two parts:

Part 1: Whether NG-RAN can broadcast one or more Secondary TAIs? The additional TAIs are associated with specific S-NSSAI(s) like the existing TAs and will be treated by UEs supporting secondary TAs as a normal Tracking area from RM standpoint.

Part 2: Whether the one or more Secondary TAIs can be reported to the CN and between gNBs as per existing Tracking Area related information exchange procedures with indication they are secondary?

We understand RAN2 is able to answer the Part 1 question while Part 2 fails into RAN3 scope.

Observation 1: Whether the one or more Secondary TAIs can be reported to the CN and between gNBs as per existing Tracking Area related information exchange procedures with indication they are secondary is within RAN3 scope.

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode                                            OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved},

...,

[[

iab-Support-r16                     ENUMERATED {true}                                               OPTIONAL       -- Need S

]],

[[

trackingAreaList-r17                SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode             OPTIONAL,      -- Need R

gNB-ID-Length-r17                   INTEGER (22..32)                                                OPTIONAL       -- Need R

]]

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

In the latest TS38.331, NG-RAN can broadcast more than one TAIs per cell. With the trackingAreaList-r17 introduced for NTN, more than one TAIs can be broadcast per PLMN for RAN sharing case. When the slice based cell reselection information is provided in SIB16 and RRCRelease, the association between TAIs and NSAGs would also be provided.

Regarding the Part 1 of the Q1 from SA2, we understand NG-RAN can now broadcast more than one TAIs per PLMN, with the association between TAIs and NSAGs provided but not differentiate which is the primary TAI and which are the secondary TAIs. RAN2 impact is foreseen if such differentiation is required when broadcasting the TAIs.

Proposal 1: RAN2 understand NG-RAN can now broadcast more than one TAIs per PLMN per cell, with the association between TAIs and NSAGs provided but not differentiate which is the primary TAI and which are the secondary TAI(s). RAN2 impact is foreseen if such differentiation is required when broadcasting the TAIs.

  - Slice availability on a per-cell basis

The second question is mainly about supporting slice availability on a per cell basis. From RAN2’s perspective, the slice availability is broadcast per cell with the supported NSAG associated with TA thus slice availability on a per cell basis can be supported in the Uu interface.

For a), Whether the NG-RAN can be configured with a slice availability on a per-cell basis and inform AMF and other gNBs in NGAP messages?

We understand it also fails into RAN3 scope.

For b), Whether in Constrained Service Area the network slice is still supported but since no dedicated resources are allocated for the network slice the SLA of the network slice is not guaranteed (as described in solution#45).

Looking at solution#45 in TR 23.700-41-100 [2], it has proposed to define the Constrained Service Area for Network Slice (CSANS) with cell granularity per S-NSSAI when the area of services over a network slice does not match the existing Tracking Area boundaries.

Figure 1: An example of CSANS

With the introduction of the CSANS, it is expected to optimize the handover procedure, i.e. if the radio quality of any cell out of the CSANS is acceptable, the NG-RAN removes the neighbor cell belonging to the CSANS from the target cell list for handover to prevent the UE with active PDU session related to the S-NSSAI from unnecessarily entering into a cell belonging to the CSANS. To avoid protocol impact on the UE, the CSANS information is not to be sent to the UE.

Based on the above understanding of solution#45, we observe that there would be RAN3 impact but no RAN2 impact foreseen for b).

Proposal 2: RAN2 understand slice availability on a per cell basis can be supported in the Uu interface but whether the NG-RAN can inform AMF and other gNBs in NGAP messages the slice availability per cell basis or whether in Constrained Service Area the network slice is still supported but since no dedicated resources are allocated for the network slice the SLA of the network slice is not guaranteed is within RAN3 scope.

  - Partially allowed S-NSSAIs

Figure.2: An example of partially allowed S-NSSAI

The third question is mainly about introduction of partially allowed S-NSSAIs. NG-RAN is expected to move UE to a cell in the supporting TA when UE is using certain S-NSSAIs, e.g. move UE from a cell in TA#1 to a cell in TA#2 when UE is using Slice#2.

In details, NG-RAN is expected to:

-	supports the intelligent activation/reactivation of PDU sessions and DRBs based on the TAI where the UE is in connected mode.

-	Attempts to trigger HO to a supporting TAI when UE is using certain S-NSSAIs upon transition from Idle mode to connected mode upon SR.

-	can determine which PDU sessions are activated or deactivated based on TAI where the UE is.

-	when the AN connection is released (and periodically, if configured to do so), can issue DL dropped data volume reports per PDU session to the AMF so it can be discounted when a PDU session is deactivated

-	can issue a NG-AP update information to AMF so the AMF detects when PDU session for a S-NSSAI are deactivated/reactivated and report this to SMF(s)

We understand most of the expected NG-RAN behaviors to support introduction of partially allowed S-NSSAIs require evaluation from RAN3.

From RAN2’s perspective, we observe that solution#29 (as copied below) has proposed to enhance the MT procedure by selecting a preferred band supporting the partially allowed S-NSSAI and indicate to UE via paging message to trigger cell reselection to a cell supporting the partially allowed S-NSSAI. Clear RAN2 impact is foreseen as we do not support paging triggered cell reselection and indicating the band info or slice info in the paging message.

----------------------------------------------------From solution#29 in 23.700-41-100-------------------------------------------

Figure 3: MT service request: success case with TAs supported by different gNBs

Figure 3 shows the MT service request with the UE which is registered in RA= TAI, TAI2 with Allowed NSSAI S-NSSAI1 and Partially allowed S-NSSAI in TAI2 S-NSSAI2 where the TAI1 and TAI2 are supported by cells under different gNBs. This DL paging applies to a PDU session for S-NSSAI2 and noticing the S-NSSAI 2 is partially allowed in the RA at step 2 the paging request includes the S-NSSAI 2. The RAN receives S-NSSAI 2 in the paging request and selects a preferred band from one of the TA2 cells bands (i.e. the preferred band for S-NSSAI2 is indicated). For paging messages sent to cells in TA1, the gNB1 sends to the UE a paging message with the indication of the preferred band (how this is done needs to be decided by RAN2) and the UE shall trigger cell reselection to this preferred band only considering cells in the RA. Regardless of the outcome of cell reselection, the UE issues then a service request. If the cell reselection had positive outcome, like in the figure 6.29.3.2.3-2, the SR is received at the AMF and the AMF detects it is from TA2 where the S-NSSAI2 is supported. This then allows the SR to successfully complete. The triggering downlink data is received at step 11. Otherwise, if the SR is received from TAI1, the AMF may trigger AN connection release.

----------------------------------------------------From solution#29 in 23.700-41-100-------------------------------------------

Proposal 3: RAN2 understand whether the NG-RAN can trigger handover procedure to a supporting TAI of the partially allowed S-NSSAIs should be evaluated by RAN3 while any enhancement to the MT procedure requiring paging triggered cell reselection or indication of preferred band or slice information via paging would have RAN2 impact and requires further discussion.

Proposal 4: Agree the draft reply LS [3] to SA2 addressing the RAN dependency of FS_eNS_Ph3 from RAN2’s perspective.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Observation 1: Whether the one or more Secondary TAIs can be reported to the CN and between gNBs as per existing Tracking Area related information exchange procedures with indication they are secondary is within RAN3 scope.

Proposal 1: RAN2 understand NG-RAN can now broadcast more than one TAIs per PLMN per cell, with the association between TAIs and NSAGs provided but not differentiate which is the primary TAI and which are the secondary TAI(s). RAN2 impact is foreseen if such differentiation is required when broadcasting the TAIs.

Proposal 2: RAN2 understand slice availability on a per cell basis can be supported in the Uu interface but whether the NG-RAN can inform AMF and other gNBs in NGAP messages the slice availability per cell basis or whether in Constrained Service Area the network slice is still supported but since no dedicated resources are allocated for the network slice the SLA of the network slice is not guaranteed is within RAN3 scope.

Proposal 3: RAN2 understand whether the NG-RAN can trigger handover procedure to a supporting TAI of the partially allowed S-NSSAIs should be evaluated by RAN3 while any enhancement to the MT procedure requiring paging triggered cell reselection or indication of preferred band or slice information via paging would have RAN2 impact and requires further discussion.

Proposal 4: Agree the draft reply LS [3] to SA2 addressing the RAN dependency of FS_eNS_Ph3 from RAN2’s perspective.

- References

[1] R2-2209355 LS Out on RAN dependency of FS_eNS_Ph3 (S2-2207435; contact: ZTE)

[2] 23.700-41-100

[3] R2-2210670_[Draft] Reply LS on RAN dependency of FS_eNS_Ph3