3GPP TSG-RAN WG2 #119-bis-e	R2-2210688

Electronical meeting, 10 October – 19 October 2022

Agenda Item:	8.5.2.2

Source:	Ericsson

Title:	Discussion on PDU Prioritization

Document for:	Discussion, Decision

# 1	Introduction

In [1], a new Study Item identifying three main areas of study, i.e., XR-Awareness in RAN, XR specific power savings and XR specific capacity improvements, was agreed.

“Objectives on XR-awareness in RAN (RAN2):

Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of.

Study how the above information aids XR-specific traffic handling.”

In RAN2#119 the following agreements related to study topic was made.

- XR awareness discussion in RAN2 should consider PDU set characteristics and how to use the information available on those (for UL and/or DL). Can also consider how to handle data bursts.

- RAN2 can study e.g. periodicity, arrival time, jitter and frame-size variations for XR awareness to enable power savings and capacity enhancements. Can study also how often such parameters change (i.e. how dynamic they are).

- RAN2 can consider how PDU sets can be mapped to DRBs (FFS if SA2 discussion on PDU set mapping to QoS (sub-)flows impacts this)

In this contribution we discuss our view on PDU prioritization

# 2	Discussion PDU Prioritization

In ongoing SA2 FS_XRM and RAN2#119 multiple companies have argued for the need of priority/importance information related to the PDU Set be conveyed to RAN in multiple ways, e.g., GTP-U and 5QI. Some companies have argued that codec related concepts such as I/P frame/slice have different requirement/characteristics that must be identified by the 5GS for differentiated handling in RAN. In this context companies argue that certain frame types should be prioritized differently in RAN, hence dynamic information about PDU set priority or importance should be exposed to the RAN scheduler. Prioritizing PDU Sets in any other way than order of arrival will make some PDU Sets to be delivered faster in expense of other PDU Sets being delivered slower. Essentially an existing PDU Set in the buffer will not be fully delivered if a to the buffer later arriving PDU Set takes priority to be delivered. The only way this can be beneficial is if the lower priority PDU Set can be skipped to transmit late, i.e. dropped, and thus free up resources for other users improving their performance and in turn potentially improving the overall capacity in the cell. The concept of dropping and implications of it is in detail described in our sister contribution on the PDU Discard sub AI [2]. It is important to note that dropping, or late delivery, will always impact user experience and thus the usage of such solutions will require careful considerations of all factors, i.e. it will be very situational if following the PDU Set priority is a beneficial thing to do. Thus it is in our understanding that usage of such detailed information would require very complex solutions and to justify such there should be clear indications of significant gains in system capacity proving its usefulness.

Considering inter PDU Set importance/priority information in RAN would create complex solutions.

Figure 1 - Evaluation of for frame dependence. Here, if a frame is missing, then all dependent frames are marked as missing as well for multi-flow GoP traffic with PF scheduler.

In Figure 1 we provide simulation results based on the traffic models defined in TR 38.838. In this simulation we evaluate the DL system capacity for Cloud Gaming assuming 30 Mbps and 15 ms PDB and compares the baseline traffic model where all video frames have the same priority and no dependency with a multi-flow Group of Picture (GOP) traffic model defined in 38.838 [3] . For the multi-flow traffic model, the simulation considers dependency between I-frames and P-frames (if a reference I-frame is lost then dependent P-frames are also marked as lost) as well as assigning different priority between the flow of I-frame and P-frame. As can be seen from the figure, prioritizing one frame type over the other show negative impact on XR system capacity when comparing the no prioritization at all case (green line) and the priority cases (blue and black lines), i.e. using prioritization increases the number of late frames. This can be described by the situations that an already existing frame in the buffer becoming delayed when a new frame incoming to the buffer takes priority to be transmitted and thus delaying the frame that is already being handled. These results show that considering even the simplest priority case (only two types of frames) and the heaviest dependency of them, i.e. making the higher priority I-frames the most valuable to receive. there is still a downside to using priority.

The usage of importance information results in negative capacity impact

However as discussed in our companion contribution [4] section 2.1.2. We support the idea that application can signal traffic flow requirements and assistance information to the 5GS. The 5GS may then map traffic flows to different QFI’s reflecting the requirements of the flow. As per existing means, the QFI is then mapped to a DRB. The application could still do the frame differentiation by separating frame types into different traffic flows with different requirement mapping to a QFI. But the 5GS would have no understanding of the frame type carried within the PDU set.

Dynamic (in-band) PDU set importance/priority is not considered.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Considering inter PDU Set importance/priority information in RAN would create complex solutions.

Observation 2	The usage of importance information results in negative capacity impact

Based on the discussion in the previous sections we propose the following:

Proposal 1	Dynamic (in-band) PDU set importance/priority is not considered.

# 4 References

RP-213587, Study on XR Enhancements for NR, Nokia, RAN#94e, December 2021.

R2-2210687, Discussion on PDU Set Discard, Ericsson, RAN2#119bis, October 2022

3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17)”, V17.0.0, Dec. 2021.

R2-2210689, Discussion on PDU Set and Data Burst, Ericsson, RAN2#119bis, October 2022