3GPP TSG-RAN WG2 #119-bis-e	R2-2210689

Electronical meeting, 10 October – 19 October 2022

Agenda Item:	8.5.2.1

Source:	Ericsson

Title:	Discussion on PDU Set and Data Burst

Document for:	Discussion, Decision

# 1	Introduction

- In [1], a new Study Item identifying three main areas of study, i.e., XR-Awareness in RAN, XR specific power savings and XR specific capacity improvements, was agreed.

“Objectives on XR-awareness in RAN (RAN2):

- Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of.

- Study how the above information aids XR-specific traffic handling.”

In RAN2#119 the following agreements related to study topic was made.

- XR awareness discussion in RAN2 should consider PDU set characteristics and how to use the information available on those (for UL and/or DL). Can also consider how to handle data bursts.

- RAN2 can study e.g. periodicity, arrival time, jitter and frame-size variations for XR awareness to enable power savings and capacity enhancements. Can study also how often such parameters change (i.e. how dynamic they are).

- RAN2 can consider how PDU sets can be mapped to DRBs (FFS if SA2 discussion on PDU set mapping to QoS (sub-)flows impacts this)

In this contribution we discuss how RAN can make use of PDU Set and Data burst in UL/DL direction and mapping to DRB.

# 2	Discussion PDU Set and Data burst

In TR 38.838, a model for XR traffic was agreed for evaluating capacity and power saving aspects in a 5G NR system. The model considers the multi-flow nature of XR traffic, which includes video, audio and pose flows, in DL and/or UL directions. The flows described in the TR have different periodicity and packet delay budget (PDB) constraints. Moreover, such flows are composed by application packets of different size (e.g., a video frame may be referred to as application packet). Therefore, to address the requirements of each XR flow in RAN, information about the flows and application packets therein is needed to enable enhancements for capacity and power saving.

In [2] we consistently used the term "application packet", following the definition of “packet” as described in section 5.1.1.1 in TR 38.838 [3], to identify an entire application information unit, e.g., a video frame.

However, in RAN2#119 it was agreed that RAN2 consider the ‘PDU Set’ and ‘Data Burst’ concept as defined in SA2. Adapting to the RAN2 agreement, we clarify in this contribution that in our view an ‘application packet’ will be mapped to a single ‘PDU Set’ and that the concept of ‘Data Burst’ as defined by SA2 is too vague to be of any use in RAN.

The term ‘Data Burst’ is currently defined in TR 23.700-60-040 [4] as: “a set of datamultiple PDUs generated and sent by the application in a short period of time.”. In our view this definition is ambiguous since it is unclear as what constitutes “a short period of time” and what is the relation between these PDUs. It is always up to the application how to map the application information. However, for the solutions that show gain in capacity and power saving it is important that the requirements are the same for all the data to be handled, e.g. the same delivery time and periodicity. The focus for RAN should thus be of handling data with a single set of requirements, i.e. one PDU Set. A ‘Data Burst’ constituting multiple PDU Sets sent back-to-back with the same requirements could just as well be mapped to a single PDU Set by the application.

The ‘Data burst’ concept is ambiguous, it is unclear what constitutes a short period of time and what relationship exist between PDUs in a burst.

RAN2 should not apply the concepts of Data Burst until concept and its need is clarified further by SA2.

## 2.1 Dynamic and static information needed in RAN

Table 1 Summary of application information useful for RAN, (x) indicates that the information may in some cases be considered dynamic depending on application, e.g application triggered events, (frame)rate adaptation etc. C = Capacity , PS = Power Saving.

To enable capacity and power saving enhancement discussed in [5] and [6], dynamic and static information about the PDU Set may be exposed to the RAN in different ways. In this context dynamic information would encompass information signalled for instance by the application in RTP extension headers in the IP packets associated to the PDU Set. Static information on the other hand would be provided once or infrequently. This information could be obtained by different means, for example, through an updated 5G Quality of Service (QoS) framework, learned by the network, or signalled by the application. SA2 and SA4 are discussing the means how to achieve this information. RAN2 should identify, however, what information is useful, how it can be used, and the benefits the information provides to the capacity or energy consumption. A short list of dynamic and static application information needed in RAN is presented in Table 1 below.

| Information type | Dynamic information | Semi Static information | Gain |
| --- | --- | --- | --- |
| Traffic flow periodicity | (x) | x | PS: up to 17% (CDRX) [6] C: up to 76 % (Pre-scheduling) [5] [7] |
| PDU Set delay budget | (x) | x | C: 10 % (delay scheduling & dropping) [5] |
| PDU Set Sequence Number | x |  | C: 10 % (delay scheduling & dropping) [5] PS: up to 12% (PDCCH Skipping) [8] |
| PDU Set size (number of bytes) | x |  | C: 10 % (delay scheduling & dropping) [5] PS: up to 12% (PDCCH Skipping) [8] |
| PDU Set size statistics |  | x | C: up to 76% (Pre-schudeling) [5] |
| Jitter statistics, e.g., distribution, range |  | x | PS: up to 17 % (CDRX) [6] |

For dynamic information the UPF can process and forward the necessary information to the network using a GTP-U header extension tunnel on the NG-U interface connecting UPF to the access network. The access network may then choose to forward information to the appropriate DU (in case of spilt architecture) or gNB on the F1-U and Xn-U interfaces, respectively, performing optimizations to select the correct terminal.

Static information on the other hand may be included as a QoS flow parameters configured by an AF function in the 5G core network and signalled on the NGAP interface. Some parameters may require infrequent updates. Rate-adaptation and/or frame rate adaption can impact both the requirements on the network and characteristics of the traffic. Required PDB may be relaxed or further restricted, periodicities and average throughput may fluctuate. These changes may occur often or seldom completely dependent on application implementation hence changes to traffic pattern information and requirements should be updated accordingly.

RAN2 should consider the following dynamic PDU Set information useful

- PDU Set Sequence Number and association of IP packets to a given PDU set

- PDU Set size (number of bytes in PDU Set)

RAN2 should consider the following (semi-) static information useful

- Traffic flow periodicity

- Jitter statistics e.g., distribution, range

- PDU Set size statistics

- PDU Set Delay Budget

Inform RAN3/SA2 about the application information useful for RAN2 solutions.

### 2.1.1	PDU Set UL Handling

It Is assumed that the parameters in the table are available in the UE for UL PDU Set handling. For example, The PDU Set size information, known up-front (on the first PDU entering the UE buffer), can be used together with the PDU Set SN to enable enhancements to the Buffer Status Report mechanism. Remaining PDU Set Delay budget forwarded in BSR can be used to enable a delay aware scheduler. On the other hand the uplink PDU Set size statistics may also be of use to the network scheduler. With such information available, the RAN can estimate the traffic in the UE buffer prior to receiving a BSR. This, together with knowledge of traffic flow periodicity can enable RAN to give speculative UL grants (pre-scheduling) matching the periodicity of the traffic flow.

PDU Set application information/awareness should be available in the UE and the NW to enable UL PDU Set handling

### 2.1.2	PDU Set DRB mapping and QoS

XR traffic will most likely constitute multiple flows in both UL and DL. Depending on application, these flows may have varying requirement in terms of bitrate and latency. Some companies have argued that codec specific concept such as I frame/slice and P frame/slices etc, have different requirement/characteristics that must be identified in the 5GS system for differentiated PDU Set handling. In our view such differentiation is not necessary as it is very complex and difficult to understand the gains in doing so. In [9] and [10] we present simulation results where frame type dependencies and prioritization have been considered. In these simulations the result shows no capacity gains differentiating between frame types.

Current QoS framework already includes various parameters that can be associated with traffic flows for differentiated treatment in RAN. In our understanding such framework can be re-used for XR-services with the extension of PDU Set granularity together with PDU Set specific parameters. With such framework in place, it is left to application to determine/identify the traffic flows and their requirements, signal them to 5GS/RAN and encapsulate IP packets into PDU Sets. In our preference we would like to see that such traffic flows will be mapped to separate QFI/5QI. As per existing means, it is then left to implementation to choose how these QFI map to different DRB to fulfill these requirements.

XR traffic flows with different requirement can be mapped to different QFI/5QI

It is left to implementation how RAN maps different QFI to different DRBs

Further enhancement in existing QFI-to-DRB mapping are not needed for PDU set level QoS support

Capture TP in Annex 1 about Dynamic and Static application information in TR 38.835 Section 5.1 XR-awareness

Capture TP in Annex 2 about PDU Set QoS and DRB mapping in TR 38.835

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	The ‘Data burst’ concept is ambiguous, it is unclear what constitutes a short period of time and what relationship exist between PDUs in a burst.

Observation 2	XR traffic flows with different requirement can be mapped to different QFI/5QI

Observation 3	It is left to implementation how RAN maps different QFI to different DRBs

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should not apply the concepts of Data Burst until concept and its need is clarified further by SA2.

Proposal 2	RAN2 should consider the following dynamic PDU Set information useful

	PDU Set Sequence Number and association of IP packets to a given PDU set

	PDU Set size (number of bytes in PDU Set)

Proposal 3	RAN2 should consider the following (semi-) static information useful

	Traffic flow periodicity

	Jitter statistics e.g., distribution, range

	PDU Set size statistics

	PDU Set Delay Budget

Proposal 4	Inform RAN3/SA2 about the application information useful for RAN2 solutions.

Proposal 5	PDU Set application information/awareness should be available in the UE and the NW to enable UL PDU Set handling

Proposal 6	Further enhancement in existing QFI-to-DRB mapping are not needed for PDU set level QoS support

Proposal 7	Capture TP in Annex 1 about Dynamic and Static application information in TR 38.835 Section 5.1 XR-awareness

Proposal 8	Capture TP in Annex 2 about PDU Set QoS and DRB mapping in TR 38.835

# 4 References

RP-213587, Study on XR Enhancements for NR, Nokia, RAN#94e, December 2021.

“R2-2208677, Discussion on XR awareness”, Ericsson, RAN2#119-e, August 2022

3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17)”, V17.0.0, Dec. 2021.

3GPP TR 23.700-60-040, “Study on XR (Extended Reality) and media services (Release 18)” Aug.

“R2-2208676 Discussion on capacity enhancements for XR”, Ericsson, RAN2#119-e, August 2022

“R2-2208680 Discussion on power saving enhancements for XR”, RAN2#119-e, August 2022

“R1-2205917 Discussion on capacity enhancements for XR”, Ericsson, RAN1#110-e, August 2022

“R1-2205916 Discussion on power savings enhancements”, RAN1#110, Toulouse, August 2022

“R2-2210687 Discussion on PDU Discard”, Ericsson, RAN2#119-bis-e, October 2022

“R2-2210688 Discussion on PDU Prioritization”, Ericsson, RAN2#119-bis-e, October 2022

# Annex 1

5.1.X XR Awareness

Two types of application information are needed in RAN to enable enhancements for XR services. Dynamic and (semi-)Static. Dynamic information is received on a per IP packet/PDU Set basis. (Semi-)static information on the other hand can be considered as QoS parameters or assistance information that is configured once or infrequently updated.

The following dynamic application information is necessary in RAN

- PDU Set Sequence Number and association of IP packets to a given PDU set

- PDU Set size (number of bytes)

The following (semi-)static information is necessary in RAN

- Traffic flow periodicity

- Jitter statistics, e.g. jitter range

- PDU Set Delay Budget

- PDU Set size statistics. E.g. (mean PDU Set size)

With PDU Set Sequence Number carried on every IP packet/PDU within the PDU Set, the RAN/UE can identify/associate the IP packets/PDUs belonging to the PDU Set.

With PDU set size (number of bytes) information received up-front i.e. on the arrival of the first PDU of the PDU set in the DL, the RAN scheduler can, combined with the remaining PDU Set Delay Budget, determine the feasibility of scheduling the entire PDU Set within the delay requirement. If the scheduler determines that the PDU Set cannot be scheduled within the delay requirement the scheduler can decide to drop the PDU Set early i.e. before the delivery deadline thus saving resources and increasing capacity.

The PDU Set Delay Budget will assist the RAN to understand the delay requirement associated to each PDU Set.

For saving more UE power, the PDU Set Size can be used jointly with the PDU Set Sequence Number to identify the end of the PDU Sets. This helps the RAN to decide when the traffic in the current DRX cycle has been transmitted and send a subsequent PDCCH skipping indication to save power until the next DRX cycle.

In the UE, PDU Set size and PDU Set Delay Budget, jointly with PDU Set SN can be used to enable enhancement to the BSR, e.g. including information of the remaining PDU Set Delay Budget of the PDU Sets currently in the buffer can enable a ‘delay aware scheduler’.

For power saving, knowledge of the traffic flow periodicity and jitter range can be used enable CDRX enhancements, e.g. to align DRX cycles with traffic pattern.

Uplink PDU Set size statistics may be of use to the network scheduler. With this information the RAN can estimate the traffic in the UE buffer prior to receiving a BSR. This, together with knowledge of traffic flow periodicity in the UL can enable RAN to give speculative UL grants (pre-scheduling) matching the periodicity of the traffic flow.

# Annex 2

5.1.X QoS Handling

Current QoS framework includes various parameters that can be associated with traffic flows for differentiated treatment in RAN. Such framework can be re-used for XR-services with the extension of PDU Set granularity together with PDU Set specific parameters. With such framework available, it is left to application to determine/identify the traffic flows and their requirements, signal them to 5GS/RAN and encapsulate IP packets into PDU Sets. Traffic flows will be mapped to separate QFI/5QI. It is left to implementation to choose how these QFI map to different DRB to fulfill the requirements.